3GPP TSG-RAN WG4 Meeting # 108bis                             R4-2317745

Xiamen, China, October 09 – October 13, 2023

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.876 to introduce minimum output power requirements for ATG UE

Agenda Item:	5.13.2.2

Document for:	Approval

Introduction

It’s proposed to capture the following technical analysis for minimum output power requirements.

- References

[1]

[2]

- Text proposal

- <<Start of Change for TR 38.876>>

- 7.1.2.5 Minimum output power

The calculation and results for minimum output power are listed below.

Table 7.1.2.5-1 the calculation for minimum output power requirements

| Parameters | Omni-directional antenna | phased array antenna |
| --- | --- | --- |
| center frequency GHz | 2 | 4 |
| Distance meter | 3000 | 3000 |
| Pathloss dB | 108.063 | 114.0836 |
| ATG BS antenna gain | 21 | 21 |
| ATG UE antenna gain | 0 | 11 |
| Thermal noise power density dBm/MHz | -114 | -114 |
| ATG BS Noise figure dB | 5 | 5 |
| Minimum output power dBm/MHz | -21.937 | -26.9164 |
| Minimum output power dBm/5MHz | -14.9473 | -19.92669996 |

The minimum output power requirements are specified below.

Table 7.1.2.5-2 the calculation for minimum output power requirements

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | X | X+10log10 (BWChannel /20) | X+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE 1: The minimum output power value is rounded to the nearest number down to one decimal point. NOTE 2: X = -15 for the ATG UE with omni-direction antenna. X = -19 for the ATG UE with phased array antenna. |  |  |  |  |

- <<End of Change>>