# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2317752

# Xiamen, China, October 9th– 13th, 2023

Title: 	WF on 4Tx UE RF requirements

Agenda Item:	5.3.4

Source: 	vivo

Document for:	Approval

Way Forward

UE capability for 4Tx TxD

- WF:

  - Introduce a new UE capability to indicate 4Tx TxD.

  - The granularity of the UE capability is per band per band combination

PCMAX,c tolerance for 4Tx

- Background

  - There were two options originally for discussion, one is using a new Pcmax tolerance curve created by shifting the 2Tx curve by 3 dB, and the other is reusing the tolerance curve for 2 Tx.A comparison is provided below (Reference from R4-2315063).

Figure 1 – PCMAX,c tolerance curves

- WF

  - In RAN4#108bis, adopting the following table which is based on shifting 2Tx curve by 3dB with certain power ranges in [] as baseline:

  - In RAN4#109, the ranges with [] will be revisited for possible reduction of shift values.

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| [26] ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| [25] ≤ PCMAX,c < [26] | 5.0 | 2.0 |
| [24] ≤ PCMAX,c < [25] | 5.0 | 3.0 |
| 23 ≤ PCMAX,c < 24 | 6.0 | 4.0 |
| 19 ≤ PCMAX,c < 23 | 5.0 |  |
| 14 ≤ PCMAX,c < 19 | 6.0 |  |
| -40 ≤ PCMAX,c < 14 | 7.0 |  |

UL-MIMO coherence for 4Tx

- Agreement:

  - Extend the current UL-MIMO coherence requirements for 2Tx requirements for any antenna connectors combination for 4Tx in this release.

CR impact of new TxD Capability

- Background

In this meeting, dedicated new signalling for 2Tx / 4Tx TxD have been introduced in Rel-18. However, this issue has not been considered in the current draft big CR yet.

- WF

  - Study how to apply 4Tx TxD UE capability in the specification