3GPP TSG-RAN WG4 Meeting #108-bis	R4-2315024

Xiamen, China, 09 – 13 October 2023

3GPP TSG-RAN WG Meeting #101	RP‑232692

Bangalore, India, September 11-15, 2023

Title:	LS on signaling support for intra-band non-collocated NR-CA,EN-DC

Response to:

Release:	Rel-18 (NR-CA), Rel-18 (EN-DC)

Work Item:	NonCol_intraB_ENDC_NR_CA-Core

Source:	RAN

To:	RAN2

Cc:	RAN4

Contact person:

Name:	Yasuki Suzuki

Email Address:	ui-suzuki@kddi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

1) New UE capability

The UE capability “interBandMRDC-WithOverlapDL-Bands-r16” was introduced since Rel-16 for UE to indicate the support of non-collocated deployment for FDD-FDD or TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping bands, while the UE capability to support non-collocated deployment for TDD-TDD intra-band NR-CA is being discussed in Rel-18.

For TDD-TDD intra-band NR-CA

During RAN4#107 meeting, RAN4 concluded a separate new capability signalling is needed, which is to indicate the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4.1/Table 7.6.4-2 in 38.133 and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in 38.101-1 (Type 2 capability requirement).

If the new UE capability is absent, the UE supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1 (Type 1 capability requirement).

RAN4 also concluded the UE indicating the “New UE capability” can also support Type 1 capability requirement.

This new UE optional capability is expected to be per BC, and applicable to only band n77 and n78.

RAN would like to request RAN2 to specify “New UE capability” based on the RAN4 agreements.

For TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands

In this case, the existing UE capability “interBandMRDC-WithOverlapDL-Bands-r16” and existing RAN4 requirements for Type 1 capability and Type 2 capability shall be used. See “interBandMRDC-WithOverlapDL-Bands-r16” in 38.306.

2) New BS signalling

During RAN#101 meeting, RAN agreed to introduce new BS signalling by RRC to indicate whether Type 1 or Type 2 capability requirements will be applied by a UE configured by the network with the value of IE maxMIMO-Layers no more than 2 and task RAN2 to develop relevant signalling.

This new BS signalling is applied to the below cases.

For TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands

This new BS signalling is expected to be applicable for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands provided the UE has indicated “interBandMRDC-WithOverlapDL-Bands-r16”.

For TDD-TDD intra-band NR-CA

This new BS signalling is expected to be applicable for TDD-TDD intra-band NR-CA provided the UE has indicated “New UE capability”.

# 2	Actions

To RAN2:

ACTION: 	RAN respectfully requests RAN2 to specify the above UE capability reporting for intra-band non-collocated NR-CA in Rel-18, and the above new BS signalling by RRC to indicate whether Type 1 or Type 2 capability requirements will be applied by a UE configured by the network with the value of IE maxMIMO-Layers no more than 2 and task RAN2 to develop relevant signalling.

# 3	Dates of next TSG-RAN meetings

TSG-RAN Meeting #102                  11th – 15th Dec 2023             	 Edinburgh, GB

TSG-RAN Meeting #103                  18th – 21th Mar 2024             	 EU, EU