3GPP TSG-RAN WG4 Meeting #108bis	R4-2315272

# Xiamen, China,  October 9th - 13th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101 | CR |  | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-1: Addition of PC3 new configuration for CA_n48A-n66(2A)-n71A-n77A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DISH Network, Fujitsu, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R18_yBDL_xBUL |  |  |  |  |  | Date: |  |  | 2023-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following CA band combinations are needed to maximize utilization of operator’s licensed spectrum: - CA_n48A-n66(2A)-n71A-n77A with UL configuration of CA_n48A-n66A, CA_n66A-n77A, CA_n48A-n71A, CA_n71A-n77A, CA_n66A-n71A |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the inter band CA operation bands (4 bands) in table 5.2A.3.3-1 to include: CA_n48-n66-n71-n77, CA_n48-n70-n71-n77 - Add the new inter-band CA configuration (4 bands) in section 5.5A.3.3: CA_n48A-n66(2A)-n71A-n77A The fallbacks are included in the latest specification. No new MSD, hence the draft CR format is adopted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requested inter-band NR-CA combinations are not included in current specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.3.3, 5.5A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Start of changes ---

#### 5.2A.2.3	Inter-band CA (four bands)

Table 5.2A.2.3-1: Inter-band CA operating bands involving FR1 (four bands)

Unaffected parts ommited

| CA_n3-n7-n8-n781 | n3, n7, n8, n78 |
| --- | --- |
| CA_n3-n7-n26-n78 | n3, n7, n26, n78 |
| CA_n3-n7-n28-n38 | n3, n7, n28, n38 |
| CA_n3-n7-n28-n78 | n3, n7, n28, n78 |
| CA_n3-n7-n67-n78 | n3, n7, n67, n78 |
| CA_n3-n18-n28-n41 | n3, n18, n28, n41 |
| CA_n3-n18-n28-n77 | n3, n18, n28, n77 |
| CA_n3-n28-n40-n77 | n3, n28, n40, n77 |
| CA_n3-n18-n41-n77 | n3, n18, n41, n77 |
| CA_n3-n28-n41-n77 | n3, n28, n41, n77 |
| CA_n3-n28-n41-n79 | n3, n28, n41, n79 |
| CA_n3-n28-n77-n79 | n3, n28, n77, n79 |
| CA_n3-n28-n41-n78 | n3, n28, n41, n78 |
| CA_n3-n41-n77-n79 | n3, n41, n77, n79 |
| CA_n5-n25-n66-n77 | n5, n25, n66, n77 |
| CA_n5-n25-n66-n78 | n5, n25, n66, n78 |
| CA_n5-n30-n66-n77 | n5, n30, n66, n77 |
| CA_n5-n48-n66-n77 | n5, n48, n66, n77 |
| CA_n7-n8-n40-n78 | n7, n8, n40, n78 |
| CA_n7-n25-n66-n77 | n7, n25, n66, n77 |
| CA_n7-n25-n66-n78 | n7, n25, n66, n78 |
| CA_n12-n30-n66-n77 | n12, n30, n66, n77 |
| CA_n13-n25-n66-n77 | n13, n25, n66, n77 |
| CA_n14-n30-n66-n77 | n14, n30, n66, n77 |
| CA_n18-n28-n41-n77 | n18, n28, n41, n77 |
| CA_n25-n38-n66-n78 | n25, n38, n66, n78 |
| CA_n25-n41-n66-n71 | n25, n41, n66, n71 |
| CA_n25-n41-n66-n77 | n25, n41, n66, n77 |
| CA_n25-n41-n66-n78 | n25, n41, n66, n78 |
| CA_n25-n41-n71-n77 | n25, n41, n71, n77 |
| CA_n25-n41-n71-n78 | n25, n41, n71, n78 |
| CA_n25-n66-n71-n77 | n25, n66, n71, n77 |
| CA_n25-n66-n71-n78 | n25, n66, n71, n78 |
| CA_n28-n41-n77-n79 | n28, n41, n77, n79 |
| CA_n29-n30-n66-n77 | n29, n30, n66, n77 |
| CA_n41-n66-n70-n78 | n41, n66, n70, n78 |
| CA_n41-n66-n71-n77 | n41, n66, n71, n77 |
| CA_n41-n66-n71-n78 | n41, n66, n71, n78 |
| CA_n48-n66-n71-n77 | n48, n66, n71, n77 |
| CA_n48-n70-n71-n77 | n48, n70, n71, n77 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |

Unaffected parts ommited

#### 5.5A.3.3	Configurations for inter-band CA (four bands)

Table 5.5A.3.3-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (four bands)

Unaffected parts ommited

|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| --- | --- | --- | --- | --- |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66(2A)-n71A-n78A | CA_n41A-n66A CA_n41A-n71A CA_n41A-n78A CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66A-n71A-n78(2A) | CA_n41A-n66A CA_n41A-n71A CA_n41A-n78A CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n41A-n66(2A)-n71A-n78(2A) | CA_n41A-n66A CA_n41A-n71A CA_n41A-n78A CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n48A-n66A-n70A-n71A | CA_n48A-n66A CA_n48A-n70A CA_n48A-n71A CA_n66A-n71A CA_n70A-n71A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66A-n70A-n77A | CA_n48A-n66A CA_n48A-n70A CA_n66A-n77A CA_n70A-n77A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n66A-n71A-n77A | CA_n48A-n66A CA_n48A-n71A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n66(2A)-n71A-n77A | CA_n48A-n66A CA_n66A-n77A CA_n48A-n71A CA_n71A-n77A CA_n66A-n71A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n70A-n71A-n77A | CA_n48A-n70A CA_n48A-n71A CA_n70A-n71A CA_n70A-n77A CA_n71A-n77A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

--- End of changes ---