3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315515

# Xiamen, China, Oct. 09 – 13, 2023

Source:	LG Electronics

Title:	Discussion on RRM requirements for Type 2 NR NTN UE in above 10GHz bands

Agenda item:	5.26.6.1.2

Document for:	Discussion

Introduction

In the last RAN4 meeting, RRM scopes and scenarios for Ka band have been agreed below [1];

- The scope of the work needed to define the RRM requirements (i.e. beam steering related) necessary for NR-NTN UEs operating in the NTN example band (i.e. Ka band) based on associated assumptions of the UE architecture - RRM requirements shall cover at least the following scenarios - Case-1: Stationary UE for GSO - Case-2: Stationary UE for LEO - Case-3: Mobile UE for GSO - The set of requirements shall include all RRM requirements relevant to NTN, based on Rel-17 NR NTN requirements unless critical issues are identified, including - UE uplink timing accuracy - RRC IDLE and INACTIVE mobility - (Conditional) Handover - RRC Re-establishment - RRC Connection Release with Redirection - Radio Link Monitoring - Link Recovery procedure (BFD/CBD) - Active TCI switching - Measurement Procedure - (L1/L3 measurement delay and scheduling/measurement restrictions) - Measurement Performance - FFS: UL spatial relation switching - The requirements need to cover the following UE architectures - Fully electronically-steered beam UEs (Type 1) - Fully mechanically-steered beam UEs (Type 2) - FFS if additional types of UEs shall be considered and subject to RAN4 RF session conclusions - Aim to reuse Rel-17 NR NTN requirements to the extent possible, except for beam sweeping aspects. - For Type 1 UEs the impact of beam sweeping on RRM requirements can be accounted for based on FR2-1 requirements in terms of scaling principles. - For Type 2 UEs additional studies are required to identify the methodology to define the requirements

In this contribution, we provided our views on RRM requirements for Type 2 NR NTN UE which has fully mechanically-steered beams in above 10GHz bands (Ka band) operations.

Discussion

In RAN#101, the scope of RRM requiremnets [2] for Ka band has been agreed below;

| RRM requirements to be defined as part of Rel-18 NR-NTN-enh WID | Electronic steering antenna | Mechanical steering antenna (inter-sat) | Mechanical steering antenna (intra-sat) |
| --- | --- | --- | --- |
|  | Terminal Type 1 | Terminal Type 2 |  |
| UE uplink timing accuracy | Y | Y | Reuse FR1 NTN requirement, except UL uplink timing accuracy |
| RRC IDLE and INACTIVE mobility | L | N |  |
| (Conditional/blind) Handover | Y | Y |  |
| RRC Re-establishment | L | L |  |
| RRC Connection Release with Redirection | N | N |  |
| Radio Link Monitoring | Y | Y |  |
| Link Recovery procedure (BFD/CBD) | N | N |  |
| Active TCI switching | N | N |  |
| Measurement Procedure | Y | N |  |
| (L1/L3 measurement delay and measurement gap, scheduling restriction due to mixed numerologies) | Y | N |  |
| Measurement Performance | Y | N |  |
| UL spatial relation switching | N | N |  |

And following assumptions are used for Rel-18 RRM requriements

- Single SAN Tx beam per radio cell in DL

- For inter satellite HO, only blind HO and no neighbor cell measurement

  - Blind HO assumption for Type 1 UE can be revisited if issue is identified in RAN4

- Intra satellite neighbor cell measurement is needed

- Same UE Rx beam is used for both serving and neighboring cells which belong to the same satellite (i.e., no Rx beam sweeping)

In RF session, following principle for beam pointing/accuracy related requirements for mobile VSAT and fixed VSAT has been agreed [3]. So, NTN UE for mobile VSAT and fixed VSAT should perform beam pointing for the satellite with specified accuracy.

<RF session> Beam pointing/accuracy related requirements Agreement: For mobile VSAT - RAN4 to specify NTN UE VSAT pointing accuracy based on manufacturer declaration. RAN4 to reuse the explanation from EN 303 978: “The applicant shall declare the peak pointing accuracy () and the associated statistical basis. The antenna shall maintain the declared peak pointing accuracy (), such that the off-axis EIRP emission density pattern projected onto the geostationary arc remains within the mask specified in clause [Total EIRP density specification] when shifted by an angle of ±(°), taking into account the following factors: the worst case operational environmental conditions; maximum ESOMP dynamics; and the range of latitude, longitude and altitude relative to the satellite orbital position.” For fixed VSAT - Proposal 3: RAN4 should specify antenna accuracy requirement for fixed VSAT, based on ETSI EN 301 360.

According to the agreements from RAN plenary and RF session, FR1 NTN requirements except for UL uplink timing accuracy can be reused. For handover requirements in inter satellite, since blind handover is considered, no measurement of inter satellite is needed before blind handover is initiated. However, exsting time or distance based condition should be considered for blind handover in NTN. Therefore, exsiting conditional handover requirements for FR1 NTN based on time/distance condition could be baseline except for measurement time. The difference with Type 1 UE is that Type 2 UE has longer beam steering time to target satellite when time or distance based condition is met. The beam steering time should be included within handover interruption time, but the beam steering time could vary by UE (device type) and degree of beam steering. RAN4 needs to discuss how to define beam steering time for handover interruption time.

- Proposal 1: Exsiting time/distance based conditional handover reuqirements for FR1 NTN except for measurement time can be baseline for blind handover for Type 2 UE.

- Proposal 2: Add beam steering time to target satellite within blind handover interruption time, and FFS on beam steering time.

Conclusion

In this contribution, we provide our views on RRM requirements for Type 2 NR NTN UE which has electronically-steered beams in above 10GHz bands (Ka band) operations, and we propose

- Proposal 1: Exsiting time/distance based conditional handover reuqirements for FR1 NTN except for measurement time can be baseline for blind handover for Type 2 UE.

- Proposal 2: Add beam steering time to target satellite within blind handover interruption time, and FFS on beam steering time.

Reference

[1] R4-2314447, “WF on R18 NR NTN RRM requirements,” Qualcomm Incorporated.

[2] RP-232694, “WF on potential RRM objectives for NR-NTN deployment in above 10 GHz (NR-NTN-enh WID),” Thales.

[3] R4-2314934, “WF on UE RF requirements for NR NTN,” ZTE Corporation, Samsung.