3GPP TSG-RAN WG4 Meeting #108-bis	R4-2315915

Xiamen, China, Oct 09 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On MultiRx Demodulation performance and CSI requirements – Simulation results

Agenda item:	5.7.4.2

Document for:	Discussion

Introduction

In the RAN4#107, simulation configurations including the channel model was agreed [1]. Further agreements were reached in the RAN#108 discussions regarding the simulation parameters and the PDSCH demodulation configurations for the simulations [2].

This paper contains Nokia’s updated simulation results.

Simulation Configuration

In the RAN4#107, it was agreed that RAN4 would adopt the virtual cable approach to test the FR2 multiRx demodulation requirements. In addition, as a result of the virtual cable approach agreement, the correlation model to be considered was agreed in RAN4#107 [1].

The simulation parameters used for Nokia’s simulations in the following are,

- Carrier frequency: FR2-1, 30 GHz

- 120 KHz SCS / 100MHz CBW

- Channel: “2x” TDLA30-75 + correlation model

- Antenna configuration of 1x1 or 2x2 for each TRP

- DMRS symbols at (2,11) symbol indexes.

- Frequency offset between TRPs: 0Hz

- Time offset between TRPs: 0us

- Gamma: 0.0625

Simulation Results

Simulation results for 2*(1x1) fully-overlapping – Separate processing.

Simulations results shown in Figure 1 consider the multi TRP scheme with 2*(1x1) antenna configuration with cross-polarization correlation coefficient of 0.0625 for different values of crosstalk interference (rho) and MCS17.

It can be observed from the simulation results that with higher crosstalk interference, it significantly reduces the achievable maximum throughput when separate processing is used.

Figure 1: 2*(1x1) - Separate processing – MCS17.

Table 1: Results for 2*(1x1) - Separate processing – MCS17 for different levels of crosstalk

| Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | MCS | Rank | Channel Model | Antenna Configuration | cross-polarization correlation coefficient (gamma) | RB Allocation Size | Cross talk factor (rho) - dB | Fraction of maximum throughput (%) | SNRBB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100/120 | FR2.120-1 | MCS17 | 2 | TDLA30-75 | 2(1x1) XPL | 0.0625 | 66 | No Crosstalk | 70 | 7.0 |
| 100/120 | FR2.120-1 | MCS17 | 2 | TDLA30-75 | 2(1x1) XPL | 0.0625 | 66 | -15 | 70 | 7.7 |
| 100/120 | FR2.120-1 | MCS17 | 2 | TDLA30-75 | 2(1x1) XPL | 0.0625 | 66 | -12 | 70 | 8.6 |
| 100/120 | FR2.120-1 | MCS17 | 2 | TDLA30-75 | 2(1x1) XPL | 0.0625 | 66 | -9 | 70 | 11.1 |
| 100/120 | FR2.120-1 | MCS17 | 2 | TDLA30-75 | 2(1x1) XPL | 0.0625 | 66 | -6 | 70 | N/A |
| 100/120 | FR2.120-1 | MCS17 | 2 | TDLA30-75 | 2(1x1) XPL | 0.0625 | 66 | -3 | 70 | N/A |
| 100/120 | FR2.120-1 | MCS17 | 2 | TDLA30-75 | 2(1x1) XPL | 0.0625 | 66 | 0 | 70 | N/A |

Simulation results for 2*(2x2) fully-overlapping – Separate processing.

Simulations results shown in Figure 2 consider the multi TRP scheme with 2*(2x2) antenna configuration with cross-polarization correlation coefficient of 0.0625 for different values of crosstalk interference (rho) and MCS13.

It can be observed from the simulation results that with higher crosstalk interference, it significantly reduces the achievable maximum throughput when separate processing is used.

Figure 2: 2*(2x2) - Separate processing – MCS13.

Table 2: Results for 2*(2x2) - Separate processing – MCS13 for different levels of crosstalk

| Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | MCS | Rank | Channel Model | Antenna Configuration | cross-polarization correlation coefficient (gamma) | RB Allocation Size | Cross talk factor (rho) - dB | Fraction of maximum throughput (%) | SNRBB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100/120 | FR2.120-1 | MCS13 | 4 | TDLA30-75 | 2(2x2) XPL | 0.0625 | 66 | No Crosstalk | 70 | 10.0 |
| 100/120 | FR2.120-1 | MCS13 | 4 | TDLA30-75 | 2(2x2) XPL | 0.0625 | 66 | -15 | 70 | 11.5 |
| 100/120 | FR2.120-1 | MCS13 | 4 | TDLA30-75 | 2(2x2) XPL | 0.0625 | 66 | -12 | 70 | 14.1 |
| 100/120 | FR2.120-1 | MCS13 | 4 | TDLA30-75 | 2(2x2) XPL | 0.0625 | 66 | -9 | 70 | N/A |
| 100/120 | FR2.120-1 | MCS13 | 4 | TDLA30-75 | 2(2x2) XPL | 0.0625 | 66 | -6 | 70 | N/A |
| 100/120 | FR2.120-1 | MCS13 | 4 | TDLA30-75 | 2(2x2) XPL | 0.0625 | 66 | -3 | 70 | N/A |
| 100/120 | FR2.120-1 | MCS13 | 4 | TDLA30-75 | 2(2x2) XPL | 0.0625 | 66 | 0 | 70 | N/A |

Conclusion

In this paper, we have provided Nokia’s updated simulation results for MultiRx topic.

- References

- R4-2309824 - WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception

- R4-2313977 - WF for Rel-18 Multi-Rx performance requirements

- Appendix A: Collection of previous agreements and proposals

Issue 1-1-1: Test method for demodulation requirements for FR2 multi-Rx. Agreement: RAN4 agreed to adopt virtual cable setup to test the FR2 multi-Rx demodulation requirements Issue 1-1-2: On new correlation matrix for FR2-1 in a Multi-TRP and Multi-Rx context for OTA demodulation performance requirements. Agreement: Consider the baseline correlation model assuming cross-talk between the TRPs and UE Rx antenna as below R16×16=Eh16×1h16×1H=11+ρ∙diag1 ρ ρ 1⊗RMIMO =11+ρ∙RMIMOρRMIMOρRMIMORMIMO, where h16×1=vec(H4x4) is the re-arranged channel values and - ⊗ denotes the Kronecker product, diag denotes the diagonal matrix. - RMIMO is the 4×4 correlation matrix for each link between TRPs to UE panels generated with parameters α (Tx correlation coefficient), β (Rx correlation coefficient), and γ (cross-polarization correlation coefficient) as stated in 38.101-4 B2.3.2 - ρ is the cross-talk power - Candidate cross-talk power values for initial simulation purpose: - -15, -12, -9, -6 dB - Other options are not precluded. Issue 1-1-3: Whether to extend candidate cross-talk power values. Agreements: - For now, don’t consider 0 dB and -3dB cross-talk levels as the SNR requirements could be quite high. - Interested companies are invited to provide sim results with relevant impartments considered. Issue 1-1-4: Choice of α and β values. Agreement: - Select α=β=0 in the correlation matrix for all cases. Issue 1-1-5: TxEVM. <way forward> - Proposals - Option 1: Consider Tx EVM at 6% since we are considering up to 64QAM modulation. Issue 1-1-6: Whether to adopt NT FR2 OTA enhancements when defining demodulation requirements. <way forward> - Proposals - Option 1: Yes Issue 1-1-7: Cross-polarization coefficient. Agreement: - Set cross-polarization coefficient g= 0.0625 Issue 1-1-8: Whether to consider ρ to be cross-talk power ratio. <way forward> - Proposals - Option 1: Yes Issue 2-1-5: Time/frequency offset between TRPs Agreements: - sDCI SDM: (-0.0625us, 600Hz) and (0.25us, 0Hz) - mDCI non-overlapping: (-0.0625us, 600Hz) - mDCI fully-overlapping: (-0.0625us, 600Hz) and (0.25us, 0Hz) - Interested companies can provide simulation results with (0.25us, 600Hz) for mDCI fully-overlapping scenario. Issue 2-1-6: PDSCH Configurations for demodulation requirements Agreements: Evaluate PDSCH demodulation performance with the following configurations. - Multi-DCI with fully-overlapping PDSCH, 1 layer per TRP - Multi-DCI with fully-overlapping PDSCH, 2 layers per TRP - Multi-DCI with non-overlapping PDSCH, 2 layers per TRP - Single-DCI SDM with 1+1 - Single-DCI SDM with 2+2 Issue 2-1-7: Test cases and simulation parameters for sDCI SDM Agreements: Tentative simulation assumptions for initial evaluations: Test parameters for sDCI SDM Parameter; Unit; Value. TRxP #1(Note 1); TRxP #2(Note 1). Transmit TRxP of SSB; TRxP #1. PDCCH configuration; TCI state; TCI State #1. CORESETPoolIndex; 0. Duplex mode; TDD. Active DL BWP index; 1. PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 1. Length (L); Specific to each Reference channel as defined in A.3.2.2. PRB bundling type; Static. PRB bundling size; 2. Resource allocation type; Type 1. RBG size; Config2. VRB-to-PRB mapping type; Non-interleaved. VRB-to-PRB mapping interleaver bundle size; N/A. PDSCH DMRS configuration; Antenna port indexes; 1000 for 1+1 1000-1001 for 2+2; 1002 for 1+1 1002-1003 for 2+2. TCI state; TCI State #1; TCI State #2. DMRS Type; Type 1. Number of additional DMRS; 1. Maximum number of OFDM symbols for DL front loaded DMRS; 1. Resource allocation; Full-overlapping. Precoding configuration; SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity.. Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: PDSCH transmission is done from both TRxPs (PDSCH Layers 0 and 1 are transmitted from TRxP #1 and PDSCH layers 2 and 3 are transmitted from TRxP #2). - Other options are not precluded. Issue 2-1-8: Test cases and simulation parameters for mDCI fully overlapping.<way forward> All tables related to this issue can be found in R4-2314258. - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for mDCI fully overlapping as presented in Tables 4-3 and 4-4. - Option 2: Use the simulation parameters for sDCI SDM (changes from RAN4#107 proposals highlighted) as presented in Tables 3-2. - Option 3: Consider PDSCH test cases and simulation parameter for mDCI fully overlapping scheme as presented in Tables 2-4 and 2-5. Issue 2-1-9: Test cases and simulation parameters for mDCI non-overlapping <way forward> All tables related to this issue can be found in R4-2314258. - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for mDCI non-overlapping scheme as presented in Tables 4-5 and 4-6. - Option 2: Define simulation parameters where the mDCI non-overlapping case is configured by splitting the available RBs equally between the two TRPs. - Option 3: Consider PDSCH test cases and simulation parameter for mDCI non-overlapping scheme as presented in Tables 2-6 and 2-7. Issue 2-1-10: Whether to consider a scaling factor for transmitted signal in mDCI fully-overlapping case <way forward> - Proposals - Option 1: Yes, SNR corresponds to SNR of TRxP #1 and TRxP #2 with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP. Issue 2-1-11: MCS and layer selection for sDCI. <way forward> - Proposals - Option 1: Select {MCS17, ρ = -6dB, rank 1+1} and/or {MCS13, ρ = -12dB, rank 2+2} for sDCI SDM cases. - Other options are not precluded. Issue 2-1-12: MCS and layer selection for mDCI. <way forward> - Proposals - Option 1: Select {MCS17, ρ = -6dB, rank 1+1} and/or {MCS13, ρ = -12dB, rank 2+2} for mDCI fully-overlapping cases - Other options are not precluded.