3GPP TSG-RAN4 Meeting #108-bis	R4-2316072

# Xiamen, China, 9 – 13 October, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on test setup for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Perf |  |  |  |  |  | Date: |  |  | 2023-09-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 needs to update the test setup for NTN based on RP-232682. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the test setup for NTN based on RP-232682. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN5 cannot implement the NTN RRM test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.2.1, A.7.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is based on agreed CR R4-2314415. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.3.36.4	Void

A.3.36.5	Satellite specific parameters configuration

A.3.36.5.1	Satellite specific configuration for serving cell

Table A.3.36.5.1-1: Satellite specific configuration pattern 1 for serving cell in GSO scenario

| Parameter | SSC.1 |
| --- | --- |
| Interval between adjacent epoch time | 10.24s |
| ntn-UlSyncValidityDuration | 900s |
| cellSpecificKoffset | 256 slots |
| ta-Common | 0 |
| ta-CommonDrift | 0 |
| ta-CommonDriftVariant | 0 |
| ntn-PolarizationDL | linear |
| ntn-PolarizationUL | linear |
| ephemerisInfo | Note 1 |
| NOTE 1: this field is set such that UE considers the the sateillite is with zero delay and zero Doppler shift for test cases other than the test case in A.14.3.2. For test case in A.14.3.2, this field is set such that UE considers the the sateillite is with [180]ms delay and zero Doppler shift. The details are defined in [RAN5 spec #]. |  |

Table A.3.36.5.1-2: Satellite specific configuration pattern 2 for serving cell in NGSO scenario

| Parameter | SSC.2 |
| --- | --- |
| Interval between adjacent epoch time | 2.56s |
| ntn-UlSyncValidityDuration | 5s |
| cellSpecificKoffset | 8 slots |
| ta-Common | 0 |
| ta-CommonDrift | 0 |
| ta-CommonDriftVariant | 0 |
| ntn-PolarizationDL | linear |
| ntn-PolarizationUL | linear |
| ephemerisInfo | Note 1 |
| NOTE 1: this field is set such that UE considers the the sateillite is with zero delay and zero Doppler shift for test cases other than the test case in A.14.3.2. For test case in A.14.3.2, this field is set such that UE considers the the sateillite is with [180]ms delay and zero Doppler shift. The details are defined in [RAN5 spec #]. |  |

A.3.36.5.2	Satellite specific configuration for neighbour cell

Table A.3.36.5.2-1: Satellite specific configuration pattern 1 for neighbour cell in GSO scenario

| Parameter | NSC.1 |
| --- | --- |
| Interval between adjacent epoch time | 10.24s |
| ntn-UlSyncValidityDuration | 900s |
| cellSpecificKoffset | 256 slots |
| ta-Common | 0 |
| ta-CommonDrift | 0 |
| ta-CommonDriftVariant | 0 |
| ntn-PolarizationDL | linear |
| ntn-PolarizationUL | linear |
| ephemerisInfo | Note 1 |
| NOTE 1: this field is set such that UE considers the the sateillite is with zero delay and zero Doppler shift for test cases other than the test case in A.14.3.2. For test case in A.14.3.2, this field is set such that UE considers the the sateillite is with [6]ms delay and zero Doppler shift. The details are defined in [RAN5 spec #]. |  |

Table A.3.36.5.2-2: Satellite specific configuration pattern 2 for neighbour cell in NGSO scenario

| Parameter | NSC.2 |
| --- | --- |
| Interval between adjacent epoch time | 2.56s |
| ntn-UlSyncValidityDuration | 5s |
| cellSpecificKoffset | 8 slots |
| ta-Common | 0 |
| ta-CommonDrift | 0 |
| ta-CommonDriftVariant | 0 |
| ntn-PolarizationDL | linear |
| ntn-PolarizationUL | linear |
| ephemerisInfo | Note 1 |
| NOTE 1: this field is set such that UE considers the the sateillite is with zero delay and zero Doppler shift for test cases other than the test case in A.14.3.2. For test case in A.14.3.2, this field is set such that UE considers the the sateillite is with [6]ms delay and zero Doppler shift. The details are defined in [RAN5 spec #]. |  |

<End of Change 1>

<Start of Change 2>

B.5	High level test procedure for SAN RRM tests

In this version of specification, for SAN RRM tests defined in A.14, the test equipment shall adjust the time and frequency of its transmission as follows.

-	For GSO as defined in A.3.36.5.1, apply zero delay and zero Doppler shift for test cases other than the test case in A.14.3.2. For test case in A.14.3.2, apply [180]ms delay and zero Doppler shift.

-	For NGSO as defined in A.3.36.5.2, apply zero delay and zero Doppler shift for test cases other than the test case in A.14.3.2. For test case in A.14.3.2, apply [6]ms delay and zero Doppler shift.

<End of Change 2>