3GPP TSG-RAN4 Meeting #108bis	 R4-2316276

# Xiamen, China, October 09 – October 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.174 | CR | TBD | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for adding RF requirements for mIAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mobile_IAB-Core |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding RF requirement for mobile IAB |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Tx dyanmic range, power control are updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No mobile IAB RF requirement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.12, 6.2.1A,6.3.2.1.2A, 6.3.3.1A, 9.3.1A, 9.4.2.1.2A,9.4.2.1.3A,9.4.3.1.1.A,9.4.3.2.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.176-1/2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- START CHANGE ------------------------------------------------------

## 4.12	Requirements for additional features

Unless stated otherwise, the suffixes shown in Table 4.12-1 is defined for mobile IAB.

Table 4.12-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| A | Mobile IAB |

A IAB which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional clause (suffixes A) in clauses 5, 6, 7,8,9,10,11 and 12. Where there is a difference in requirement between the general requirements and the additional clause requirements (suffixes A) in clauses 5, 6, 7,8,9,10,11 and 12, the tighter requirements are applicable unless stated otherwise in the additional clause.

### 6.2.1A	General

Only Local Area IAB-MT is applied.

##### 6.3.2.1.2A	Minimum requirement for IAB-MT type 1-H

For a local area IAB-MT the total power dynamic range for each NR carrier shall be larger than or equal to 73 -10log10 (BWChannel /20)) dB where BWChannel  is the IAB-MT configured channel bandwidth.

#### 6.3.3.1A	Relative power tolerance for local area IAB-MT type 1-H

The relative power tolerance is the ability of the transmitter to set its output power in a target sub-frame (1 ms) relatively to the power of the most recently transmitted reference sub-frame (1 ms) if the transmission gap between these sub-frames is less than or equal to 20 ms.

The minimum requirements specified for each TAB-connector in Table 6.3.3.1A-1 apply only when the output power is within the limits set by declared maximum output power and specified dynamic range.

2 exceptions are allowed for each of two test patterns. The test patterns are a monotonically increasing power sweep and a monotonically decreasing power sweep. For those exceptions, the power tolerance limit is a maximum of [± 6.0 dB] in Table 6.3.3.1A-1.

Table 6.3.3.1A-1: Relative power tolerance

| Power step P (Up or down) (dB) | Power tolerance (dB) |
| --- | --- |
| ΔP < 2 | [± 2.5] |
| 2 ≤ ΔP < 3 | [± 3.5] |
| 3 ≤ ΔP < 4 | [± 4.5] |
| 4 ≤ ΔP < 10 | [± 5.5] |
| 10 ≤ ΔP < 15 | [± 6] |
| 15 ≤ ΔP | [± 6.5] |

### 9.3.1A	General

Only Local Area IAB-MT type 1-O and type 2-O is applied.

##### 9.4.2.1.2A	Minimum requirement for IAB-MT type 1-O

For a local area IAB-MT the total power dynamic range for each NR carrier shall be larger than or equal to [45] dB.

##### 9.4.2.1.3A	Minimum requirement for IAB-MT type 2-O

For a local area IAB-MT the total power dynamic range for each NR carrier shall be larger than or equal to [30] dB.

##### 9.4.3.1.1A	Relative EIRP tolerance for local area IAB-MT type 1-O

The relative EIRP tolerance is the ability of the transmitter to set its radiated output power in a target sub-frame (1 ms) relatively to the power of the most recently transmitted reference sub-frame (1 ms) if the transmission gap between these sub-frames is 20 ms.

The minimum requirements specified in Table 9.4.3.1.1A-1 apply only when the output power is within the limits set by declared maximum output power and specified dynamic range.

2 exceptions are allowed for each of two test patterns. The test patterns are a monotonically increasing power sweep and a monotonically decreasing power sweep. For those exceptions, the power tolerance limit is a maximum of [± 11.0 dB] in Table 9.4.3.1.1A-1.

Table 9.4.3.1.1A-1: Relative EIRP tolerance for local area IAB-MT type 1-O

| Power step ∆P (Up or down) (dB) | EIRP tolerance (dB) |
| --- | --- |
| ΔP < 2 | [± 2.5] |
| 2 ≤ ΔP < 3 | [± 3.5] |
| 3 ≤ ΔP < 4 | [± 4.5] |
| 4 ≤ ΔP < 10 | [± 5.5] |
| 10 ≤ ΔP < 15 | [± 6] |
| 15 ≤ ΔP | [± 6.5] |

##### 9.4.3.2.1A	Relative EIRP tolerance for local area IAB-MT type 2-O

The relative EIRP tolerance is the ability of the transmitter to set its radiated output power in a target sub-frame (1 ms) relatively to the power of the most recently transmitted reference sub-frame (1 ms) if the transmission gap between these sub-frames is 20 ms.

The minimum requirements specified in Table 9.4.3.1.1A-1 apply only when the output power is within the limits set by declared maximum output power and specified dynamic range.

2 exceptions are allowed for each of two test patterns. The test patterns are a monotonically increasing power sweep and a monotonically decreasing power sweep. For those exceptions, the power tolerance limit is a maximum of [± 11.0 dB] in Table 9.4.3.1.1A-1.

Table 9.4.3.2.1A-1: Relative EIRP tolerance for local area IAB-MT type 2-O

| Power step ∆P (Up or down) (dB) | EIRP tolerance (dB) |
| --- | --- |
| ΔP < 2 | [±3.0] |
| 2 ≤ ΔP < 3 | [±4.0] |
| 3 ≤ ΔP < 4 | [±5.0] |
| 4 ≤ ΔP < 10 | [±6.0] |
| 10 ≤ ΔP < 15 | [± 6] |
| 15 ≤ ΔP | [± 6.5] |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------