3GPP TSG-RAN WG4 Meeting #108bis                                       R4-2316372

Xiamen, China, October 09 – 13, 2023

Source: 	Huawei, HiSilicon

Title: 	On the RF requirement for STxMP

Agenda Item:	5.29.1

Document for:	Approval

Introduction

In the previous RAN4 meeting, fruitful discussion as to this feature has been done. Following conclusions and study points are excerpted from WF [1]:

<Agreement> PCMAXf,c,k - LS is sent to RAN1 to inform that RAN4 will introduce PCMAXf,c,k for STxMP (See R4-2314698) - How to incorporate the PCMAXf,c,k into the spec will be discussed in RAN4#108-bis <Way forward> MPRf,c,k/A-MPRf,c,k - MPRf,c,k/A-MPRf,c,k will be further discussed and determined in RAN4#108-bis from the following options - Option 1: MAX[(MPRk , A-MPRk, MPRp, A-MPRp)] +3dB in lower bound for beam k and p - Option 2: MAX(X, MPRf,c,k, A- MPRf,c,k), X = 10*log10(number of UL TCI-states indicated for [STxMP]) dB in lower bound - Option 3: Define ‘per-panel’ requirements of MPRf,c,k = MPRf,c + 3dB, and A-MPRf,c,k = A-MPRf,c + 3dB - Option 4: Reuse MPRf,c and A-MPRf,c requirements, and add 3dB relaxation to lower bound - Option 5: Do not extend the current MPR concept at least in this release - Option 6: Other proposals based on legacy MPR/A-MPR requirements are not precluded for RAN4#108-bis <Way forward> P-MPRf,c,k - In RAN4#108-bis, it will be discussed how to ensure EIRP compliance <Way forward> PUMAXf,c,k - Whether to introduce PUMAXf,c,k will be determined based on the discussion results of MPRf,c,k/A-MPRf,c,k <Way forward> New signalling - Whether to introduce new signalling for overlapped beams indication depends on the discussion results of MPRf,c,k/A-MPRf,c,k in RAN4#108-bis <Way forward> Testability - RAN4 will check the testability issue before PUMAXf,c,k is introduced, e.g., sending LS to RAN5 and/or other means

In this contribution, we would like to share our further views on this topic.

Discussion

- PCMAXf,c,k

As one remaining issue, RAN4 will discuss how to incorporate per panel PCMAX into the specification. In our view, the implementation would highly depend on RAN4 view regarding whether the outcome of current discussion progress could be sufficient to build the framework for transmitted power configuration of STxMP. In previous meeting, the supplemental LS [2] other than previous one [3] has been approved. During RAN4 discussion, some certain reluctance regarding explicit definition on per panel PUMAX, MPR/A-MPR and potential testability issue deserves attention because it may be hard for RAN4 to go through a complete discussion on a full set of those per panel requirements along with test set up within Rel-18 time frame. Hence, if such concern gradually becomes popular in RAN4, it is better to go with only defining per panel PCMAX for this release and probably carrying on the remaining aspects if clear demands show up in the future.

Proposal 1: Check if RAN4 can only define PCMAX, f, c, k for STxMP in Rel-18 and postpone remaining aspects e.g. per panel PUMAX, MPR/A-MPR and potential testability issue until clear demands show up in the future.

In case RAN4 will think it is necessary to introduce a full set of requirements in Rel-18, we would like to share our further views.

- MPRf,c,k/A-MPRf,c,k

Since it was agreed that the total EIRP/TRP over all panels cannot exceed the existing EIRP/TRP limitation, it is our understanding that the overall output power would still be what can be achieved by the advertised power class under STxMP mode.

Proposal 2: For STxMP operation, clarify that the overall output power capability would still be what can be achieved by the advertised power class considering all legacy transmitter RF requirements for single band operation as defined in TS 38.101-2 clause 6.

As for MPR, it is well understood that it allows UE to reduce output power so that all of EVM, ACLR and SEM requirements can be satisfied under certain transmission configuration. While it is no harm to further extend it to per panel MPR, the equivalent maximum reduction per UE level should not increase the risk for the UE’s violation of those requirements.

Examine one solution that has been proposed in [4] as below.

It was concisely demonstrated that such 3dB ‘alternative’ MPR is necessary for ‘equal TRP split’ case, in which the gNB would expect the UE to achieve maximum EIRP for both beam peak directions under STxMP mode, while it is still required for the UE to comply with the legacy per UE TRP limitation as agreed by RAN4.

Observation 1: When the gNB would expect the UE to achieve maximum EIRP for both beam peak directions under STxMP mode, 3dB ‘alternative’ MPR is necessary in case the UE is risky to comply with the legacy per UE TRP limitation.

In our view the proposed formula cannot fulfil the purpose. For instance, 2dB MPR applies for edge RB allocation with QPSK DFT-s-OFDM waveform. (PC 4/5/6 share the same MPR requirement as for PC3)

For STxMP, though 3dB relaxation will be allowed by MAX(MPRlegacy, 3dB) for each panel, there would be no TRP relaxation in total, which is 2dB stringent than legacy requirement. So under the ‘equal TRP split’ scenario, the proposed formula can only be effective for those combinations of RB allocation and waveform with 0 dB MPR.

Observation 2: The total TRP relaxation with per panel MPR=MAX(MPRlegacy, 3dB) can be 3dB only when the legacy MPR requirement equals to 0dB.

So we have the following proposal for both MPR and A-MPR.

Proposal 3: If RAN4 further defines per panel MPR/A-MPR and PUMAX in Rel-18, the following definition can be considered.

- Reuse the legacy MPRf, c/A-MPRf, c for single band operation for per panel MPR/A-MPR respectively and introduce additional 3dB relaxation into the lower bound for PUMAX, f, c, k.

Another interesting issue from last meeting is the necessity to introduce new signalling for overlapping beams indication. In our view, it should be firstly clarified the definition on overlapping beam pair. It is suitable for RAN1 but not RAN4 to discuss whether such indication could be necessary in case the current measurement & reporting procedure cannot avoid the risk that the network would schedule an “overlapping” beam pair but with unexpected worse throughput performance.

Proposal 4: With clear definition on “overlapping”, RAN1 should handle the discussion on whether indication on “overlapping” beam pair could be needed in case current measurement & reporting procedure cannot avoid the risk that the network would schedule an “overlapping” beam pair but with unexpected worse throughput performance.

Conclusion

In this contribution we provide our views on STxMP operation from RF perspective, we have the following observations and proposals:

Observation 1: When the gNB would expect the UE to achieve maximum EIRP for both beam peak directions under STxMP mode, 3dB ‘alternative’ MPR is necessary in case the UE is risky to comply with the legacy per UE TRP limitation.

Observation 2: The total TRP relaxation with per panel MPR=MAX(MPRlegacy, 3dB) can be 3dB only when the legacy MPR requirement equals to 0dB.

Proposal 1: Check if RAN4 can only define PCMAX, f, c, k for STxMP in Rel-18 and postpone remaining aspects e.g. per panel PUMAX, MPR/A-MPR and potential testability issue until clear demands show up in the future.

Proposal 2: For STxMP operation, clarify that the overall output power capability would still be what can be achieved by the advertised power class considering all legacy transmitter RF requirements for single band operation as defined in TS 38.101-2 clause 6.

Proposal 3: If RAN4 further defines per panel MPR/A-MPR and PUMAX in Rel-18, the following definition can be considered.

- Reuse the legacy MPRf, c/A-MPRf, c for single band operation for per panel MPR/A-MPR respectively and introduce additional 3dB relaxation into the lower bound for PUMAX, f, c, k.

Proposal 4: With clear definition on “overlapping”, RAN1 should handle the discussion on whether indication on “overlapping” beam pair could be needed in case current measurement & reporting procedure cannot avoid the risk that the network would schedule an “overlapping” beam pair but with unexpected worse throughput performance.

- References

- R4-2314911, “WF on NR MIMO evo DL UL UERF”, Samsung, 3GPP TSG-RAN4 #108, Toulouse, France, August 21 – 25, 2023.

- R4-2314698, “LS on the PCmax requirement for STxMP in FR2”, RAN4, Qualcomm, 3GPP TSG-RAN4 #108, Toulouse, France, August 21 – 25, 2023.

- R4-2306657, “Reply LS on configured Tx power for STxMP in FR2”, RAN4, Qualcomm, 3GPP TSG-RAN4 #106bis-e, Electronic meeting, April 21 – 25, 2023.

- R4-2309280, “On configured Tx power for STxMP in FR2”, Qualcomm, 3GPP TSG-RAN4 #107, Incheon, Korea, May 22 – 26, 2023.