# 3GPP TSG-RAN WG4 Meeting # 108bis		R4-2316505

# Xiamen, China, October 09 – October 13, 2023

Agenda Item:	5.13.2.2

Source: 		Qualcomm Incorporated

Title: 	Discussion on remaining ATG UE RF requirements

Document for:	Approval

# 1 Introduction

In RAN4#108, WFs on ATG UE RF in [1] were approved with some remaining issues for ATG UE RF requirements. In this paper, we provide our views on remaining ATG UE RF requirements.

# 2 Discussion

## 2.1 Minimum distance assumption

The minimum distance assumption has been widely discussed in previous meetings. The following proposals were discussed but there was no conclusion:

Issue 2-1-1: Minimum distance assumption between ATG BS and ATG UE - Proposal: - Option 1: Considering the possibility of ATG serving the aircraft directly above in the future still exists, 3km could be used as the minimum distance between ATG UE from BS. - Option 2: 20km referring to coexistence study. - Option 3: Others

The remaining UE RF requirements including minimum output power and maximum input level are pending on the conclusion of minimum distance assumption between ATG BS and ATG UE.

In ATG co-existence study, 3km is the minimum altitude for ATG UE. But if we consider the boresight of the ATG sector and antenna uptilt, the minimum distance in the horizontal domain is given by the following table and illustration from TR 38.876.

Table 6.2.1.2-1: Simulation scenarios for ATG coexistence study

| Parameters | No sub-arrays | Sub-arrays |
| --- | --- | --- |
| Minimum ATG BS-UE horizontal distance | 20km | 50km |
| Maximum ATG BS-UE horizontal distance | 100km | 100km |

Figure 6.2.1.3-1 ATG BS layout (In horizontal boresight direction of ATG antenna)

Therefore, assuming 3km as the minimum distance between ATG BS and ATG UE does not align with the simulation scenario conducted for co-existence study.

Observation 1: Assuming 3km as the minimum distance between ATG BS and ATG UE does not align with the simulation scenario conducted for co-existence study.

As the co-existence simulation results have been well calibrated among the companies, it is reasonable to use the minimum coupling loss (MCL) from co-existence simulation as the reference to decide the minimum output power and maximum input level.

Proposal 1: To use minimum coupling loss (MCL) as the reference to decide the minimum output power and maximum input level requirements for ATG UE.

## 2.2 Minimum output power

For legacy NR FR1 UE, the minimum output power requirement was derived with the assumptions of 70dB MCL.

With the co-existence simulation calibration results (which is attached in the Annex), it could be observed that the MCL for ATG 2GHz and 4GHz are 100dB and 94dB, respectively. The gap between 2GHz and 4GHz are mainly due to the pathloss and UE antenna type assumptions, i.e., omni-directional or phased array. Considering the UE antenna type are depending on the implementation on each band, it is reasonable to define two set of requirements based on different UE antenna types.

With that, it is proposed to apply 25dB and 19dB relaxation for omni-directional and phased array UE antenna respectively considering 5dB margin.

Proposal 2: RAN4 to adopt the following minimum output power for different ATG UE antenna types.

Table 1: Minimum output power for ATG UE with Omni-directional antenna

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -15 | -15+10log10 (BWChannel /20) | -15+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

Table 2: Minimum output power for ATG UE with phased array antenna

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -21 | -21+10log10 (BWChannel /20) | -21+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

## 2.3 Maximum input level

For maximum input level, as discussed in section 2.2, the same relaxation of 25dB and 19dB for omni-directional and phased array UE antenna respectively should be applied.

Proposal 3: RAN4 to adopt the following maximum input level for different ATG UE antenna types.

Table 3: Maximum input level ATG UE with Omni-directional antenna

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in Transmission Bandwidth Configuration4 | dBm | -502 | -50 + 10log10(BWChannel /20)Note 2 | -452 |
|  |  | -523,5 | -52 + 10log10(BWChannel /20)Note 3,5 | -473,5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 5: Reference measurement channel is A.3.2.x for 1024 QAM. |  |  |  |  |

Table 4: Maximum input level ATG UE with phased array antenna

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in Transmission Bandwidth Configuration4 | dBm | -442 | -44 + 10log10(BWChannel /20)Note 2 | -392 |
|  |  | -463,5 | -46 + 10log10(BWChannel /20)Note 3,5 | -413,5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 5: Reference measurement channel is A.3.2.x for 1024 QAM. |  |  |  |  |

## 2.4 p-Max

The equation of configured transmitted power was agreed in [2].

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds: PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with PCMAX_L,f,c = MIN {PEMAX,c, [PMaxOutputPower]} PCMAX_H,f,c = PEMAX,c where PEMAX,c is the value given by [either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE], whichever is applicable according to TS 38.331[7]; [PMaxOutputPower] is the maximum UE output power at maximum modulation order and full PRB configurations which is indicated by ATG UE;

Within the equation, the range of IE p-Max will be different from the legacy IE p-Max which is given below:

Figure 1: Snippet from TS 38.331

With the upper limit of maximum UE output power of 40dBm, the range of p-Max will be changed for the ATG UE. Meanwhile, the lower limit of the range will be decided by minimum output power discussed in the section 2.2. It should be noted that the lower limit of legacy IE p-Max is somehow not in line with the minimum output power of legacy NR FR1 UE requirements. It is saying that the minimum output power requirements of legacy NR FR1 UE is -40dBm but the lower limit of p-Max is -33. Therefore, the lower limit is not necessarily consistent with minimum output power requirements. Based on the calculation of minimum output power in section 2.2, we have the following proposal on p-Max for ATG UE:

Proposal 4: RAN4 suggests extending the p-Max range of ATG UE as (-21…42).

## 2.5 Out-of-band blocking

In [3], the issue of co-existence between altimeter and ATG UE was raised referring to the altimeter transmitted PSD defined in ITU-R M.2059-0 and the assumptions of 40dB MCL is applied.

It should be noted that the co-existence between the altimeter and ATG UE on the same aircraft is depending on the frequency allocation. It means the potential issue might be identified only when ATG UE and altimeter are in the adjacent bands. In addition, the out of band interference from altimeter to ATG UE is also decided by MCL which is related to many aspects such as isolation, antenna pattern, etc. Considering the above reasons, we propose not to consider the issue of co-existence between altimeter and ATG UE at this stage. The additional requirement could be specified for specific band if the issue is identified in future. Therefore, we have the following proposal:

Proposal 5: RAN4 to reuse the legacy out-of-band blocking requirements for ATG UE.

# 3 Conclusions

This contribution provided our considerations on the remaining ATG UE requirements. We have the following observations and proposals:

Observation 1: Assuming 3km as the minimum distance between ATG BS and ATG UE does not align with the simulation scenario conducted for co-existence study.

Proposal 1: To use minimum coupling loss (MCL) as the reference to decide the minimum output power and maximum input level requirements for ATG UE.

Proposal 2: RAN4 to adopt the following minimum output power for different ATG UE antenna types.

Table 1: Minimum output power for ATG UE with Omni-directional antenna

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -15 | -15+10log10 (BWChannel /20) | -15+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

Table 2: Minimum output power for ATG UE with phased array antenna

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -21 | -21+10log10 (BWChannel /20) | -21+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

Proposal 3: RAN4 to adopt the following maximum input level for different ATG UE antenna types.

Table 3: Maximum input level ATG UE with Omni-directional antenna

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in Transmission Bandwidth Configuration4 | dBm | -502 | -50 + 10log10(BWChannel /20)Note 2 | -452 |
|  |  | -523,5 | -52 + 10log10(BWChannel /20)Note 3,5 | -473,5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 5: Reference measurement channel is A.3.2.x for 1024 QAM. |  |  |  |  |

Table 4: Maximum input level ATG UE with phased array antenna

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in Transmission Bandwidth Configuration4 | dBm | -442 | -44 + 10log10(BWChannel /20)Note 2 | -392 |
|  |  | -463,5 | -46 + 10log10(BWChannel /20)Note 3,5 | -413,5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 5: Reference measurement channel is A.3.2.x for 1024 QAM. |  |  |  |  |

Proposal 4: RAN4 suggests extending the p-Max range of ATG UE as (-21…42).

Proposal 5: RAN4 to reuse the legacy out-of-band blocking requirements for ATG UE.

# 4 References

R4-2314761, WF on NR_ATG_UERF_part2, Huawei, HiSilicon

R4-2312964, TP for TR 38.876 to introduce ATG UE Tx requirements part 1, Huawei, HiSilicon

R4-2312965, Discussion on ATG UE Rx requirements, Huawei, HiSilicon

# Annex