3GPP TSG-RAN WG4 Meeting #108bis 	R4-2316632

Xiamen, China, October 09 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation Results on NCR PDSCH and PDCCH Demodulation Requirements

Agenda item:	5.28.6

Document for:	Information

Introduction

In [1], the way forward (WF) from the discussions on NCR (Network-Controlled Repeaters) demodulation in RAN4#108 is documented. Some of the agreements in the WF related to simulations for NCR-Mobile Termination (MT) for the PDSCH requirements are summarized below:

Issue 1-1-2: MCS for PDSCH FR2 requirements - Agreements: - MCS 4 is for PDSCH FR2. - QPSK and 16QAM for PDSCH FR1. Issue 1-1-3: Test scope for PDSCH FR1 - Agreements: Case number; Bandwidth (MHz); SCS (kHz); MCS; Propagation condition; Antenna configuration; Test metric. 1; 10; 15; 4; TDLA30-10; 2x2; 70% max throughput 1% BLER. 2; 10; 15; 4; TDLA30-10; 2x4; 70% max throughput 1% BLER. 3; 10; 15; 13; TDLA30-10; 2x2; 70% max throughput 1% BLER. 4; 10; 15; 13; TDLA30-10; 2x4; 70% max throughput 1% BLER. 5; 40; 30; 4; TDLA30-10; 2x2; 70% max throughput 1% BLER. 6; 40; 30; 4; TDLA30-10; 2x4; 70% max throughput 1% BLER. 7; 40; 30; 13; TDLA30-10; 2x2; 70% max throughput 1% BLER. 8; 40; 30; 13; TDLA30-10; 2x4; 70% max throughput 1% BLER. Issue 1-1-4: Test scope for PDSCH FR2 - Agreements: Case number; Bandwidth (MHz); SCS (kHz); Propagation condition; Antenna configuration; Test metric. 1; 100; 120; TDLA30-75; 2x2; 70% max throughput 1% BLER.

Furthermore, some of the agreements in the WF related to simulations on NCR-Mobile Termination (MT) for the PDCCH requirements are summarized below:

Issue 1-2-1: Whether to define new requirements on PDCCH for signaling of Access link beam change indication - Agreements: - FFS whether new PDCCH requirements needed or not for DCI format 5_0/2_8 with large payload size. Issue 1-2-2: Test scope for PDCCH FR1 - Agreements: - Following table used for simulation purpose on the requirements with existing DCI if introduced. Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Propagation Condition; Antenna configuration and correlation Matrix. 10; 48; 1; 8; TDLA30-10; 2x4 Low. Issue 1-2-3: Test scope for PDCCH FR2 - Agreements: - No new simulation for existing DCI , FFS for new DCI if introduced.

Simulation Results on PDSCH Requirements

In this paper, we provide simulation results on PDSCH requirements for the test metrics and test cases agreed in RAN4#108. Our observations and proposals in details on the remaining open issues on NCR-MT demodulation can be found in [2].

While there are already agreements on the test cases, nonetheless, there are no details on the simulation parameters discussed by the companies in RAN4#108. Therefore, similar to our simulation results presented in RAN4#108 in [5], we are using related simulation parameters of UE and IAB-MT PDSCH requirements as in [3] and [4]. We note here that for 1% BLER we consider it from the 1st transmission without HARQ retransmission, while for 70% Throughput from the HARQ retransmission with number of retransmissions 4.

PDSCH Simulation in FR1

In the following, we provide the results of our simulation on NCR PDSCH requirements in FR1, i.e., using FR1 FRCs. While the test cases already been defined and agreed in RAN4#108, nonetheless the corresponding FRCs were not yet defined. In our simulation, if similar test cases can be found in IAB-MT specification, we use the corresponding FRCs of IAB-MT. Otherwise, we use the FRCs as in UE specification. Our simulation results on PDSCH in FR1 are tabulated in Table 1 below.

Table 1. Simulation Results on PDSCH in FR1

| Case number | Bandwidth (MHz) | SCS (kHz) | MCS | Propagation condition | Antenna configuration | Reference Values |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) | BLER (%) | SNR (dB) |
| 1 | 10 | 15 | 4 | TDLA30-10 | 2x2 | 70 | -2.39 | 1 | 0.6 |
| 2 | 10 | 15 | 4 | TDLA30-10 | 2x4 | 70 | -4.77 | 1 | -2.09 |
| 3 | 10 | 15 | 13 | TDLA30-10 | 2x2 | 70 | TBA | 1 | TBA |
| 4 | 10 | 15 | 13 | TDLA30-10 | 2x4 | 70 | 1.66 | 1 | 4.98 |
| 5 | 40 | 30 | 4 | TDLA30-10 | 2x2 | 70 | -2.79 | 1 | -0.13 |
| 6 | 40 | 30 | 4 | TDLA30-10 | 2x4 | 70 | TBA | 1 | TBA |
| 7 | 40 | 30 | 13 | TDLA30-10 | 2x2 | 70 | 4.09 | 1 | 7.19 |
| 8 | 40 | 30 | 13 | TDLA30-10 | 2x4 | 70 | 1.72 | 1 | 4.09 |

PDSCH Simulation in FR2

In the following, we provide our simulation results of NCR PDSCH requirements in FR2, i.e., using FR2 FRCs. While the test cases already been defined and agreed in RAN4#108, nonetheless the corresponding FRC was not yet defined. In our simulation, we use the FRC as in IAB-MT specification. The simulation results on PDSCH in FR2 is tabulated in Table 2 below.

Table 2. Simulation Results on PDSCH in FR2

| Case number | Bandwidth (MHz) | SCS (kHz) | Propagation condition | Antenna configuration | Reference Values |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) | BLER (%) | SNR (dB) |
| 1 | 100 | 120 | TDLA30-75 | 2x2 | 70 | 3.86 | 1 | 5.27 |

Simulation Results on PDCCH Requirements

As stated in the WF [1], companies are encouraged to provide simulation results on PDCCH for DCI Type 2_8. In [2], we have provided our observations and proposals related to the need of having new PDCCH requirements for NCR-MT due to the use of DCI Type 5_0/2_8, which has higher payload than DCI Type 1_0 and 1_1 that are currently available in the specifications. In this paper, we provide our simulation results of PDCCH for DCI Type 5_0/2_8, in which we adapt the DCI payload size according to the maximum value for DCI Type 5_0/2_8, which is 128 bits.

PDCCH Simulation in FR1

We will first provide the simulation results on PDCCH FR1 TDD, followed by PDCCH FR1 FDD.

PDCCH FR1 TDD

Simulation results on NCR-MT PDCCH FR1 TDD are tabulated below:

Table 3. Simulation Results on PDCCH FR1 TDD: Legacy DCI vs. DCI 5_0/2_8

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value (Legacy DCI 1_0) |  | Reference value (DCI 5_0/2_8) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) | Pm-dsg (%) | SNR (dB) |
| 40 | 102 | 1 | 2 | TDLA30-10 | 1x2 Low | 1 | 3.4 | 1 | 10.6 |
| 40 | 102 | 1 | 4 | TDLA30-10 | 1x2 Low | 1 | 1.0 | 1 | 4.5 |
| 40 | 90 | 1 | 8 | TDLA30-10 | 2x2 Low | 1 | -1.4 | 1 | 1.4 |
| 40 | 102 | 1 | 2 | TDLA30-10 | 1x4 Low | 1 | -1.6 | 1 | 3.2 |
| 40 | 102 | 1 | 4 | TDLA30-10 | 1x4 Low | 1 | -3.7 | 1 | -0.8 |
| 40 | 90 | 1 | 8 | TDLA30-10 | 2x4 Low | 1 | -6.0 | 1 | -3.5 |

PDCCH FR1 FDD

Simulation results on NCR-MT PDCCH FR1 FDD are tabulated below:

Table 4. Simulation Results on PDCCH in FR1 FDD: Legacy DCI vs. DCI 5_0/2_8

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value (Legacy DCI 1_0) |  | Reference value (DCI 5_0/2_8) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) | Pm-dsg (%) | SNR (dB) |
| 10 | 24 | 2 | 2 | TDLA30-10 | 1x2 Low | 1 | 7.3 | 1 | 12.2 |
| 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 Low | 1 | 4.7 | 1 | 7.7 |
| 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 Low | 1 | 0.7 | 1 | 3.4 |
| 10 | 24 | 2 | 2 | TDLA30-10 | 1x4 Low | 1 | -0.2 | 1 | 4.4 |
| 10 | 48 | 2 | 4 | TDLA30-10 | 1x4 Low | 1 | -2.0 | 1 | 0.7 |
| 10 | 48 | 1 | 8 | TDLA30-10 | 2x4 Low | 1 | -4.3 | 1 | -2.0 |

PDCCH Simulation in FR2

Simulation results on NCR-MT PDSCH FR2 TDD are tabulated below:

Table 5. Simulation Results on PDCCH in FR2: Legacy DCI vs. DCI 5_0/2_8

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value (Legacy DCI 1_0) |  | Reference value (DCI 5_0/2_8) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) | Pm-dsg (%) | SNRBB (dB) |
| 100 | 60 | 1 | 2 | TDLA30-75 | 1x2 Low | 1 | 3.0 | 1 | 9.5 |
| 100 | 60 | 1 | 4 | TDLA30-75 | 1x2 Low | 1 | 0.6 | 1 | 3.7 |
| 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 Low | 1 | -2.1 | 1 | 0.3 |

Conclusion

In this paper, we provide our simulation results on PDSCH and PDCCH for NCR-MT demodulation requirements. The PDSCH simulation results take into account the new agreed metrics, namely, BLER 1%. The PDCCH simulation results are using the payload size of DCI Type 5_0/2_8. It can be seen that with the higher number of payload size of DCI 5_0/2_8, higher SNR is required as compared to DCI Type 1_0 and 1_1 specified in IAB and UE specifications. More on our observations and proposal can be found in [2].

- References

- R4-2313923, WF on NCR_MT demodulation requirements, ZTE Corporation, 3GPP RAN4#108, Toulouse, France, August 21 – 25, 2023

- R4-2316631, NCR Demodulation Performance Requirements, Nokia, Nokia Shanghai Bell, Xiamen, China, October 09 – 13, 2023

- 3GPP TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements, V17.7.0, 12-2022

- 3GPP TS 38.174, Integrated access and backhaul radio transmission and reception, V17.2.0, 12-2022

- R4-2312803, Simulation Results on NCR-MT PDSCH and PDCCH Demodulation Requirements, Nokia, Nokia Shanghai-Bell, 3GPP RAN4#108, Toulouse, France, August 21 – 25, 2023