3GPP TSG-RAN WG4 Meeting #108bis	R4-2317641

Xiamen, China, 9th October ‒ 13th October, 2023                            (was R4-2317639)

Title: 	WF on IoT NTN Extended L-band

Agenda Item:	6.3.5

Source: 	Moderator (Inmarsat)

Document for:	Approval

# Topic #2: UE RF Requirements

### Sub-topic 2-1

General UE Parameters

Issue 2-1-1: Channel raster, carrier frequency and EARFCN

Agreement:

  - Maintain channel raster of 100 kHz. Table 2.1.1 from Proposal 1 can be agreed as a requirement for Band 253.

- Table 2.1-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 253 | 100 | 1518 | 228501 | 228501-<1>-228570 | 1668 | 261269 | 261269-<1>-261338 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

  - The need for additional flexibility in channel raster for IoT NTN LTE bands has been identified, especially to accommodate future deployments of NB-IoT NTN and eMTC NTN together with NR NTN around existing services, taking into account possible guard bands for protection of other services, Doppler, and for deployment within the same SAN (i.e. same FFT).  This issue could be further discussed independently of band 253, with more general applicability to all IoT NTN LTE bands.

    - Capture this aspect in the TR.

    - FFS whether to capture a note in the specification

Issue 2-1-2: Default UE TX-RX frequency separation

Agreement:

  - Agree the follow bullets as a starting point.

  - Agree on the following Default TX-RX separation:

- Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 253 | -150 MHz |

  - Based on operator input, there is a strong desire to consider flexible TX-RX separation for the band itself, and moreover for NTN FR1 bands, provided the frequency separation is sufficiently large.  For next meeting, companies to further check if this can be supported.

### Sub-topic 2-2

Issue 2-2-1: REFSENS

- Tentative Agreement:

  - Companies to further check the values of REFSENS to clarify original background assumptions for values inherited from TN specs

Issue 2-2-2, 2-2-3: Blocking requirements for Cat M1 and Cat NB1 and NB2

- Tentative Agreements:

  - Further discuss how to reconcile blocking requirements from EN 301 681 and ECC Report 263 with 3GPP blocking requirements and coexistence assumptions, taking also into account alignment with ongoing ETSI SES work.

    - Companies to check whether existing 3GPP blocking requirements can be sufficient to meet regulatory requirements.

# Topic #3: SAN RF Requirements

### Sub-topic 3-1

Issue 3-1-1: Channel Raster and EARFCN

- Agreement:

  - Maintain channel raster of 100 kHz. Table 2.1.1 from Proposal 1 can be agreed as a requirement for Band 253.

- Table 2.1-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 253 | 100 | 1518 | 228501 | 228501-<1>-228570 | 1668 | 261269 | 261269-<1>-261338 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

  - The need for additional flexibility in channel raster for IoT NTN LTE bands has been identified, especially to accommodate future deployments of NB-IoT NTN and eMTC NTN together with NR NTN around existing services, taking into account possible guard bands for protection of other services, Doppler, and for deployment within the same SAN (i.e. same FFT).  This issue could be further discussed independently of band 253, with more general applicability to all IoT NTN LTE bands.

    - Capture this aspect in the TR.

    - FFS whether to capture a note in the specification