3GPP TSG-RAN WG4 Meeting #108bis                                   R4-2317647

Xiamen, China, Oct 9th – 13th, 2023

Agenda Item:	6.4.3

Source:	MediaTek Inc.

Title: 	WF on new FDD band (L+S band) for IoT NTN

Document for:	Discussion

Issue 1-1-1: For A-MPR for Cat. M1, which of the following options should be specified in TS 36.102 for B254?

- Proposals

  - Option 1: No A-MPR is specified.

  - Option 2: Others, please elaborate.

- Recommended WF

  - Option 1.

Agreement:

- Agree on Option 1.

Issue 1-1-2: For A-MPR for Cat. NB1/NB2, which of the following options should be specified in TS 36.102 for B254?

- Proposals

  - Option 1: No A-MPR is specified

  - Option 2: Others, please elaborate.

- Recommended WF

  - To be discussed.

Issue 1-1-3: For B254, which of the following options should be specified as NS flag(s) to support ETSI regulatory requirements?

- Proposals

  - Option 1: One NS value as below

| NS flag | Carrier frequency range |
| --- | --- |
| NS_04N | 1610 MHz – 1618.25 MHz |
|  | 1618.25 MHz – 1626.5 MHz |

  - Option 2: Two NS values as below

| NS value | Regulatory domain | Applicable range (MHz) | In-band emissions | Out-of-band emissions |
| --- | --- | --- | --- | --- |
| NS_04N | ETSI | 1610-1618.25 | ETSI (1610-1618.25) | ETSI (1559-1610MHz) ETSI (>1628.5MHz) |
| NS_05N |  | 1618.25-1626.5 | ETSI (1618.25-1626.5) |  |

  - Option 3: Pending capturing ETSI requirements

- Recommended WF

  - To be discussed.

Issue 1-2-1 : For occupied bandwidth, should the existing TS 36.102  requirements be re-used for B254?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Option 1.

Agreement: agree on Option1.

Issue 1-2-2 : For out of band emission, should the existing TS 36.102  requirements be re-used for B254?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Option 1.

Agreement: agree on Option1.

Issue 1-2-3 : For general spurious emission, should the existing TS 36.102  requirements be re-used for B254?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Option 1.

Agreement: agree on Option1.

Issue 1-2-4: For Cat. NB1/NB2 spurious emission band UE co-existence, should the existing TS 36.102  requirements for Cat. M1 be re-used for B254?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Option 1.

Agreement: agree on Option1.

Issue 1-2-5 : For Tx inter-modulation, should the existing TS 36.102  requirements be re-used for B254?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Option 1.

Agreement: agree on Option1.

Issue 2-1-1: Should the note “The power level of the interferer (Pinterferer) for Range 3 shall be modified to -20dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz” be introduced for B254?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - To be discussed.

Agreement: agree on Option1.

Issue 2-1-2: For ACS, should the existing TS 36.102 requirements be re-used for B254?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Option 1.

Agreement: agree on Option1.

Issue 2-2: For Rx inter-modulation, should the existing TS 36.102 requirements be re-used for B254?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Option 1.

Agreement: agree on Option1.

# References

- R4-2315678 Discussion on UE RF requirements for FDD band (L+S band) for IoT NTN operation,	ZTE Corporation

- R4-2315679 Draft CR to TS36.102 Introduction of a new FDD band (L+S band) for IoT NTN operation,	ZTE Corporation

- R4-2315680 Draft CR to TS36.108 Introduction of a new FDD band (L+S band) for IoT NTN operation,	ZTE Corporation

- R4-2315681 Draft CR to TS36.133 Introduction of a new FDD band (L+S band) for IoT NTN operation,	ZTE Corporation

- R4-2316131 DraftCR to TS 36.102 on Blocking requirement for IoT NTN operation,	OPPO

- R4-2316132 further discusiion on IOT NTN LS band,	OPPO

- R4-2316133 TP to Draft TR for IoT NTN bands,	OPPO

- R4-2316657 Running draftCR to TS 36.102 on introducing L+S FDD band for IoT NTN operation,	MediaTek Inc.

- R4-2316658 DraftCR to TS 36.102 on A-MPR for B254 for IoT NTN operation,	MediaTek Inc.

- R4-2316659 Draft TR for IoT NTN bands v0.0.3,	 MediaTek Inc.

- R4-2316660 Further discussion on UE RF requirements for B254 for IoT NTN operation,	MediaTek Inc.

- R4-2317572 Further discussion on UE RF requirements for B254 for IoT NTN operation,	MediaTek Inc.

- R4-2317646 Running draftCR to TS 36.102 on introducing L+S FDD band for IoT NTN operation, MediaTek Inc.

- R4-2317242 Topic summary for [108-bis][118] IoT_NTN_FDD_LS_band, Moderatro(MediaTek)