3GPP TSG-RAN WG4 Meeting #109	R4-2321863

Chicago, USA, 13th November – 17th November 2023

Source:	ZTE Corporation

Title:	TP for TR 38.718-03-01_3DL_xUL CA_n34A-n41A-C-n79

Agenda Item:	7.11.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce 3DL/2UL CA_n34A-n41A-n79 in TR 38.718-03-01.

- Reference

- [1] TR38.718-03-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for3 bands DL with x bands UL (x=1,2),v0.0.1, v0.2.0

- Text Proposal

----- Start of TP -----

  - 5.x			CA_n34-n41-n79

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| n34-n41-n79 | n34, n41, n79 |

      - 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n34+n41+n79

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n34A-n41A-n79A | CA_n34A-n41A CA_n34A-n79A CA_n41A-n79A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n41C-n79A | CA_n34A-n41A CA_n34A-n79A CA_n41A-n79A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |

      - 5.x.1.3	∆TIB,c and ∆RIB,c values

The TIB,c and RIB,c values are defined in the following:

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| n34-n41-n79 | 0.3 | 0.5 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.x.1.3-1: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| n34-n41-n79 | - | 0.5 | 0.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

    - 5.x.2	Specific for 2 bands UL CA

      - 5.x.2.1	UE co-existence studies

The co-existence for the fallback 2DL/2UL of CA_n34A-n41A, CA_n34A-n79A and CA_n41A-n79A have already been analyzed. In terms of the co-existence studies of corresponding fallbacks, it can be observed:

IMD2 caused by 2UL  n34+n41 may fall into the its own band n79 Rx

IMD2 caused by 2UL  n34+n79 may fall into the its own band n41 Rx

IMD2 and IMD5 caused by 2UL  n41+n79 may fall into the its own band n34 Rx

      - 5.x.2.2	REFSENS requirements

Based on co-existence studies additional MSD is needed to be defined, shown in table 5.x.2.2-1.

Table 5.x.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n34-n41-n79 | n34 | 2020 | 5 | 25 | 2020 | N/A | N/A |
|  | n41 | 2660 | 5 | 25 | 2660 | N/A | N/A |
|  | n79 | 4680 | 40 | 216 | 4680 | 19.3 | IMD2 |
|  | n34 | 2020 | 5 | 25 | 2020 | N/A | N/A |
|  | n41 | 2550 | 5 | 25 | 2550 | 27.2 | IMD2 |
|  | n79 | 4570 | 40 | 216 | 4570 | N/A | IN/A |
|  | n34 | 2015 | 5 | 25 | 2015 | 28.6 | IMD2 |
|  | n41 | 2585 | 5 | 25 | 2585 | N/A | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | N/A |
|  | n34 | 2015 | 5 | 25 | 2015 | 7.5 | IMD5 |
|  | n41 | 2515 | 5 | 25 | 2515 | N/A | N/A |
|  | n79 | 4780 | 40 | 216 | 4780 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

----- End of TP -----