3GPP TSG-RAN WG4 Meeting #109						                 					     R4-2318606

Chicago, USA, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on FR2 SCell/SCG setup delay improvement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Solution based on existing measurement for improvement on SCG/SCell setup delay is stable in RAN4. However, corresponding RRM requirements are still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introudce RRM requirement for solution based on existing measurement for improvement on SCG/SCell setup delay |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM requirement for solution based on existing measurement for improvement on SCG/SCell setup delay would still be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 4.x, 5.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

## 4.x	Measurement report for fast CA/DC setup

### 4.x.1	Introduction

A UE supporting [solution based on existing measurement] shall perform the idle mode measurement on the inter-frequency CA and DC candidate frequencies/cells indicated by higher layers and meet the requirement specified in this clause. The UE shall perform idle mode measurements provided that the serving cell support early measurement and is within the validity area. The idle mode measurement requirements apply to a configured carrier frequency and the serving cell are among the supported band combination of the UE.

### 4.x.2	Measurement Requirements

For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 the UE shall support the idle mode CA measurements on the serving cell, and carriers configured for idle mode CA/DC measurement and meet corresponding measurement requirements defined in clause 4.4.2.

[For a UE which supports [solution based on existing measurement] but not idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 the UE shall support the idle mode CA measurements on the serving cell, and carriers configured for idle mode CA/DC measurement and meet corresponding measurement requirements defined in clause 4.2.2.]

### 4.x.3	Measurement Report Requirements

For a UE which supports [solution based on existing measurement] the UE shall be able to report valid measurement results upon RRC setup complete. The measurement results are considered valid if the following conditions are met:

- the measurements are performed within the last [X] seconds before it is reported, where [X] is configured by [TBD], and

- the measurement results can meet corresponding accuracy requirement at the measurement instance.

Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results.

The UE shall be able to report idle mode CA/DC measurements when idle mode CA/DC measurement reporting is requested by the network.

End of Change 1

Start of Change 2

## 5.x	Measurement report for fast CA/DC setup

### 5.x.1	Introduction

A UE supporting [solution based on existing measurement] shall perform the inactive mode measurement on the inter-frequency CA and DC candidate frequencies/cells indicated by higher layers and meet the requirement specified in this clause. The UE shall perform idle mode measurements provided that the serving cell support early measurement and is within the validity area. The idle mode measurement requirements apply to a configured carrier frequency and the serving cell are among the supported band combination of the UE.

### 5.x.2	Measurement Requirements

The requirements in clause 4.x.2 shall apply.

### 5.x.3	Measurement Report Requirements

The requirements in clause 4.x.3 shall apply.

End of Change 2