3GPP TSG-RAN WG4 Meeting #109                                 	                               R4-2318674

# Chicago, USA, November 13 – November 17, 2023

Agenda item:	8.3.3.1.4

Source:	Apple Inc.

Title:	Discussion on CSI Requirements for 8Rx UEs

Release:	Rel-18

Document for:	Discussion

# Introduction

Overall, this WI has been focusing on:

- Enable 8Rx for CPE/FWA/vehicle/industrial devices

  - Specify UE demodulation performance and CSI requirements with up to 8 layers to support 8Rx

    - Investigate and, if necessary, specify the requirements with up to 8 DL MIMO layers

  - Specify the SDR requirements with 8 MIMO layers

In addition, as per the revised WID RP-231490 in RAN#100, we wait for RAN4 RF to specify UE RF requirements for both single carrier and CA/DC for 8Rx. About specific band combos and configurations, these are FFS.

  - Specify the UE RF requirements to support 8Rx for both single carrier and CA/DC

    - Example band combos and configurations need to be defined

The main aim of this paper is to summarize the status and still open discussions related to PDSCH demodulation requirements for 8Rx UEs.

# Discussion

During RAN4#108 and RAN4#108bis, several issues have been closed and all agreements have been reached. As a summary, we state in what follows:

TDD Requirements

Report quantity

- two-one-TypeI-SinglePanel-Restriction = 00000001

- RI Restriction = 00001000

- TE schedules fixed PMI matrix with i1,1=i1,2= i2=i1,3= 0

SNR points

[4, 5]dB and [10, 11]dB

FDD Requirements

Test parameters

- 10 MHz/15 kHz, 52 RBs.

- CQI index for up to 64QAM.

- Rank 4.

Report quantity

Same as TDD requirements

- two-one-TypeI-SinglePanel-Restriction = 00000001

- RI Restriction = 00001000

- TE schedules fixed PMI matrix with i1,1=i1,2= i2=i1,3= 0

SNR points

[4, 5]dB and [10,11]dB

Finally, the last agreement was to not consider CQI reporting tests for the 8Rx CA case. Since all open items are closed, this agenda item should be declared closed.

Proposal#1: RAN4 to confirm during RAN4#109 that since no more open items are to be discussed, and therefore, the CSI agenda item should be closed.

# Conclusion

In this contribution we provided our view on 8Rx UE CQI reporting requirements. We use this contribution to clarify our stance with respect to

Proposal#1: RAN4 to confirm during RAN4#109 that since no more open items are to be discussed, and therefore, the CSI agenda item should be closed.

# References

- R4-2316914, “WF for 8Rx and 4Tx performance requirements”, Huawei, HiSilicon, RAN4#108bis