3GPP TSG-RAN WG4 Meeting #109	R4-2319014

Chicago, USA, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on MRTD/MTTD and interruption requirements due to introduction of new BS signaling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NonCol_intraB_ENDC_NR_CA-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update MRTD/MTTD and interruption requirements due to introduction of new BS signalling for UE type. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To update the MRTD/MTTD and interruption requirements for FR1 non-collocated intra-band NR CA to add the condition of indication of UE type in RRC signalling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for MRTD/MTTD and interruption requirements for FR1 non-collocated intra-band NR CA are not completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.4, 7.6.4, 8.2.1.2.3, 8.2.1.2,4, 8.2.2.2.1, 8.2.2.2.2, 8.2.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RF discussion is still ongoing. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of change 1>

### 7.5.4	Minimum Requirements for NR Carrier Aggregation

The UE shall be capable of handling at least a relative transmission timing difference between slot timing of all pairs of TAGs in FR1 and/or FR2-1 as shown in Table 7.5.4-1, provided that the UE is:

-	configured with the pTAG and the sTAG for inter-band NR carrier aggregation in SA or NR-DC mode, or

-	configured with more than one sTAG for inter-band NR carrier aggregation in EN-DC or NE-DC mode.

The UE shall be capable of handling at least a relative transmission timing difference between subframe timing of all pairs of TAGs between FR1 and FR2-2 as shown in Table 7.5.4-1, provided that the UE is:

-	configured with the pTAG and the sTAG for inter-band NR carrier aggregation in SA or NR-DC mode.

For FR1 intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative transmission timing difference as shown in Table 7.5.4-1, between slot timing of all FR1 pairs of TAGs provided that UE indicates that it is capable of [intraBandNRCA-NonCollocated-r18] and [New BS signalling] is indicated [2].

Table 7.5.4-1: Maximum uplink transmission timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | 34.6 |
| FR2-1 | 8.5 Note1 |
| Between FR1 and FR2-1 | 26.1 |
| Between FR1 and FR2-2 | 26.1 |
| Note1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA. |  |

# < End of change 1>

# < Start of change 2>

### 7.6.4	Minimum Requirements for NR Carrier Aggregation

For intra-band contiguous CA, only co-located deployment is applied. For intra-band non-contiguous NR carrier aggregation, the UE not capable of [intraBandNRCA-NonCollocated-r18] shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in Table 7.6.4-1 below.

For FR1 intra-band non-contiguous NR carrier aggregation, the UE capable of [intraBandNRCA-NonCollocated-r18] shall be capable of handling at least a relative receive timing difference as shown in 7.6.4-2, between slot timing of different FR1 carriers to be aggregated at the UE receiver provided [New BS signalling] is indicated [2]. Otherwise, the UE capable of [intraBandNRCA-NonCollocated-r18] shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in Table 7.6.4-1 below provided [New BS signalling] is not indicated.

Table 7.6.4-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation

| Frequency Range | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 31 |
| FR2-1 | 0.26 |
| Note 1: In the case of different SCS on different CCs, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot. |  |

For inter-band NR carrier aggregation,

-	the UE shall be capable of handling at least a relative receive timing difference between slot timing of all pairs of carriers in FR1 and FR2-1 to be aggregated at the UE receiver as shown in Table 7.6.4-2 below.

-	the UE shall be capable of handling at least a relative receive timing difference between subframe timing of all pairs of carriers in FR1 and FR2-2 to be aggregated at the UE receiver as shown in Table 7.6.4-2 below.

Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2-1 | 8 note1 |
| Between FR1 and FR2-1 | 25 |
| Between FR1 and FR2-2 | 25 |
| Note1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA. |  |

# < End of change 2>

# < Start of change 3>

##### 8.2.1.2.3	Interruptions at SCell addition/release

The requirements in this clause shall apply for the UE configured with PSCell.

When one E-UTRA SCell in MCG is added or released:

-	the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X1 slot, if the active serving cell is not in the same band as any of the E-UTRA SCells being added or released, or

-	of up to X1 slot, if the active serving cell is in the band overlapping or partially overlapping with the E-UTRA SCell being added or released and UE indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair, or

-	of up to max{Y1 slot + TSMTC_duration, 5ms}, if the active serving cells are in the same band or in the band overlapping or partially overlapping with the E-UTRA SCells being added or released, and UE does not indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair,provided the cell specific reference signals from the active serving cells and the E-UTRA SCells being added or released are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above active serving cells in SCG;

Where X1 and Y1 are specified in Table 8.2.1.2.3-1.

When one SCell in SCG is added or released:

-	the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X1 slot, if the active serving cell and the SCell being added or released are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X1 slot, if the active serving cell and the SCell being added or released are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair

-	of up to X1 slot, if the active serving cell is non-contiguous to the SCell being added or released in the same FR1 band and UE is capable of [intraBandNRCA-NonCollocated-r18] and [New BS signalling] is indicated [2].

or

-	of up to Y1 slot + TSMTC_duration, if the active serving cells are contiguous to any of the SCells being added or released in the same FR1 band, or if the active serving cells are in the same FR1 band as any of the SCells being added or released and UE is not capable of [intraBandNRCA-NonCollocated-r18] or [New BS signalling] is not indicated [2], provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being added when one SCell is added. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being added, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being added is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being added. If no SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being added, TSMTC duration for the SCell being added is 0 ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is released.

-	of up to Y1 slot + TSMTC_duration if the active serving cells are in the same FR2 band as any of the SCells being added or released, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being added when one SCell is added. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being added, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being added is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being added. If no SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being added, TSMTC duration for the SCell being added is 0 ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is released.

Where X1 and Y1 are specified in Table 8.2.1.2.3-2.

Table 8.2.1.2.3-1: Interruption length X1 and Y1 at E-UTRA SCell addition/Release

|  | NR Slot length | Interruption length X1 (slots) |  | Interruption length Y1 (slots) |  |
| --- | --- | --- | --- | --- | --- |
|  | (ms) | Sync | Async | Sync | Async |
| 0 | 1 | 1 | 2 | 1 | 2 |
| 1 | 0.5 | 2 | 3 | 2 | 3 |
| 2 | 0.25 | 5 |  | 4 | 5 |
| 3 | 0.125 | 9 |  | N/A | - N/A |

Table 8.2.1.2.3-2: Interruption length X1 and Y1 at SCell addition/Release

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) |  | Interruption length Y1 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 2 |  | 2 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 | 4 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |  |

##### 8.2.1.2.4	Interruptions at SCell activation/deactivation

The requirements in this clause shall apply for the UE configured with PSCell and one SCell.

When one E-UTRA SCell in MCG is activated from deactivated or dormant state, or deactivated from activated or dormant state:

-	the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X2 slot, if the active serving cell is not in the same band as any of the E-UTRA SCells being activated or deactivated, or

-	of up to X2 slot, if the active serving cell is in the band overlapping or partially overlapping with the E-UTRA SCell being activated or deactivated and UE indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair, or

-	of up to max{Y2 slot + TSMTC_duration, 5ms}, if the active serving cells are in the same band or in the band overlapping or partially overlapping with the E-UTRA SCells being activated or deactivated, and UE does not indicate it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair, provided the cell specific reference signals from the active serving cells and the E-UTRA SCells being activated or deactivated are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above active serving cells in SCG.

Where X2 and Y2 are specified in Table 8.2.1.2.4-1.

When one SCell in SCG is activated or deactivated:

-	an interruption on any active serving cell in SCG:

-	of up to X2 slot, if the active serving cell and the SCell being activated or deactivated are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X2 slot, if the active serving cells and the SCells being activated or deactivated are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair.

-	of up to X1 slot, if the active serving cell is non-contiguous to the SCell being activated or deactivated in the same FR1 band and UE is capable of [intraBandNRCA-NonCollocated-r18] and [New BS signalling] is indicated [2].

or

-	of up to Y1 slot + TSMTC_duration, if the active serving cells are contiguous to any of the SCells being activated or deactivated in the same FR1 band, or if the active serving cells are in the same FR1 band as any of the SCells being activated or deactivated and UE is not capable of [intraBandNRCA-NonCollocated-r18] or [New BS signalling] is not indicated [2],  provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being activated when one SCell is activated. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being activated, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being activated is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being activated. If no SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being activated, TSMTC duration for the SCell being activated is 0ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is deactivated.

-	of up to Y2 slot + TSMTC_duration if the active serving cells are in the same FR2 band as any of the SCells being activated or deactivated, provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being activated when one SCell is activated. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being activated, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being activated is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being activated. If no SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being activated, TSMTC duration for the SCell being activated is 0ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is deactivated.

Where X2 and Y2 are specified in Table 8.2.1.2.4-2.

Table 8.2.1.2.4-1: Interruption length X2 and Y2 at E-UTRA SCell activation/deactivation

|  | NR Slot length | nterruption length X2 (slots) |  | Interruption length Y2 (slots) |  |
| --- | --- | --- | --- | --- | --- |
|  | (ms) | Sync | Async | Sync | Async |
| 0 | 1 | 1 | 2 | 1 | 2 |
| 1 | 0.5 | 1 | 2 | 1 | 2 |
| 2 | 0.25 | 3 |  | 2 | 3 |
| 3 | 0.125 | 5 |  | N/A | N/A |

Table 8.2.1.2.4-2: Interruption length X2 and Y2 at SCell activation/deactivation

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 1 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |  |

# < End of change 3>

# < Start of change 4>

##### 8.2.2.2.1	Interruptions at SCell addition/release

When any number of SCells between one and 7 is added or released using the same RRCConnectionReconfiguration message as defined in TS 38.331 [2], the UE is allowed an interruption on any active serving cell during the RRC reconfiguration procedure as follows:

-	an interruption on any active serving cell:

-	of up to X1 slot, if the active serving cell and the SCell being added or released are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X1 slot, if the active serving cell and the SCell being added or released are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair.

-	of up to X1 slot, if the active serving cell is non-contiguous to the SCell being added or released in the same FR1 band and UE is capable of [intraBandNRCA-NonCollocated-r18] and [New BS signalling] is indicated [2].

Where X1 is specified in Table 8.2.2.2.1-1.

or

-	of up to the duration shown in table 8.2.2.2.1-2, if the active serving cells are contiguous to any of the SCells being added or released in the same FR1 band, or if the active serving cells are in the same FR1 band as any of the SCells being added or released and UE is not capable of [intraBandNRCA-NonCollocated-r18] or [New BS signalling] is not indicated [2], provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot.

-	of up to the duration shown in table 8.2.2.2.1-2, if the active serving cells are in the same band as any of the SCells being added or released, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot.

Table 8.2.2.2.1-1: Interruption length X1 for SCell addition/release for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 | 1 |  |
| 1 | 0.5 | 2 |  |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 33 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 65 |

Table 8.2.2.2.1-2: Interruption duration for SCell addition/release for intra-band CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 8 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 32+ TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 64+ TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being added when one SCell is added. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being added, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being added is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being added. If no SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being added, TSMTC duration for the SCell being added is 0ms; - the longest SMTC duration among all active serving cells in the same band when one SCell is released. NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |

##### 8.2.2.2.2	Interruptions at SCell activation/deactivation

When an SCell is activated or deactivated as defined in TS 37.340 [17], the UE is allowed

-	an interruption on any active serving cell:

-	of up to X2 slot, if the active serving cell and the SCell being activated or deactivated are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X2 slot, if the active serving cell and the SCell being activated or deactivated are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair.

-	of up to X2 slot, if the active serving cell is non-contiguous to the SCell being activated or deactivated in the same FR1 band and UE is capable of [intraBandNRCA-NonCollocated-r18] and [New BS signalling] is indicated [2].

Where X2 is specified in Table 8.2.2.2.2-1.

or

-	of up to the duration shown in table 8.2.2.2.2-2, if the active serving cells are contiguous to any of the SCells being activated or deactivated in the same FR1 band, or if the active serving cells are in the same FR1 band as any of the SCells being activated or deactivated and UE is not capable of [intraBandNRCA-NonCollocated-r18] or [New BS signalling] is not indicated [2], provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot.

-	of up to the duration shown in table 8.2.2.2.2-2, if the active serving cells are in the same FR2 band as any of the SCells being activated or deactivated provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot.

Table 8.2.2.2.2-1: Interruption length X2 for SCell activation/deactivation for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 |

Table 8.2.2.2.2-2: Interruption duration for SCell activation/deactivation for intra-band CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 16+ TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 32+ TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being activated when one SCell is activated. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being activated, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being activated is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being activated. If no SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being activated, TSMTC duration for the SCell being activated is 0ms; - the longest SMTC duration among all active servingNOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |

##### 8.2.2.2.3	Interruptions during measurements on deactivated SCC

Interruptions on PCell or activated SCell(s) due to measurements when an SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer.

-	If the PCell or activated SCell(s) is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in Table 8.2.2.2.2-1.

-	If the PCell or activated SCell(s) is non-contiguous to the deactivated SCell in the same FR1 band and UE is capable of [intraBandNRCA-NonCollocated-r18] and [New BS signalling] is indicated [2] on this FR1 band, the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in Table 8.2.2.2.2-1.

-	If the PCell or activated SCell(s) is contiguous to the deactivated SCell in the same FR1 band, or if the PCell or activated SCell(s) is in the same FR1 band as the deactivated SCell and UE is not capable of [intraBandNRCA-NonCollocated-r18] or [New BS signalling] is not indicated [2], the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by Table 8.2.2.2.3-1. The interruption shall not exceed requirements in Table 8.2.2.2.3-1.

-	If the PCell or activated SCell(s) is in the same FR2 band as the deactivated SCell, the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by Table 8.2.2.2.3-1. The interruption shall not exceed requirements in Table 8.2.2.2.3-1.

The interruption requirements in Table 8.2.2.2.3-1 are not applicable when a UE is configured with NCSG in the same frequency range as the SCell unless the SMTC on the deactivated SCC is fully non-overlapped with NCSG.

Table 8.2.2.2.3-1: Interruption duration for measurement on deactivated SCell for intra-band CA

|  | NR Slot length (ms) | X (slots) | Interruption length (slots) |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 2 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 4 | 8 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 16 | 32 + TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 32 | 64 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is the longest SMTC duration among all above active serving cells and the deactivated SCell to be measured; NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |  |

# < End of change 4>