3GPP TSG-RAN WG4 Meeting #109	R4-2319128

# Chicago, US,  13th – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | <CR#> | rev | <Rev#> | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on interruption requirements for UE reporting NFG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New interruption requirements apply when a NeedforInterruptionInfoNR capable UE measures intra-frequency and inter-frequency MO-s without measurement gap but with interruptions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the interruption length/ratio allowed for a UE measuring intra-frequency and inter-frequency MO-s without measurement gap but with interruptions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirement is applied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 8.2.2.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <<Start of Change1>>

##### 8.2.2.2.X	Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18]

When a UE supports [NeedForInterruptionInfoNR-R18] measurements and indicates [no-gap-with-interruption] on intra-frequency SSB-based or inter-frequency SSB-based measurements, the UE is allowed to cause interruptions while performing measurements on the frequency layers of the bands for which [no-gap-with-interruption] is indicated. Requirements in this section apply only when the UE is in SA operation mode.

The UE is allowed to cause interruption with interruption ratio no more than the requirements specified below upon UE measurements on a specific frequency layer that corresponds to the configured MO, where Tcycle,i is the interruption cycle on a certain frequency layer i according to the network configuration, specified in Table 8.2.2.2.X-1 and 8.2.2.2.X-2.

Table 8.2.2.2.X-1: Tcycle,i length for intra-frequency measurement target carrier i

| DRX cycle | TCycle,i |
| --- | --- |
| No DRX | max (80ms, SMTC period) x CSSFintra |
| DRX cycle ≤ 320ms | max(80ms, SMTC period, DRX cycle) x CSSFintra |
| DRX cycle>320ms | DRX cycle x CSSFintra |

Table 8.2.2.2.X-2: Tcycle,i length for inter-frequency measurement target carrier i

| DRX cycle | TCycle,i |
| --- | --- |
| No DRX | max (80ms, SMTC period) x CSSFinter |
| DRX cycle ≤ 320ms | max(80ms, SMTC period, DRX cycle) x CSSFinter |
| DRX cycle>320ms | DRX cycle x CSSFinter |

UE is allowed to cause interruption on a certain frequency layer i:

-	up to [2.50%] probability of missed ACK/NACK when 80ms ≤ Tcycle,i < 160ms, or

-	up to [1.25%] probability of missed ACK/NACK when 160ms ≤ Tcycle,i < 320ms, or

-	up to [0.625%] probability of missed ACK/NACK when 320ms ≤ Tcycle,i.

The total allowed maximum interruption ratio on each of the active serving cells due to UE measurements without gap applied in this sub-clause is specified as

D= i=1N2LTcycle,i

Where,

N is the total number of configured SSB based measurement target frequency layers, including intra-frequency and inter-frequency target carriers, and

L is the maximum interruption length for each interruption occasion specified in the Table 8.2.2.2.X-3 and 8.2.2.2.X-4.

If measurement gap is configured and the gap occasions are partially or fully overlapped with the SMTC occasions on a certain frequency layer that UE indicates [no-gap-with-interruption], no interruption from measurements on such layer is allowed.

The interruptions are allowed for all the active serving cells in the same FR as NR MO being measured if UE supports per-FR measurement gaps, and all the serving cells if UE does not support per-FR measurement gaps.

Table 8.2.2.2.X-3: Interruption length L in FR1

|  | SCS (kHz) | NR Slot | Interruption length L (slots) |
| --- | --- | --- | --- |
|  |  | length (ms) |  |
| 0 | 15 | 1 | [1] |
| 1 | 30 | 0.5 | [2] |
| 2 | 60 | 0.25 | [4] |

Table 8.2.2.2.X-4: Interruption length L in FR2

|  | SCS (kHz) | NR Slot | Interruption length L (slots) |
| --- | --- | --- | --- |
|  |  | length (ms) |  |
| 2 | 60 | 0.25 | [3] |
| 3 | 120 | 0.125 | [6] |

### <<End of Change1>>