# 3GPP TSG-RAN WG4 Meeting # 109	R4-2319138

# Chicago, US, November 13 – 17, 2023

Agenda Item:	8.25.2.3

Source: 	Ericsson

Title:	Discussions on NW-A’s requirement in MUSIM gaps

Document for:	Discussion

# Introduction

In last meeting, RAN4 had a discussion on NW-A’s requirements for MUSIM gap as follow [1] and the related big CR has endorsed. In this contribution, we will discuss the remaining issues on NW-A’s requirements for MUSIM gaps.

# NW-A’s requirement

One of the remaining issues in NW-A’s requirement is when UE performs a measurement without gap which is partially overlapping with the gap but fully overlapping with the union of the NW-A’s gap and MUSIM gaps. For example, we show a simple example in the figure below. In this case, UE had to perform the measurement within the MG1. The cell identification requirement will be max(600ms, 5* MGRP)* CSSFwithin_gap,i.

Figure 1. SMTCs are fully overlapping with the union of the gaps

Proposal 1: When UE performs a measurement without gap which is partially overlapping with the gap but fully overlapping with the union of the NW-A’s gap and MUSIM gaps, UE shall perform the measurement within MG. The requirement will be

Table 1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, 5 x Kgap x max(MGRP, SMTC period)) x CSSFwithin_gap,i |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 x 5 x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFwithin_gap,i |
| DRX cycle>320ms | ceil(5 x Kgap) x DRX cycle x CSSFwithin_gap,i |

- Summary

In this contribution, we discussed the MUSIM gaps requirements. Based on the discussions, we have made following proposals and observations:

Proposal 1: When UE performs a measurement without gap which is partially overlapping with the gap but fully overlapping with the union of the NW-A’s gap and MUSIM gaps, UE shall perform the measurement within MG. The requirement will be

Table 1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, 5 x Kgap x max(MGRP, SMTC period)) x CSSFwithin_gap,i |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 x 5 x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFwithin_gap,i |
| DRX cycle>320ms | ceil(5 x Kgap) x DRX cycle x CSSFwithin_gap,i |

- References

- R4-2317425, “WF on RRM requirements for Rel-17 MUSIM gaps”, Vivo