# 3GPP TSG-RAN WG4 Meeting # 109                              R4-2319548

# Chicago, US, November 13 – 17, 2023

# Title: 	Simulation results for ATG UE demodulation

# Source: 	ZTE Corporation

# Agenda item:	8.13.8.2

# Document for:	Discussion

# 1 Introduction

In RAN#99e meeting[1], the revised work item on Air-to-ground network for NR was approved.

This contribution shares our updated simulation results with impairments results appended.

- Simulation results

In last meeting, we provided the simulation results with the AWGN channel of [1, 1; 1, 1]. Based on the simulation assumption listed in Annex A, we provide the simulation results with impairment results appended in table 2-1 for reference.

| Index | SCS | TDD pattern | Bandwidth | MCS | Antenna configuration | Propagation condition | SNR@70% max TP (ideal) | SNR@70% max TP (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 15kHz | - | 10MHz | MCS13 Table1 | 2x2 | AWGN + 220Hz Doppler | 3.40 | 5.90 |
| 1-2 | 15kHz | - | 10MHz | MCS22 Table1 | 2x2 | AWGN + 220Hz Doppler | 10.40 | 12.90 |
| 1-4 | 15kHz | - | 10MHz | MCS24 Table2 | 2x2 | AWGN + 220Hz Doppler | 19.40 | 21.90 |
| 1-6 | 30kHz | 7D1S2U | 40MHz | MCS13 Table1 | 2x2 | AWGN + 500Hz Doppler | 3.40 | 5.90 |
| 1-7 | 30kHz | 7D1S2U | 40MHz | MCS22 Table1 | 2x2 | AWGN + 500Hz Doppler | 10.40 | 12.90 |
| 1-9 | 30kHz | 7D1S2U | 40MHz | MCS24 Table2 | 2x2 | AWGN + 500Hz Doppler | 19.41 | 21.91 |
| 1-11 | 30kHz | 30D4S6U | 40MHz | MCS13 Table1 | 2x2 | AWGN + 500Hz Doppler | 3.41 | 5.91 |
| 1-12 | 30kHz | 30D4S6U | 40MHz | MCS22 Table1 | 2x2 | AWGN + 500Hz Doppler | 10.41 | 12.91 |
| 1-14 | 30kHz | 30D4S6U | 40MHz | MCS24 Table2 | 2x2 | AWGN + 500Hz Doppler | 19.41 | 21.91 |
| 2-1 | 15kHz | - | 10MHz | MCS13 Table1 | 2x4 | AWGN + 220Hz Doppler | 0.40 | 2.90 |
| 2-2 | 15kHz | - | 10MHz | MCS22 Table1 | 2x4 | AWGN + 220Hz Doppler | 7.41 | 9.91 |
| 2-4 | 15kHz | - | 10MHz | MCS24 Table2 | 2x4 | AWGN + 220Hz Doppler | 16.40 | 18.90 |
| 2-6 | 30kHz | 7D1S2U | 40MHz | MCS13 Table1 | 2x4 | AWGN + 500Hz Doppler | 0.40 | 2.90 |
| 2-7 | 30kHz | 7D1S2U | 40MHz | MCS22 Table1 | 2x4 | AWGN + 500Hz Doppler | 7.41 | 9.91 |
| 2-9 | 30kHz | 7D1S2U | 40MHz | MCS24 Table2 | 2x4 | AWGN + 500Hz Doppler | 16.41 | 18.91 |
| 2-11 | 30kHz | 30D4S6U | 40MHz | MCS13 Table1 | 2x4 | AWGN + 500Hz Doppler | 0.41 | 2.91 |
| 2-12 | 30kHz | 30D4S6U | 40MHz | MCS22 Table1 | 2x4 | AWGN + 500Hz Doppler | 7.41 | 9.91 |
| 2-14 | 30kHz | 30D4S6U | 40MHz | MCS24 Table2 | 2x4 | AWGN + 500Hz Doppler | 16.41 | 18.91 |

# 3 Conclusion

In this contribution, we give some simulation results for ATG UE DL demodulation .

# 4	Reference

[1]  RP-230279, Revised WID on Air-to-ground network for NR, CMCC

[2]  3GPP TS 38.101-4 V18.1.0

# Annex A

Table A-1 Simulation assumption

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 10 | 40 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U |
|  |  |  |  | 30D4S6U S:40G |
| Propagation channel |  |  | Static channel defined in B.1.1 [2]+220Hz doppler | Static channel defined in B.1.1 [2]+500Hz doppler |
| Antenna configuration |  |  | 2x2 | 2x2 |
|  |  |  | 2x4 | 2x4 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| MCS/Rank |  |  | Rank 1 16QAM Table 1 MCS13 64QAM Table 1 MCS 22 256QAM Table 2 MCS 24 | Rank 1 16QAM Table 1 MCS13 64QAM Table 1 MCS 22 256QAM Table 2 MCS 24 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 |
|  | Length (L) |  | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 |
|  | PRB bundling type |  | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 3 | 3 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 0 | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  | TCI state #1 | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 5 | 5 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 4 | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2' |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Number of HARQ Processes |  |  | 4 | 8 for 7DS2U 32 for 30D4S6U |
| Test metric |  |  | 70% of max throughput at target SNR. |  |

# Annex B

Figure B-1 Normalized throughput of MCS 13(table 1) for FDD 15kHz SCS 10MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-2 Normalized throughput of MCS 22(table 1) for FDD 15kHz SCS 10MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-3 Normalized throughput of MCS 20(table 2) for FDD 15kHz SCS 10MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-4 Normalized throughput of MCS 24(table 2) for FDD 15kHz SCS 10MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-5 Normalized throughput of MCS 27(table 2) for FDD 15kHz SCS 10MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-6 Normalized throughput of MCS 13(table 1) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-7 Normalized throughput of MCS 22(table 1) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-8 Normalized throughput of MCS 20(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-9 Normalized throughput of MCS 24(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-10 Normalized throughput of MCS 27(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-11 Normalized throughput of MCS 13(table 1) for TDD(30D4S6U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-12 Normalized throughput of MCS 22(table 1) for TDD(30D4S6U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-13 Normalized throughput of MCS 20(table 2) for TDD(30D4S6U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-14 Normalized throughput of MCS 27(table 2) for TDD(30D4S6U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-15 Normalized throughput of MCS 27(table 2) for TDD(30D4S6U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

2X4

Figure B-16 Normalized throughput of MCS 13(table 1) for FDD 15kHz SCS 10MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-17 Normalized throughput of MCS 22(table 1) for FDD 15kHz SCS 10MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-18 Normalized throughput of MCS 20(table 2) for FDD 15kHz SCS 10MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-19 Normalized throughput of MCS 24(table 2) for FDD 15kHz SCS 10MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-20 Normalized throughput of MCS 27(table 2) for FDD 15kHz SCS 10MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-21 Normalized throughput of MCS 13(table 1) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-22 Normalized throughput of MCS 22(table 1) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-23 Normalized throughput of MCS 20(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-24 Normalized throughput of MCS 24(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-25 Normalized throughput of MCS 27(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-26 Normalized throughput of MCS 13(table 1) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-27 Normalized throughput of MCS 22(table 1) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-28 Normalized throughput of MCS 20(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-29 Normalized throughput of MCS 24(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-30 Normalized throughput of MCS 27(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift