3GPP TSG-RAN WG4 Meeting #109             	R4-2319842

Chicago, USA, 13th Nov-17th Nov, 2023

Agenda item:	8.6.4.1

Source:	Samsung

Title:	Discussion and simulation results on BS demodulation requirements for FR2 256QAM

Document for:	Discussion

- Introduction

In the last meeting, the test scope and test setup of BS demodulation requirement for UL 256QAM is provided. The related agreements were captured in the WF [1] as following

- Tx EVM - Impact for TxEVM shall be included in impairment results - SNR limit - If impairment result for MCS 20 is over 20dB then check with TE vendors if this is testable. Request companies to include results with impairment for next meeting - Tx Phase Noise model - Do not specify Tx phase noise model or impact for requirement derivation - Rx Phase Noise model - Do not specify Rx phase noise model but consider Rx phase noise impact for impaired results - Channel Model - TDL D 30-45 - Note: Review channel if impaired results create testability issues - Antenna Configuration - 1T2R - Carrier BW and SCS - 60KHz SCS and corresponding carrier BW - Companies to use 50MHz for 60KHz, however, decision whether to include requirements for SCS of 60KHz to be defined to RAN4#109 - 120KHz SCS and corresponding carrier BW - Companies to use 50 MHz initially for requirements derivation, with FFS on 100 MHz and 200 MHz. - Companies to provide simulations for 50, 100, 200 with a priority in order of 50, 200, 100, with a view on testable SNR. - Note: Could use existing applicability rule to extend 50 MHz to other CBW - DMRS - Both 1+1 and 1+0 cases selected for tests to be based on BS Manufacturer Declaration and applicability rule to test requirements where a testable case is available. - For information below is a table of expected testable cases (with impairment, and testable being less than 20dB SNR) - MCS - MCS 20

In this contribution, the initial simulation results on BS demodulation requirement with 256QAM is provided for alignment purpose.

# 2	Discussion

Regarding the SCS and BW, both 60KHz and 120KHz SCS are under discussion and further evaluation. In Rel-15, PUSCH requirement with both 60KHz and 120KHz were introduced.

For FR2, 120KHz SCS should be the typical deployment based on commercial product in the field. Meanwhile, with larger SCS, the robustness to overcome the impact of phase noise is better. Therefore, we prefer to only define PUSCH requirement with FR2 256QAM for 120KHz SCS.

Proposal 1: Only define PUSCH requirement with FR2 256QAM for 120KHz SCS

Regarding the BW, since RAN4 has defined the test applicability rule with different CBWs, 50MHz CBW should fulfill the test purpose of FR2 256QAM.  We are also open to further discuss whether introducing PUSCH requirement with 100MHz and 200MHz.  Pending on the evaluation results, if there is a large performance difference with testable SNR, 100MHz and 200MHz can be introduced

Proposal 2: Define PUSCH requirement with 50MHz CBW. The requirement with 100MHz and 200MHz can be defined, if there is a large performance difference among difference CBW with testable SNR

# 3	Simulation Results

In this section, the initial simulation results are provided based on agreed simulation assumption as following table

Table 1: Simulation Assumption

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | pos0, pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2, Disabled |
| configuration | Time density (LPT-RS) | 1, Disabled |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |

Table 2: Ideal and impairment simulation results for PUSCH with 256QAM (To be updated)

| Case | MCS | Channel Model | SCS&BW | DMRS | PTRS | Number of symbols | Mapping type | SNR@70TP ideal | SNR@70TP Impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 20 | TDLD30-45 | 60KHz SCS 50MHz | 1+0 | Enable (K_PTRS=2, and L_PTRS=1) | 10 | B |  |  |
| Case 2 | 20 | TDLD30-45 | 60KHz SCS 50MHz | 1+1 (0,8) | Enable (K_PTRS=2, and L_PTRS=1) | 10 | B |  |  |
| Case 3 | 20 | TDLD30-45 | 60KHz SCS 50MHz | 1+1 (0,8) | Disable | 10 | B |  |  |
| Case 4 | 20 | TDLD30-45 | 120KHz SCS 50MHz | 1+0 | Enable (K_PTRS=2, and L_PTRS=1) | 10 | B |  |  |
| Case 5 | 20 | TDLD30-45 | 120KHz SCS 50MHz | 1+1 (0,8) | Enable (K_PTRS=2, and L_PTRS=1) | 10 | B |  |  |
| Case 6 | 20 | TDLD30-45 | 120KHz SCS 50MHz | 1+1 (0,8) | Disable | 10 | B |  |  |
| Case 7 | 20 | TDLD30-45 | 120KHz SCS 100MHz | 1+0 | Enable (K_PTRS=2, and L_PTRS=1) | 10 | B |  |  |
| Case 8 | 20 | TDLD30-45 | 120KHz SCS 100MHz | 1+1 (0,8) | Enable (K_PTRS=2, and L_PTRS=1) | 10 | B |  |  |
| Case 9 | 20 | TDLD30-45 | 120KHz SCS 100MHz | 1+1 (0,8) | Disable | 10 | B |  |  |
| Case 10 | 20 | TDLD30-45 | 120KHz SCS 200MHz | 1+0 | Enable (K_PTRS=2, and L_PTRS=1) | 10 | B |  |  |
| Case 11 | 20 | TDLD30-45 | 120KHz SCS 200MHz | 1+1 (0,8) | Enable (K_PTRS=2, and L_PTRS=1) | 10 | B |  |  |
| Case 12 | 20 | TDLD30-45 | 120KHz SCS 200MHz | 1+1 (0,8) | Disable | 10 | B |  |  |

# 4	Conclusion

In this contribution, the initial simulation results for BS demodulation requirement with UL 256QAM was provided for alignment purpose.

Proposal 1: Only define PUSCH requirement with FR2 256QAM for 120KHz SCS

Proposal 2: Define PUSCH requirement with 50MHz CBW. The requirement with 100MHz and 200MHz can be defined, if there is a large performance difference among difference CBW with testable SNR

# References

R4-2316903, WF on BS demodulation requirement for FR2 256QAM