3GPP TSG-RAN WG4 Meeting #109              	R4- 2319846

Chicago, USA, 13th Nov-17th Nov, 2023

Agenda item:	8.26.8.1

Source:	Samsung

Title:	View on BS demodulation requirements for NTN enhancement

Document for:	Discussion

- Introduction

In RAN#99 meeting, the WI on NR NTN (Non-Terrestrial Networks) enhancements was approved. In the last RAN4 meeting, the test scope of BS demodulation requirements for NTN enhancement was discussed. The related agreement was captured in the WF as following [1]

- General - Scenario and channel model for above 10GHz bands - Scenario - At least NGSO scenario to be considered for requirement definition, companies can check whether GSO can also be considered for NTN demod. - Focus on the mobility scenario assumed by RRM. (Mobility VSAT with LEO is not considered) - Channel Model - For FR2-NTN, consider NTN-TDLA and/or NTN-TDL-C with down selection if necessary. - FFS delay and spread - For delay selection, consider the worst case based on typical angle selection, e.g., [30] - How to derive the doppler - Option 1: Based on residual frequency error - Option 2: Based on UE speed - FFS UE speed: [120km/h, 1000km/h], other Options are not precluded - Interested companies are encourage to propose values for doppler and delay spread - Other options are not precluded. - SAN demodulation requirement - Which feature can be considered for SAN PUSCH demodulation performance requirements for above 10 GHz bands - Define PUSCH with DFT-S-OFDM and CP-OFDM demodulation performance requirements for above 10 GHz bands. - FFS PUSCH repetition Type A, companies can do some analysis on the link budget. - Do not consider UL timing adjustment - Which format can be considered for SAN PUCCH demodulation performance requirements for above 10 GHz bands - Define PUCCH format 0/1/2/3/4 demodulation performance requirements for above 10 GHz bands - Which format can be considered for SAN PRACH demodulation performance requirements for above 10 GHz bands? - Define PRACH burst format B4/C2 demodulation performance requirements for above 10 GHz bands - Do not consider PRACH burst format A2. - Whether to define SAN PUSCH demodulation performance requirements for DMRS bundling - Define SAN PUSCH demodulation performance requirements for DMRS bundling for FR1 only if RF confirm that phase continuous is feasible - Whether to define SAN PUCCH demodulation performance requirements for Msg4 HARQ-ACK - Do not define PUCCH requirements Msg4 HARQ-ACK for NLOS channel. - WF - Option 1: Add new requirements under LOS channel for multi-slot PUCCH format 1 - Option 2: Do not define PUCCH requirements Msg4 HARQ-ACK for LOS channel

In this contribution, the view on the remaining issue of test scope and test setup of BS demodulation requirement for NTN enhancement was provided.

# 2	General

Scenario

Regarding the scenario for requirement definition, as agreed at least NGSO scenario to be considered.

For Rel-18 NTN enhancement, Both GSO and NGSO are included into the WID for above 10GHz, similar as Rel-17 NTN WI. Since assuming NTN UE can do the pre-compensation, from demodulation requirement perspective, we think the SAN requirement can be applied for both GSO and NGSO. Therefore, similar as Rel-17 NTN demodulation, only focuses on the LEO scenario, the SAN requirement can be applied for both GSO and NGSO

Proposal 1: Only focus on the NGSO for performance definition, the SAN requirement can be applied for both GSO and NGSO.

Channel Model

In Rel-17 NTN WI, NTN-TDLA100 and NTN-TDLC5 were selected for demodulation requirement in S band to cover both NLOS and LOS scenario.

Regarding how to select the proper delay spread and doppler shift, RAN4 has discussion with several meeting in Rel-17 NTN WI. As for delay spread, small value is expected with considering high carrier frequency.  Based on agreement in the last meeting, the selection of delay spread is considering the elevation angle >=30 as the worst case from link budget perspective, also the timing limitation. Therefore, the same approach can be reused to select the proper delay spread. The following is the RMS delay spread for different scenarios NLOS and LOS in Ka band

The RMS delay spread is within range of 0.5~114.8ns regarding to elevation angle >= 30o.

Table 1 RMS delay spread for different scenarios NLOS in Ka band

| DS (ns) | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dense Urban NLOS | 138.0384 | 144.5440 | 100.0000 | 61.6595 | 38.0189 | 13.8038 | 17.3780 | 8.5114 | 11.2202 |
| Urban NLOS | 57.5440 | 19.9526 | 15.1356 | 9.1201 | 8.3176 | 7.9433 | 6.9183 | 9.3325 | 4.6774 |
| Suburban NLOS | 37.1535 | 23.4423 | 13.8038 | 10.9648 | 10.4713 | 3.5481 | 6.1660 | 2.0417 | 2.0417 |
| Rural NLOS | 0.7413 | 4.0738 | 7.9433 | 10.9648 | 10.2329 | 8.9125 | 9.7724 | 8.9125 | 12.3027 |

For LOS channel NTN-TDLC, the relevant parameters are K-factor and delay spread

Table 2 RMS delay spread for different scenarios LOS in Ka band

| DS (ns) | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dense Urban LOS | 37.1535 | 23.9883 | 17.3780 | 9.5499 | 7.4131 | 5.0119 | 4.5709 | 4.0738 | 3.5481 |
| Urban LOS | 3.0200 | 2.5704 | 3.0903 | 3.2359 | 3.3113 | 3.6308 | 3.9811 | 4.2658 | 4.4668 |
| Suburban LOS | 8.5114 | 2.4547 | 1.9055 | 2.3442 | 2.8840 | 3.3113 | 3.8019 | 4.0738 | 4.2658 |
| Rural LOS | 0.2089 | 1.3804 | 2.5704 | 3.4674 | 4.3652 | 5.0119 | 5.4954 | 6.0256 | 6.1660 |

As for delay spread, small value is preferred considering high frequency.  Based on the RMS delay spread, for NLOS scenario, the DS is around 10ns for most of cases with elevation angle >=30, and for LOS scenario, the DS is around with 5ns. Considering that, we would like to apply TDLA10 and TDLC5 for specifying PUSCH requirement above 10GHz, if TE vendor can confirm the delay spread with 10ns and 5ns is feasible to implement.

As for Maximum Doppler spread, up to 1000km/h can be supported based on VSIT UE. How to derive the doppler, based on the residual frequency error, another is based on UE speed.

With basing residual frequency error, assuming UE will do the doppler frequency tracking and pre-compensation, then, only the UE speed will be included for doppler spread calculation. Based on frequency range for above 10GHz in Ka-band as following, up to 30GHz is supported in UL.

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN1 | 410 MHz – 7125 MHz |
| FR2-NTN2 | 17300 MHz – 30000 MHz |
| NOTE 1: [NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.] NOTE 2: [NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.] |  |

Assuming the 0.1 ppm for residual Frequency error after UL pre-compensation, the related frequency offset is around 3000Hz.  With basing on UE speed with 120km/h, the maximum doppler should be 3333Hz.  The doppler value should be very high. If taking the UE speed with 1000km/h, the maximum doppler will be extremally high. It is very challenge to handle such high doppler

The feasibility should be checked based on the simulation results.

Proposal 2: The following channel model can be considered for feasibility evaluation as starting point

- [NTN-TDLA10-3000]

- [NTN-TDLC5-3000]

Antenna configuration for requirement above 10GHz

For above 10GHz, only VSAT UE support, both co-phase array (dual linear polarization) and circular polarization. For co-phase array, it is possible to support 2Tx. For circular polarization, we are not sure how to support 2Tx transmission simultaneously. Given there is no 2Tx UE RF requirement, we prefer to prioritize to define SAN requirement with 1Tx1Rx and 1Tx2Rx antenna configuration

Proposal 3: Prioritize to define SAN requirement with 1Tx1Rx and 1Tx2Rx antenna configuration for above 10GHz.  FFS on 2Tx2Rx pending on conclusion of the 2Tx UE RF requirement

# 3	Test scope of BS demodulation requirement

Coverage Enhancement

Regarding the objective for coverage enhancement, there are two issues need to be investigated. One is PUCCH for msg4 HARQ-ACK with parameter set-1 for LEO-1200 operating at LOS, another is PUSCH with DMRS bundling.

Based on the last meeting, there is no conclusion to define the related requirement yet.

For PUCCH, the following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarization loss (per antenna port).

Based on RAN1 feasibility analysis as following

- For PUCCH for Msg4 HARQ-ACK with parameter set-1 for LEO-1200 operating at LOS, - One source observed that the existing specification can meet the performance requirement - Three sources observed that the existing specification cannot meet the performance requirement with a gap of 1.8 to 6 dB.

it was concluded that PUCCH for Msg4 HARQ-ACK should be enhanced to meet the coverage requirements for parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation 1: Existing specification cannot meet the performance requirement for PUCCH with Msg4 HARQ-ACK with parameter set-1 for LEO-1200 operating at LOS.

In Rel-17 NTN, only NLOS was introduced for PUCCH requirements including single-slot transmission and multi-slot transmission.

From PUCCH receiver processing aspect, there is no difference between NLOS and LOS channel condition. While from the test coverage aspect and the performance limitation with LOS condition based on RAN1 study, the PUSCH requirements for Msg4 HARQ-ACK under NTN deployment scenario with LOS channel could be introduced.

Proposal 4: PUCCH requirement with Msg4 HARQ-ACK with repetition could be introduced

In Rel-17, the PUSCH requirements with DMRS bundling for TN deployments have been specified. Although there is no new physical layer design, considering the test coverage and new deployment scenario of NTN channel, PUSCH requirement with DMRS bundling under the NTN channel condition could be introduced.

As for PUSCH with DMRS bundling, it was agreed to define SAN PUSCH demodulation performance requirements for DMRS bundling for FR1 only RF confirm that phase continuous is feasible.

Based on RF discussion,

No requirement to be defined in RAN4 for PUSCH DMRS bundling for NR NTN coverage enhancement in Rel-18. RAN4 to wait for the definition of more advanced channel model with variable Doppler and variable Delay

According to the most recent discussion on [108bis][332] LS_BSRF_RAN_task, UE testing conditions with constant fixed Doppler and constant fixed Delay (corresponding to the ephemeris/orbit information) are currently considered for both GSO and NGSO

Based on current channel model, if assuming that no frequency and timing drift as modelling, the exiting RF requirement for phase continuity can still be used. Therefore, PUSCH requirement with DMRS bundling can be introduced.

Proposal 5: PUSCH requirement with DMRS bundling can be introduced for NTN-specific UE if no frequency and timing drift modeling.

PUSCH test scope for above 10GHz

In the last meeting, it was agreed to define PUSCH with DFT-s-OFDM and CP-OFDM demodulation performance requirement. Regarding PUSCH with repetition Type A, the coverage can be improved. Similar as FR1, the PUSCH requirement with repetition Type A can be considered. We are also open to further discuss whether it is needed or not pending on the link budget analysis.

Proposal 6: PUSCH requirement with repetition Type A can be considered

# 4	Test setup of BS demodulation requirement

In this part, the detail test setup for specific demodulation requirements is discussed based on the test scope discussion in above.

## 4.1	Test setup of PUCCH requirement for Msg4 HARQ-ACK

PUCCH format

In Rel-15, PUCCH requirements with several formats were specified. For Msg4 HARQ-ACK is transmitted based on the structure of format 1. Therefore, only PUCCH format 1 requirement should be specified.

Proposal 7: Only PUCCH format 1 requirement should be specified

Number of Repetition

The number of transmissions supported for PUCCH transmission for Msg4 HARQ-ACK was agreed in RAN1

Agreement For PUCCH transmission for Msg4 HARQ-ACK, supported number of transmissions are 1, 2, 4, 8.

In Rel-17, multi-slot PUCCH with format 1 has been specified for NTN scenario with NLOS channel, where number of repetitions is 2. From the baseband processing, there is no different between LOS and NLOS channel. Considering the test coverage, and the coverage limitation of LEO-1200 satellite operating at LOS channel, additional test compared with Rel-17 multi-slots with format 1 under LOS channel can be introduced. For LOS channel, NTN-TDLC5-200 was introduced in Rel-17 for PUSCH requirement, then, it can be used for specifying the PUCCH format 1 with 2 repetition requirements

Proposal 8: Introduce multi-slot PUCCH requirement with format 1 under 2 repetition for NTN-TDLC5-200 channel

Antenna configuration

For antenna configuration, both 1Tx1Rx and 1Tx2Rx were considered for PUCCH requirement definition with repetition, where the related test applicability rule for different number of receiver antenna was introduced to reduce the test effort. Therefore, the same approach can be applied for performance test.

Proposal 9: Introduce PUCCH requirement with both 1Tx1Rx and 1Tx2Rx for NTN-specific UE with repetition. Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test

Other test parameters

For other test parameters, the existing one for multi-slot PUCCH with format 1 can be used as

Table 3: Test parameters for multi-slot PUCCH with format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

Proposal 10: Using the following test parameters for specifying PUCCH requirement for Msg4 HARQ-ACK

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

## 4.2	Test setup of PUSCH requirement with DMRS bundling

In Rel-17, DMRS bundling was introduced for coverage enhancement. To enable DMRS bunding feature, RAN4 has introduced the related phase difference, timing error and frequency error limitation requirement. For NTN-specific UE, the following work assumption was made in RAN1

Observation For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed. - FFS: whether/how to consider the initial timing error at the beginning - FFS: TA pre-compensation update is assumed - Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed. - FFS: impact of phase difference limit Observation For NTN-specific PUSCH DMRS bundling, - In LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots (for carrier bandwidth 5 MHz or larger), if the PRB allocation is not within 6 PRBs from the DC carrier, pre-compensation by UE and post-compensation by gNB are not assumed, and 70.5 (us/s) timing drift rate is assumed. - Note: this does not imply that UE shall be scheduled within 6 PRBs from the DC carrier. Working assumption For NTN-specific PUSCH DMRS bundling, to satisfy the phase difference limit without causing phase discontinuity, it is assumed that pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side. - UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit. - FFS: how to determine the actual TDW - FFS: specification impact - Send an LS to RAN4

Based on RAN1 observation, the timing error and frequency error can be stratified within at most 13 slots, and 32 slots, separately, if TA pre-compensation update is not assumed.

Regarding the phase difference limit, as observed by RAN1, it cannot be satisfied over multiple slots (for carrier bandwidth 5 MHz or larger), if the PRB allocation is not within 6 PRBs from the DC carrier.

Timing drifting model

In Rel-17, the NTN requirements was defined based on UE can do the pre-compensation in very UL transmission. Therefore, there is no timing drift modeling. For NTT-specific UE, based on RAN1 WA, UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit. If TA pre-compensation is applied for the first transmission in the TDW, and there is no TA pre-compensation operation for other UL transmission slot, it may have some impact on demodulation requirement.

Proposal 11: FFS on introduce the timing drift model for specifying the PUSCH requirement with DMRS bundling in NTN scenario.

Actual TDW and nominal TDW

Based on RAN1 agreement, the following is agreed for nominal TDW

For NTN-specific PUSCH DMRS bundling, reuse clause 6.1.7 in TS38.214 for nominal TDW determination, except for aspects related to UE capabilities and assistance information (if needed). - i.e., if PUSCH-TimeDomainWindowLength is configured, nominal TDW is determined by PUSCH-TimeDomainWindowLength; otherwise, nominal TDW is determined based on UE capability(ies) signaling. - FFS: which UE capability(ies) signaling is(are) used - FFS: whether/how to use UE assistance information, if supported

And the following is agreed for actual TDW

Agreement For NTN-specific PUSCH DMRS bundling, actual TDW is determined by the existing events and no additional event is defined

For Rel-17 CE with DMRS bundling, the value of nominal TDW is configured for 8 for FDD and 2 for TDD, and the values are same the configured PUSCH aggregation factor.  If the timing drifting model included, our preference is to consider the small value for nominal TDW for specifying PUSCH requirement with DMRS bundling.

Proposal 12: The following value for actual TDW and nominal TDW can be considered for PUSCH requirement with DMRS bundling in NTN scenario

- Actual TDW and PUSCH-TimeDomainWindowLength = 2 or 4 for FDD

- Actual TDW and PUSCH-TimeDomainWindowLength 2 for TDD

BW& SCS

In Rel-17, CE with DMRS bundling, the minimum CBW for each SCS was considered for requirement definition. As observed in RAN1, phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots (for carrier bandwidth 5 MHz or larger), if the PRB allocation is not within 6 PRBs from the DC carrier. Therefore, large CBWs or larger PRB may be not applied for NTN-specific UE with DMRS bundling, considering for satisfying the phase difference limit.

Proposal 13: The following PRB allocation can be considered for PUSCH requirement with DMRS bundling in NTN scenario

- PRB: 6RBs for both 15KHz and 30KHz

Mapping type

In Rel-17, both PUSCH mapping type A and mapping type B were considered for PUSCH requirement definition with DMRS bundling. To reduce the test effort, we prefer to specify the PUSCH DMRS bunding with mapping type A., while we are also open to further discuss whether the PUSCH mapping type B is necessary considering the test coverage.

Proposal 14: Introduce PUSCH requirement with mapping type A for NTN-specific UE with DMRS bundling

Waveform

Since the DMRS bundling introduced is mainly targeting for coverage enhancement, similar as Rel-17 CE, only CP-OFDM wave is considered for PUSCH requirement for NTN-specific UE with DMRS bundling.

Proposal 15: Introduce PUSCH requirement with CP-OFDM waveform for NTN-specific UE with DMRS bundling

Antenna configuration

For Antenna configuration, both 1Tx1Rx and 1Tx2Rx were considered for PUSCH requirement definition with DMRS bundling, where the related test applicability rule for different number of receiver antenna was introduced to reduce the test effort. Therefore, the same approach can be applied for performance test.

Proposal 16: Introduce PUSCH requirement with both 1Tx1Rx and 1Tx2Rx for NTN-specific UE with DMRS bundling. Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test

Number Of DMRS

For number of DMRS, although both 1 DMRS symbol and 2 DMRS symbols were considered for PUSCH requirement definition with DMRS bundling, and minor performance between 1 DMRS and 2 DMRS symbols, we would like to select the typical DMRS configuration for specifying the PUSCH requirement for NTN-specific UE with 1+1 DMRS symbols.

Proposal 17: Introduce PUSCH requirement for NTN-specific UE with 1+1 DMRS symbols.

Other

For other test parameters, the existing test parameters for specifying PUSCH requirement with DMRS bundling for TN-specific UE can be reused for NTN-specific UE with DMRS bundling.

Proposal 18: Apply the existing test parameters for specifying PUSCH requirement with DMRS bundling for NTN-specific UE with DMRS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Example UL-DL pattern [Note 1] |  | 15 kHz SCS: FDD and TDD 7D1S2U, S=6D:4G:4U 30 kHz SCS: FDD and TDD 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence [Note 2] | 0, 3, 0, 3 for FDD 0, 3, 0, 3 for TDD |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | [n2] or [n8] for FDD n2 for TDD |
| pusch-TimeDomainWindowLength |  | [2] or [4] slots for FDD 2 slots for TDD |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pusch-TimeDomainWindowLength and PUSCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUSCH is only scheduled for the actual TDW including 2 consecutive UL slots. Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

## 4.3	Test setup of BS demodulation requirement above 10GHz

In this subsection, the test setup for BS demodulation requirements above 10GHz, including PUSCH, PUCCH and PRACH are specified, separately.

SCS & BW

For numerologies in NR NTN above 10GHz bands, the following agreement are made in the RRM session

- RAN4 to consider the following numerologies for RRM requirement definition for NR-NTN above 10 GHz bands - Consider SSB SCS larger than 30KHz, i.e., 120KHz and 240KHz - Consider UL SCS larger than 30KHz, i.e., 60KHz and 120KHz - RAN4 to send an LS to RAN1 and RAN2

As agreed, RAN4 will start demodulation work with 120 kHz SCS for initial alignment, For the CBW, 50MHz can be taken as starting point for evaluation

Proposal 19: The following SCS and CBW can be considered for PUSCH performance evaluation above 10GHz

- 120 KHz SCS, 50MHz

## 4.3.1	Test setup of PUSCH requirement above 10GHz

For other test parameters, the existing test parameters for PUSCH performance evaluation in FR2 can be reused as following

Proposal 20: The following test parameters could be considered for PUSCH performance evaluation for above 10GHz for CP-OFDM waveform

Table 5:  Test parameters of PUSCH with CP-OFDM

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | [Pos2[ |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2, |
| configuration | Time density (LPT-RS) | 1, |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |

Proposal 21: The following test parameters could be considered for PUSCH performance evaluation for above 10GHz for DFS-s-OFDM waveform

Table 6:  Test parameters of PUSCH with DFT-s-OFDM

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Enabled |
| Default TDD UL-DL pattern (Note 1) |  | N/A |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | [Pos2] |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID0=0, group hopping and sequence hopping are disabled |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 10 |
| Frequency domain resource | RB assignment | FR2-1: 30 PRBs in the middle of the test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS |  | Not configured |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns. |  |  |

Proposal 22: The following test parameters could be considered for PUSCH performance evaluation for above 10GHz for PUSCH repetition type A if introduced

Table 7:  Test parameters of PUSCH with repetition type A

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  |  |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0,3 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | Disabled |
| configuration | Time density (LPT-RS) | Disabled |
| NOTE 1: The effective RV sequence is {0,2,3,1} with slot aggregation |  |  |

## 4.3.2	Test setup of PUCCH requirement above 10GHz

In Rel-17 NTN WI, the PUCCH requirements with all the formats were considered for NTN-specific UE. For above 10GHz, the same PUCCH formats can be considered for PUCCH requirement. The same test parameters can be reused for requirement definition.

Proposal 23: The following test parameters could be considered for specifying PUCCH requirement for above 10GHz

Table 8:  Test parameters of PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Test metric | DTX to ACK probability ACK missed detection probability |
| Channel Model | [NTN-TDLA10-3000] |

Table 9:  Test parameters of PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test metric | NACK to ACK probability ACK missed detection probability |

Table 10   Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 11:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 12:  Test parameters of PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Test metric | BLER |  |

Table 13:  Test parameters of PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | BLER |

## 4.3.3	Test setup of PRACH requirement above 10GHz

PRACH formats

In Rel-17 NTN WI, the PRACH requirements with long short formats 0 and 2, short formats B4 and C2, were considered for NTN-specific UE. For above 10GHz, as agreed, only define PRACH burst format B4/C2 demodulation performance requirement for above 10 GHz bands

Test preamble configuration

Regarding the test preamble configuration, the existing value in Rel-15 FR2 can be reused as following

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
|  | 60 | 69 | 0 | 0 |
| B4, C2 | 120 | 69 | 0 | 0 |

Frequency offset

Regarding the frequency offset, the value in both AWGN and fading are considered. For AWGN, the corresponding frequency offset should be 0. For fading channel, assuming UE will do the pre-compensation, the residual frequency offset is 0.1ppm. Given the carrier frequency is 30GHz in uplink, then, the frequency offset can be calculated as 3000Hz.

Time error tolerance

Similar as Rel-15, the time error tolerance can be calculated as: 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel. The detail value of Tdelay pending on the selected channel model

In summary, the following test parameters could be considered for specifying PRACH requirement above 10GHz

- PRACH format: C2 and B4

- Test preamble configuration

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
|  | 60 | 69 | 0 | 0 |
| B4, C2 | 120 | 69 | 0 | 0 |

- Frequency offset

  - AWGN: 0Hz,

  - Fading: 3000Hz

- Time error tolerance

  - 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel

- Channel Model: [NTN-TDLA10-3000]

Proposal 24: The following test parameters could be considered for specifying PRACH requirement for above 10GHz

- Test preamble configuration

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
|  | 60 | 69 | 0 | 0 |
| B4, C2 | 120 | 69 | 0 | 0 |

- Frequency offset

  - AWGN: 0Hz,

  - Fading: 3000Hz

- Time error tolerance

  - 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel

- Channel Model: [NTN-TDLA10-3000]

# 5	Conclusion

In this contribution, the view on the test scope of BS demodulation requirement for NTN enhancement is provided.

General

Proposal 1: Only focus on the NGSO for performance definition, the SAN requirement can be applied for both GSO and NGSO.

Proposal 2: The following channel model can be considered for feasibility evaluation as starting point

- [NTN-TDLA10-3000]

- [NTN-TDLC5-3000]

Proposal 3: Prioritize to define SAN requirement with 1Tx1Rx and 1Tx2Rx antenna configuration for above 10GHz.  FFS on 2Tx2Rx pending on conclusion of the 2Tx UE RF requirement

Observation 1: Existing specification cannot meet the performance requirement for PUCCH with Msg4 HARQ-ACK with parameter set-1 for LEO-1200 operating at LOS.

Test scope

Proposal 4: PUCCH requirement with Msg4 HARQ-ACK with repetition could be introduced

Proposal 5: PUSCH requirement with DMRS bundling can be introduced for NTN-specific UE if no frequency and timing drift modeling.

Proposal 6: PUSCH requirement with repetition Type A can be considered

Test setup

Proposal 7: Only PUCCH format 1 requirement should be specified

Proposal 8: Introduce multi-slot PUCCH requirement with format 1 under 2 repetition for NTN-TDLC5-200 channel

Proposal 9: Introduce PUCCH requirement with both 1Tx1Rx and 1Tx2Rx for NTN-specific UE with repetition. Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test

Proposal 10: Using the following test parameters for specifying PUCCH requirement for Msg4 HARQ-ACK

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

Proposal 11: FFS on introduce the timing drift model for specifying the PUSCH requirement with DMRS bundling in NTN scenario.

Proposal 12: The following value for actual TDW and nominal TDW can be considered for PUSCH requirement with DMRS bundling in NTN scenario

- Actual TDW and PUSCH-TimeDomainWindowLength = 2 or 4 for FDD

- Actual TDW and PUSCH-TimeDomainWindowLength 2 for TDD

Proposal 13: The following PRB allocation can be considered for PUSCH requirement with DMRS bundling in NTN scenario

- PRB: 6RBs for both 15KHz and 30KHz

Proposal 14: Introduce PUSCH requirement with mapping type A for NTN-specific UE with DMRS bundling

Proposal 15: Introduce PUSCH requirement with CP-OFDM waveform for NTN-specific UE with DMRS bundling

Proposal 16: Introduce PUSCH requirement with both 1Tx1Rx and 1Tx2Rx for NTN-specific UE with DMRS bundling. Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test

Proposal 17: Introduce PUSCH requirement for NTN-specific UE with 1+1 DMRS symbols.

Proposal 18: Apply the existing test parameters for specifying PUSCH requirement with DMRS bundling for NTN-specific UE with DMRS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Example UL-DL pattern [Note 1] |  | 15 kHz SCS: FDD and TDD 7D1S2U, S=6D:4G:4U 30 kHz SCS: FDD and TDD 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence [Note 2] | 0, 3, 0, 3 for FDD 0, 3, 0, 3 for TDD |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | [n2] or [n8] for FDD n2 for TDD |
| pusch-TimeDomainWindowLength |  | [2] or [4] slots for FDD 2 slots for TDD |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pusch-TimeDomainWindowLength and PUSCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUSCH is only scheduled for the actual TDW including 2 consecutive UL slots. Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

Proposal 19: The following SCS and CBW can be considered for PUSCH performance evaluation above 10GHz

- 120 KHz SCS, 50MHz

Proposal 20: The following test parameters could be considered for PUSCH performance evaluation for above 10GHz for CP-OFDM waveform

Table 5:  Test parameters of PUSCH with CP-OFDM

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | [Pos2[ |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2, |
| configuration | Time density (LPT-RS) | 1, |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |

Proposal 21: The following test parameters could be considered for PUSCH performance evaluation for above 10GHz for DFS-s-OFDM waveform

Table 6:  Test parameters of PUSCH with DFT-s-OFDM

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Enabled |
| Default TDD UL-DL pattern (Note 1) |  | N/A |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | [Pos2] |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID0=0, group hopping and sequence hopping are disabled |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 10 |
| Frequency domain resource | RB assignment | FR2-1: 30 PRBs in the middle of the test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS |  | Not configured |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns. |  |  |

Proposal 22: The following test parameters could be considered for PUSCH performance evaluation for above 10GHz for PUSCH repetition type A if introduced

Table 7:  Test parameters of PUSCH with repetition type A

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  |  |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0,3 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | Disabled |
| configuration | Time density (LPT-RS) | Disabled |
| NOTE 1: The effective RV sequence is {0,2,3,1} with slot aggregation |  |  |

Proposal 23: The following test parameters could be considered for specifying PUCCH requirement for above 10GHz

Table 8:  Test parameters of PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Test metric | DTX to ACK probability ACK missed detection probability |
| Channel Model | [NTN-TDLA10-3000] |

Table 9:  Test parameters of PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test metric | NACK to ACK probability ACK missed detection probability |

Table 10   Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 11:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 12:  Test parameters of PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Test metric | BLER |  |

Table 13:  Test parameters of PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | BLER |

Proposal 24: The following test parameters could be considered for specifying PRACH requirement for above 10GHz

- Test preamble configuration

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
|  | 60 | 69 | 0 | 0 |
| B4, C2 | 120 | 69 | 0 | 0 |

- Frequency offset

  - AWGN: 0Hz,

  - Fading: 3000Hz

- Time error tolerance

  - 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel

- Channel Model: [NTN-TDLA10-3000]

# References

R4-2316921, WF on NR_NTN_enh_SAN_UE_demod