3GPP TSG-RAN WG4 Meeting # 109						R4-2320037

Chicago, USA, November 13 – November 17, 2023

Agenda item:			7.1.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	A-MPR study for CA_n102B and CA_n102C uplink

Document for:	Information

Introduction

As discussed in [1] a study has been conducted to ensure sufficient A-MPR have been defined when introducing intra-band uplink CA configurations CA_n102B and CA_n102C to the specification. This contribution presents the results of this study.

Simulation Assumptions

- CA_n102B and CA_n102C

- PA calibration point: 20MHz 15kHz 100RB@3 DFT-S-OFDMA QPSK, MPR = 1 dB

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- Additional emission requirements according to NS_58:

  - Additional spurious emission limit from TS 38.101-1, subclause 6.5F.3.3.6

- Table 6.5F.3.3.6-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 87.5 ≤ f ≤ 118 | -54 | 100 kHz |
| 174 ≤ f ≤ 230 | -54 | 100 kHz |
| 470 ≤ f ≤ 694 | -54 | 100 kHz |
| f ≤ 5935 | -22 | 1 MHz |

- Waveforms: OFDM, DFT-S-OFDM

- Modulations: pi/2-BPSK, QPSK, 16-QAM, 64-QAM, 256-QAM

- Full and partial RB allocations

- Lower edge of aggregated channel  5945 MHz according to TS 38.101-1, subclause 6.5F.3.3.6

Simulation Results

The maximum power back-off required by the NS_58 additional spurious limit is 1 dB, which is well below the lowest NR-U CA MPR. Hence, no A-MPR is needed. This is caused by the high value of the additional spurious limit (-22 dBm in 1MHz measurement bandwidth) as well as the 10 MHz minimum distance between the spurious limit and the aggregated channel. Fig. 1 shows an example spectrum.

Figure 1: Spectrum of 80+40 MHz, 30 kHz, 1111-11, interlace 0, OFDM, QPSK, 
simulated using the specified MPR = 4 dB

No A-MPR is needed for CA_102B and CA_102C when NS_58 is signaled.

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: No A-MPR is needed for CA_102B and CA_102C when NS_58 is signaled.

- References

- R4-2320035, On addition of CA_n102B and CA_n102C uplink, Nokia, Nokia Shanghai Bell