3GPP TSG-RAN WG4 Meeting #108-bis	R4-2320171

Chicago, USA, 13th Nov. – 17th Nov. 2023

Agenda Item:	7.1.1.2

Source:	Charter Communications, Inc.

Title:	A-MPR tables for CA_NS_53 and CA_NS_54 when introducing NR-U CA_n96

Document for:	Approval

# Introduction

In RAN4#106 meeting, a WF for introducing Carrier aggregation to Power class 5 (PC5) in NR-unlicensed spectrum for was approved [1]. In RAN4#106-bis-e, a WF with the requirements for Carrier aggregation in NR-unlicensed spectrum for was approved [2].

“In Release 17 the CA_n96B/C UL CA and configurations are specified. Even so, it was clarified in RAN that although NR-U ULCA requirements were completed, NR-U ULCA could not be deployed since the A-MPR for n96B/C UL was not specified, especially for the U.S. NS_53 and NS_54.” [1]

Then in RAN4#108, two papers were submitted with simulation results [4] and [5].

In RAN4#108-bis, another paper was submitted with simulation results [6].

# Discussion

## Requirements

In its essence, same requirements shall be mapped for CA as for Single Carrier.

For in-band PSD, the maximum allowed PSD is defined as -1dBm/MHz for NS_53 and 17dBm/MHz for NS_54.

These additional requirements for the transmit power density are found in Table 6.2F.1-2 in TS 38.101-1 [3] for single carrier and the part for NS_53 and NS_54 of Table 6.2F.1-2 is shared below in Table 1.

Both network signalling flags have same out-of-band requirements as shown in Table 3. The maximum emission limits are defined with -27dBm/MHz at lower and upper side of the band. This clause is also found in TS 38.101-1 [3]. For NS_54, the additional out-of-band requirement is only affecting on the lower edge of the band, because NS_54 is only partially allowed in band, up to frequency range of 6875MHz, as seen in Table 1. Also, for both, the first 20MHz in the band on the lower edge is not used in Uplink. For NS_53, the upper edge is allowed to be up to the edge of the band and then the additional out-of-band requirements start where the band ends.

Table 1: In-band PSD requirements for NS_53 and NS_54.

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n96 | NS_53 | 20, 40, 60, 80 | 5925 – 7125 | -1 |
|  | NS_54 | 20, 40, 60, 80 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |

Table 2: CA_n96 in-band PSD requirements

| NR CA Band | NS value | Aggregated channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| CA_n96 | CA_NS_53 | 40, 60, 80, 100, 120, 140, 160 | 5925 – 7125 | -1 |
|  | CA_NS_54 | 40, 60, 80, 100, 120, 140, 160 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |

Table 3: Out-of-band requirements for NS_53 and NS_54.

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

In this paper, I have calibrated the conversion from time domain to spectrum domain, which has lowered the spectrum plot by approximated 0.25dB when looking at in-band PSD.

## Results

Here are my A-MPR results based on the latest simulations.

Figure 1: Backoff results CA_NS_53 with QPSK for PC5.

Figure 2: Backoff results CA_NS_54 with QPSK for PC5.

Before presenting our updated numbers, we would like to give an explanation to the numbers in the Table 3 and Table 4:

If the number is in-band PSD limited, then the number in the table has a backoff number in the table that is 0-0.49dB higher.

If the number is out-of-band limited, then the number in the table has a backoff number in the table that is 0.6-1.09dB higher.

| Pre-coding | Modulation | Bandwidth of contiguously transmitted sub-bands / RB Allocation / A-MPR (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100MHz |  | 120MHz |  | 140MHz |  | 160MHz |  |
|  |  | Full | Partial | Full | Partial | Full | Partial | Full | Partial | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| DFT-s-OFDM | PI/2 BPSK2 | ≤ 9.0 | ≤ 11.5 | ≤ 5.5 | ≤ 8.5 | ≤ 4.0 | ≤ 6.5 | ≤ 3.5 | ≤ 5.5 | ≤ 3.5 | ≤ 5.0 | ≤ 3.5 | ≤ 4.0 | ≤ 3.5 | ≤ 4.0 | ≤ 3.5 | ≤ 4.0 |
|  | QPSK | ≤ 9.0 | ≤ 11.5 | ≤ 5.5 | ≤ 8.5 | ≤ 4.0 | ≤ 6.5 | ≤ 3.5 | ≤ 5.5 | ≤ 3.5 | ≤ 5.0 | ≤ 3.5 | ≤ 4.0 | ≤ 3.5 | ≤ 4.0 | ≤ 3.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 5.5 | ≤ 8.5 | ≤ 4.0 | ≤ 6.5 | ≤ 3.5 | ≤ 5.5 | ≤ 3.5 | ≤ 5.0 | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 | ≤ 4.0 | ≤ 3.5 | ≤ 4.0 |
|  | 64 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 5.0 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 8.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 11.0 | ≤ 5.5 | ≤ 8.5 | ≤ 5.0 | ≤ 6.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.0 | ≤ 4.5 | ≤ 5.0 | ≤ 4.5 | ≤ 5.0 | ≤ 4.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 9.0 | ≤ 11.0 | ≤ 5.5 | ≤ 8.5 | ≤ 5.0 | ≤ 6.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 4.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 9.0 | ≤ 11.0 | ≤ 6.0 | ≤ 8.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 9.0 | ≤ 11.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4: A-MPR for NS_53 power class 5

Proposal 1: Consider Table 4 when specifying ULCA A-MPR for NS_53 as in Issue 2-3 in WF [8].

Table 5: A-MPR for NS_54 power class 5

| Pre-coding | Modulation | Bandwidth of contiguously transmitted sub-bands / RB Allocation / A-MPR (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RB Allocation (Note 4) |  | 40MHz – 80MHz (Note 5) |  | 100MHz – 120MHz (Note 5) |  | 140MHz – 160MHz (Note 5) |  |
|  |  | Full2 | Partial3 | Full2 | Partial3 | Full2 | Partial3 | Full2 | Partial3 |
| DFT-s-OFDM | PI/2 BPSK7 | ≤ 3.0 |  | ≤ 3.5 | ≤ 4.5 | ≤ 3.0 | ≤ 4.0 | ≤ 3.0 | ≤ 3.5 |
|  | QPSK | ≤ 3.0 |  | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 | ≤ 4.0 | ≤ 3.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 4.0 | ≤ 4.5 | ≤ 3.5 | ≤ 4.0 | ≤ 3.5 | ≤ 3.5 |
|  | 64 QAM | ≤ 4.5 |  | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 6.0 |  | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
| CP-OFDM | QPSK | ≤ 4.0 |  | ≤ 4.5 | ≤ 5.0 | ≤ 4.0 | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 |
|  | 16 QAM | ≤ 4.5 |  | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.0 |
|  | 64 QAM | ≤ 6.0 |  | ≤ 5.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.0 |  | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated. NOTE 2: The A-MPR for Full allocation applies to all RBs in all contiguously transmitted sub-bands for operation that are fully allocated. NOTE 3: The A-MPR for Partial RB allocation applies to all contiguously transmitted sub-bands with interlaced allocations with uplink resource allocation type 2 as specified in TS38.214 [10]. NOTE 4: Applicable for all valid channels and bandwidth of contiguously transmitted sub-bands other than those enumerated in NOTE 5. NOTE 5: Applicable to aggregated channel’s lower edge at 5945MHz. NOTE 6: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |

Proposal 2: Consider Table 5 when specifying ULCA A-MPR for NS_54.

Alternative proposal to Issue 2-5:

The NOTE 5 in Table 4, is a proposal to generically writing what was said in NOTE 5 and NOTE 6 in Issue 2-4: A-MPR table for NS_54 proposal 2 from the WF [8].

Proposal 3: Consider NOTE 5 in Table 5, NOTE 5: Applicable to aggregated channel’s lower edge at 5945MHz.

# Conclusion

In this paper, we at Charter are proposing the followings:

Proposal 1: Consider Table 4 when specifying ULCA A-MPR for NS_53.

Proposal 2: Consider Table 5 when specifying ULCA A-MPR for NS_54.

Proposal 3: Consider NOTE 5 in Table 5; NOTE 5: Applicable to aggregated channel’s lower edge at 5945MHz.

# Reference

[1] R4-2303484, WF on NR-U enhancement, Apple

[2] R4-2306481, WF on NR-U contiguous ULCA A-MPR, Charter Communications, Inc., Apple

[3] TS 38.101-1: User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V17.8.0 (2022-12)

[4] R4-2313823, Simulation results on UE RF NR-U UL CA A-MPR for PC5, Charter Communications Inc

[5] R4-2311250, On NR-U ULCA A-MPR for NS_53 and NS_54, Apple Inc

[6] R4-2317603 PC5 NR-U CA_n96B A-MPR for CA_NS_53 CA_NS_54, 3GPP TSG-RAN WG4 #108-bis, Xiamen, China, Skyworks Solutions, Inc.

[7] R4-2316808, Draft CR for introducing NR-U uplink CA for NS_53 and NS_54, 3GPP TSG-RAN WG4 #108-bis, Xiamen, China, Charter Communications, Inc.

[8] R4-2317655 WF on NS53 and NS54 A-MPR requirement, 3GPP TSG-RAN WG4 #108-bis, Xiamen, China, Charter Communications, Skyworks Solutions, Inc.