3GPP TSG-RAN WG4 Meeting # 109	R4-2320195

# Chicago, US, November 13-17, 2023

Title: 	Discussions on Rel-18 sidelink performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.36.2

Document for:	Discussion

Background

In last meeting, a WF [1] on UE demodulation performance for NR SL evolution was approved. This contribution provides our views on the remaining open issues.

- Discussions

CA scenario

The candidate options for requirements with CA scenario are listed as follows:

#### Issue 2-2-1: NR sidelink CA scenario Way Forward: - Option 1: Consider defining PSSCH requirements for NR sidelink CA with the same performance metric as in LTE sidelink CA. (Nokia) - Option 2: Define following tests for sidelink CA: (HW) - PSSCH performance requirements - HARQ buffer test - PSCCH decoding capability test - PSFCH decoding capability test - Option 3: Consider CA scenario as work scope for demodulation performance. (LGE)

Based on the latest UE feature list [2], the UE feature related to CA are listed as follows:

| 47. NR_SL_enh2 | 47-v1 | NR SL communication with SL CA | 1) UE supports transmitting/receiving PSCCH/PSSCH/PSFCH simultaneously over multiple X SL carriers: FFS candidate values for X FFS whether to report following - Maximum number of simultaneous PSCCH/PSSCH TX/RX - Maximum number of non-overlapping RBs per slot across all carriers the UE can attempt to decode 2) UE can adjust the transmission power of the PSCCH/PSSCH/PSFCH across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. FFS whether/how to merge FG for SL-CA | [15-3, 15-11] | Yes | No |  | [Per band] | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-v3 | PSFCH for SL CA | 1) UE supports receiving up to X PSFCH resources in a slot over multiple SL carriers 2) UE supports transmitting up to Y PSFCH resources in a slot over multiple SL carriers [3) UE can adjust the transmission number of PSFCH across aggregated carriers such that its total transmission number does not exceed the maximum transmission power.] FFS whether/how to merge FG for SL-CA | 47-v1, [15-11] | Yes | No |  | [Per band] | N/A | N/A |  | Candidate values for X are {FFS} Candidate values for Y are {FFS} | Optional with capability signalling |

Compared to operating on single CC, UE has to process PSSCH decoding for multiple CCs simultaneously, which means higher capability on buffer size, multiple demodulation paths including channel estimator and decoder are expected. Same as logic of requirements definition for normal UE, it’s very important to define CA requirements for the sidelink UE. Same as Rel-16 V2X test, CA test can be categorized as two types: Performance test and capability test.

Performance test

For performance test, RAN4 can follow the procedure of legacy CA test. Considering requirements with 20MHz bandwidth have been defined, the additional work is to define single CC requirements with 10MHz/30MHz/40MHz and apply these requirements to CA. We can reuse the test parameters Rel-16 V2X test cases as much as possible with limiting the test cases to one combination of MCS and propagation. We propose to use test parameters listed in Table 2-1 and 2-2 as stating point.

Table 2-1: Proposed common test parameters for CA

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
| Carrier configuration |  | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) |  | RBs | 0 |
|  |  | Subcarrier spacing |  | kHz | 30 |
| SL BWP configuration #1 |  | Cyclic prefix |  |  | Normal |
|  |  | RB offset |  | RBs | 0 |
|  |  | Number of contiguous PRB |  | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing |
| PT-RS configuration |  |  |  |  | PT-RS is not configured |
| 2nd stage SCI format 2-A configuraion |  |  | Payloads | Bits | 35 |
|  |  |  | α |  | 1 |
|  |  |  | βoffset |  | 5 |
| Resource pool configuration | PSCCH Time resource |  |  | Symbols | 2 |
|  | PSCCH Frequency resource |  |  | PRBs | 10 |
|  | PSFCH number of cyclic shift pairs |  |  |  | n1 |
|  | PSFCH hopping ID |  |  |  | 0 |
|  | PSFCH candidate resource type |  |  |  | allocSubCH |
|  | Set of PRBs for PSFCH transmission |  |  |  | ones(1,100) for 40 MHz ones(1,70) for 30 MHz ones(1,20) for 10 MHz |
|  | PSSCH RSRP threshold |  |  |  | 66 (infinity dBm) |
|  | Synchronization reference |  |  |  | GNSS |
|  | Subchannel size |  |  | PRBs | 10 |
|  | Number of sub-channels |  |  |  | 2 for 10MHz, 7 for 30MHz and 10 for 40 MHz |
|  | Start PRB for first sub-channel |  |  |  | 0 |
|  | Time resource bitmap |  |  |  | ones(1, 160) |
| Note 1: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. |  |  |  |  |  |

Table 2-2: Proposed test parameters for SL CA

| Test num. | Reference channel | Bandwidth (MHz)/ Subcarrier spacing(kHz) | Modulation format and code rate | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PSSCH BLER (%) | SNR(dB) of PSSCH |
| 1 | TBD | 20 / 30 | 16QAM, 0.37 | TDLA30-1400 | 10% | TBD |

Proposal 1: Use test parameters listed in Table 2-1 and Table 2-2 for SL CA performance test as starting part.

Meanwhile, RAN1 is discussing whether to introduce maximum number of non-overlapping RBs UE attempts to decode. This capability impacts the number of allocated RBs for each CC for CA test. So we propose to this open until the related capabilities discussions are finalized by RAN1

Proposal 2: Keep number of allocated RBs for each CC open until the RAN1’s discussions on capability of  “maximum number of non-overlapping RBs UE attempts to decode” is finalized.

Capability test

CA capability test, i.e, PSCCH/ PSFCH decoding capability test on CA operation is just a simple extension of Rel-16 single test without simulation work, therefore it could be introduced. Considering it is still on discussion, RAN4 should keep tracking on the RAN1’s progress and start the discussion once it is finalized by RAN1

Proposal 3: RAN4 to keep tracking on the RAN1 progress on following CA capability discussion and start the discussion once it is finalized by RAN1

Maximum number of receiving PSCCHs in a slot

Maximum number of receiving PSFCHs in a slot

Unlicensed band

The candidate options for test scope for unlicensed band are shown as follows:

Test scope

#### Issue 2-3-4: Requirements for PSSCH/PSCCH Way Forward: - Option 1: Consider defining requirements for PSSCH for interlaced RBs allocation in frequency selective channels in sidelink unlicensed. (Nokia, LGE) - Option 2: Introduce SL-U PSSCH/PSCCH requirements at least with following configurations: (HW) - Only interlaced RB based - 1 interlaced, 1 subchannel allocation - For the first slot of the COT, the start symbol of PSSCH/PSCCH is not #0 - Option 3: Do not introduce new requirement for PSSCH/PSCCH in SL-U. (Qualcomm) #### Issue 2-3-6: Requirements for PSFCH Way Forward: - Option 1: Discuss whether to define requirements for PSFCH by considering interlacing of RBs in PSFCH. (Nokia) - Option 2: Define SL-U PSFCH requirements considering the following: (HW, LGE) - Interlaced based - K3 PRBs is configured. The configuration of K3 can be FFS - ACK-NACK mode - Other parameters can be FFS - Option 3: Do not introduce new requirement for PSFCH in SL-U. (Qualcomm)

The main feature for this WI is interlaced RB allocation, which was a main motivation for NR-U PUSCH/ PUCCH requirements definition. We propose to introduce PSSCH, PSCCH and PSFCH requirements with interlaced RB allocation with following reasons.

For interlaced RB allocation, it’s more feasible to use per RB granularity for channel estimation due to the large gap between two used RBs, which means there are fewer DMRS samples available for MMSE filter interpolation, resulting in larger channel estimation error and poorer performance for PSSCH/PSCCH. Also, per RB channel estimation require different MMSE filter coefficient compared to legacy channel estimation. So, we see there are performance and baseband processing difference for interlace RB based PSSCH/PSCCH. That’s also the reason of introducing requirements for PUSCH/PUCCH with interlaced RB allocation in Rel-16 and Rel-14 LTE LAA.

For PSFCH with interlaced RB allocation, which is same as design of NR-U PF0, UE should perform energy detection K3 times where K3 is number of repetition in frequency domain and sum the energy for ACK/NACK detection. Furthermore interlaced RB allocation can bring energy and frequency selective gain. Therefore we see there are performance and baseband processing difference for interlace RB based PSFCH. That’s also the reason of introducing requirements for PF0 with interlaced RB allocation in Rel-16.

Proposal 4: RAN4 to introduce performance requirements for PSSCH, PSCCH and PSFCH with interlaced RB allocation.

Test scenario

Three unlicensed bands n46, n96 and n102 are defined for sidelink according to [1]. The details are captured as follow:

| NR SL-U operating band | Sidelink (SL) Transmission operating band | Sidelink (SL) Reception operating band | Duplex Mode | Interface |
| --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |  |
| n461 | 5150 MHz – 5925 MHz | 5150 MHz – 5925 MHz | HD | PC5 |
| n961,2 | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | HD | PC5 |
| n1021 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | HD | PC5 |
| NOTE 1: Direct connection between client devices and between vehicular devices in the shared spectrum bands or portions of the shared spectrum bands is subject to country-specific conditions and can be prohibited per region-specific regulatory rules, e.g., in USA and Canada. |  |  |  |  |

Same as Rel-16 V2X, mode 2(Standalone) scenario with GNSS synchronization source could be considered for requirements definition.

Carrier frequency offset (CFO) and Doppler spread depend on the center frequency of carrier, we propose to assume the center frequency to be 6.5GHz leading to 650Hz CFO with respect to GNSS (Frequency error requirement of SL-U is 0.1PPM as per [1]). 1300Hz CFO should be assumed in the simulation with the assumption  that both transmitting UE and receiving UE have max synchronization error.  As for the Doppler spread, three typical scenarios were considered for Rel-16 V2X: 2700Hz for 500km/h, 1500Hz for 260km/h and 180Hz for 30km/h, which can be reused with changing the Doppler spread to 2900Hz for 500km/h, 1500Hz for 260km/h, 195Hz for 30km/h based on the new center frequency. To reduce the test number, we propose to choose one channel condition. Also, the time spread can be reused.

For SCS, considering 30kHz is mandatory and 15kHz is optional, we propose to only consider 30kHz SCS.

For antenna configuration, 1T2R Low can be considered as baseline.

For channel bandwidth, 20MHz, 40MHz, 60MHz and 80MHz are defined for unlicensed band. Same as NR-U, 20MHz is defined as the unit of LBT bandwidth. RAN4 only considered 20MHz for NR-U performance requirements definition. We propose to also consider 20MHz bandwidth for SL-U performance requirements since different bandwidth have negligible impact on performance.

For channel estimation, same as Rel-16 V2X, MMSE based interpolation in frequency domain and linear interpolation in time domain can be considered as baseline for simulation assumption.

Time offset with respect to GNSS can be reused: CP/2-12*64*Tc.  The time offset 24*64*Tc should be assumed in the simulation with the assumption that both transmitting UE and receiving UE have max synchronization error.

To simplify the test, we propose to only consider 1 interlace (1 Sub-channel) for requirements definition. The RB allocation is 0, 5, 10, 15,…, 50

Proposal 5: Consider following test setup for SL-U test:

Carrier center frequency: 6.5GHz

- Operation mode: Mode2(Standalone)

- Synchronization source: GNSS based

- Carrier frequency offset with respect to GNSS: 650Hz

- Carrier frequency offset for simulation assumption: 1300Hz

- Time offset with respect to GNSS: CP/2-12*64*Tc

- Time offset for simulation assumption: 24*64*Tc

- SCS: 30kHz

- Antenna configuration: 1T2R Low

- Channel bandwidth: 20MHz

- Propagation conditions: Select one from {TDLA30-2900, TDLA30-1500, TDLA30-195}

- Channel estimation: MMSE based interpolation in frequency domain and linear interpolation in time domain

- Only consider 1 interlace (1 sub-channel) with RB index 0,5,10,15,…50

LBT model

LBT is mandatory in most regions, same as NR-U, LBT should be modelled during the test to comply the regulation. We have following proposals for LBT model.

Due to the utilization of HARQ-ACK feedback, LBT failure probability shall be set to 0 (pLBT=1) to guarantee that HARQ-ACK feedback is not impacted by the LBT failure.

It's typical to configure gap between two consecutive COTs to give TE more time to perform LBT, one potential way is to set the start position of PSSCH transmission in the first slot of each COT to #7

As agreed by RAN1, transmission from one UE with gap larger than 16us is considered as two DL transmission bursts. So CPE extension should be configured for the first AGC symbol of each SL slot within the COT to make the gap between two consecutive slots smaller than 16us.

The COT duration should be designed to guarantee that PSFCH is always transmitted in the slot with 14 symbol allocation.

35 SCI2 information bits are assumed during Rel-16 V2X test. However, to convey the COT information, SCI2 information bits is expected to be more which should be re-discussed.

Proposal 6: RAN4 to consider following principle for LBT model:

Due to the utilization of HARQ-ACK feedback, LBT failure probability shall be set to 1 (pLBT=0) to guarantee that HARQ-ACK feedback is not impacted by the LBT failure.

It's typical to configure gap between two consecutive COTs to give TE more time to perform LBT, one potential way is to set the start position of PSSCH transmission in the first slot of each COT to #7

CPE extension should be configured for the first AGC symbol of each SL slot within the COT to make the gap between two consecutive slots smaller than 16us

The COT duration should be designed to guarantee that PSFCH is always transmitted in the slot with 14 symbol allocation.

35 SCI2 information bits are assumed during Rel-16 V2X test. However, to convey the COT information, SCI2 information bits is expected to be more which should be re-discussed.

We give an example of LBT model as follows:

TE performs LBT to initial a COT with a LBT failure probability equaling to 0 (pLBT=1) and share this COT with tested UE

The start symbol of first slot in each COT is #7

The COT duration is randomly selected from {2,4,8} slots

COT information is conveyed in SCI stage 2.

CPE extension is configured for the first AGC symbol of each SL slot within the COT

Tested UE uses the sharing COT to transmit PSFCH by via type 2 channel access

- The illustration is Figure 2-1:

Figure 2-1: Proposed LBT model

Proposal 7: RAN4 to consider following LBT model as starting point:

TE performs LBT to initial a COT with  LBT failure probability equaling to 0 (pLBT=1) and share this COT with tested UE

The start symbol of first slot in each COT is #7

The COT duration is randomly selected from {2,4,8} slots

COT information is conveyed in SCI stage 2.

CPE extension is configured for the first AGC symbol of each SL slot within the COT

Tested UE uses the sharing COT to transmit PSFCH by via type 2 channel access

PSSCH

As designed by RAN1, multiple PSFCH occasions corresponding to one PSSCH is introduced to against LBT failure, as we have proposed to configure pLBT=1, it’s enough to configure 1 PSSCH occasion for each PSSCH.

We have 3 tests in Rel-16 PSSCH test, to reduce the work load, we propose to only choose one test for Rel-18 PSSCH requirements definition: MCS: 16QAM, 0.37; Propagation condition: TDLA30-1500.

Meanwhile, we propose to reuse the DMRS pattern, PSFCH resource period and MinTimeGapPSFCH of Rel-16 V2X test.

Proposal 8: RAN4 to consider following test configuration for PSSCH requirements definition:

Configure 1 PSSCH occasion for each PSSCH

MCS:16QAM, 0.37

Propagation: TDLA30-1500

PSFCH resource period: 4

MinTimeGap: 3

PSSCH DMRS pattern: 3 symbols for slot without PSFCH transmission and 2 symbols for slot with PSFCH transmission.

PSCCH

We can reuse the test configuration of Rel-16 V2X by configuring interlace RB allocation

Table 2-3: Common Parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Active cell(s) |  |  | None |
| PSCCH payloads |  | bit | 26 |
| Sidelink UE 1 | Sidelink Transmissions |  | PSCCH+PSSCH |
|  | Timing offset (Note 1) | s | CP/2-12*64*Tc |
|  | Frequency offset (Note 2) | Hz | +600 |
|  | Synchronization |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 Low |
|  | PSSCH RMC |  | TBD |
| NOTE 1: Time offset of transmitted Sidelink UE signal with respect to GNSS reference timing. NOTE 2: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. NOTE 3: OCC index i for PSCCH DMRS is randomly selected from {0, 1, 2} for each PSCCH transmission. |  |  |  |

Table 2-4: Test parameters

| Test number | PSCCH Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Probability of missed PSCCH (%) | SNR (dB) of PSCCH |
| 1 | TBD | 20 / 30 | TDLA30-1500 | 1 | TBD |

Proposal 9: RAN4 to consider the parameters in Table 2-3 and 2-4 for PSCCH requirements definition

PSFCH

In Rel-16 PSFCH test, RAN4 used NACK-only mode for requirements definition. However, UE has more complexity for ACK/NACK detection compared to NACK only detection. The reason is that for ACK/NACK mode, tested UE has to detect the energy on two cyclic shifts which is more complicated than NACK-only mode where tested UE only to detect the energy on one cyclic shift. So we propose to choose ACK/NACK model instead of NACK only model for PSFCH requirements definition

It's specified that PSFCH occupies K3 RBs each of which have same information, where K3 can be configured with {1,2,5}, we propose to configure K3=5 to verify the UE’s max capability.

Legacy PSFCH test procedure specifies that tested UE transmits PSSCH to TE firstly, then TE transmits PSFCH to UE and TE counts the number of retransmission to derive the NACK miss detection probability, resulting that test UE is responsible for initialling COT, which may mix the functional and performance test. It also causes the risk that COT duration is unpredictable, which may bring the challenge for designing the test setup.

Observation 1: Legacy PSFCH test procedure specifies that tested UE transmits PSSCH to TE firstly, then TE transmits PSFCH to UE and TE counts the number of retransmission to derive the NACK miss detection probability, resulting that tested UE is responsible for initialling COT, which may mix the functional and performance test. It also causes the risk that COT duration is unpredictable, which may bring the challenge for designing the test setup.

Proposal 10: RAN4 to further discuss how to design the LBT model and test setup for PSFCH performance test.

- Conclusion

In this contribution we provide our views on Rel-18 sidelink performance requirements definition. The proposals are:

Proposal 1: Use test parameters listed in Table 2-1 and Table 2-2 for SL CA performance test as starting part.

Proposal 2: Keep number of allocated RBs for each CC open until the RAN1’s discussions on capability of  “maximum number of non-overlapping RBs UE attempts to decode” is finalized.

Proposal 3: RAN4 to keep tracking on the RAN1 progress on following CA capability discussion and start the discussion once it is finalized by RAN1

Maximum number of receiving PSCCHs in a slot

Maximum number of receiving PSFCHs in a slot

Proposal 4: RAN4 to introduce performance requirements for PSSCH, PSCCH and PSFCH with interlaced RB allocation.

Proposal 5: Consider following test setup for SL-U test:

Carrier center frequency: 6.5GHz

- Operation mode: Mode2(Standalone)

- Synchronization source: GNSS based

- Carrier frequency offset with respect to GNSS: 650Hz

- Carrier frequency offset for simulation assumption: 1300Hz

- Time offset with respect to GNSS: CP/2-12*64*Tc

- Time offset for simulation assumption: 24*64*Tc

- SCS: 30kHz

- Antenna configuration: 1T2R Low

- Channel bandwidth: 20MHz

- Propagation conditions: Select one from {TDLA30-2900, TDLA30-1500, TDLA30-195}

- Channel estimation: MMSE based interpolation in frequency domain and linear interpolation in time domain

- Only consider 1 interlace (1 sub-channel) with RB index 0,5,10,15,…50

Proposal 6: RAN4 to consider following principle for LBT model:

Due to the utilization of HARQ-ACK feedback, LBT failure probability shall be set to 1 (pLBT=0) to guarantee that HARQ-ACK feedback is not impacted by the LBT failure.

It's typical to configure gap between two consecutive COTs to give TE more time to perform LBT, one potential way is to set the start position of PSSCH transmission in the first slot of each COT to #7

CPE extension should be configured for the first AGC symbol of each SL slot within the COT to make the gap between two consecutive slots smaller than 16us

The COT duration should be designed to guarantee that PSFCH is always transmitted in the slot with 14 symbol allocation.

35 SCI2 information bits are assumed during Rel-16 V2X test. However, to convey the COT information, SCI2 information bits is expected to be more which should be re-discussed.

Proposal 7: RAN4 to consider following LBT model as starting point:

TE performs LBT to initial a COT with  LBT failure probability equaling to 0 (pLBT=1) and share this COT with tested UE

The start symbol of first slot in each COT is #7

The COT duration is randomly selected from {2,4,8} slots

COT information is conveyed in SCI stage 2.

CPE extension is configured for the first AGC symbol of each SL slot within the COT

Tested UE uses the sharing COT to transmit PSFCH by via type 2 channel access

Proposal 8: RAN4 to consider following test configuration for PSSCH requirements definition:

Configure 1 PSSCH occasion for each PSSCH

MCS:16QAM, 0.37

Propagation: TDLA30-1500

PSFCH resource period: 4

MinTimeGap: 3

PSSCH DMRS pattern: 3 symbols for slot without PSFCH transmission and 2 symbols for slot with PSFCH transmission.

Proposal 9: RAN4 to consider the parameters in Table 2-3 and 2-4 for PSCCH requirements definition

Observation 1: Legacy PSFCH test procedure specifies that tested UE transmits PSSCH to TE firstly, then TE transmits PSFCH to UE and TE counts the number of retransmission to derive the NACK miss detection probability, resulting that tested UE is responsible for initialling COT, which may mix the functional and performance test. It also causes the risk that COT duration is unpredictable, which may bring the challenge for designing the test setup.

Proposal 10: RAN4 to further discuss how to design the LBT model and test setup for PSFCH performance test.

- Reference

[1]    R1-2317000 WF on UE demodulation performance for NR SL evolution.

[2]    R1-2310635 Updated RAN1 UE features list for Rel-18 NR after RAN1#114bis