3GPP TSG-RAN WG4 Meeting # 109	R4-2320293

Chicago, USA, November 13 – October 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	General aspects

Agenda item:	5.25.2.1

Document for:	Discussion

Introduction

In the last RAN4 meetings, RAN4 have been discussing the general aspects related to defining UE requirements related to the MUSIM gaps introduced in Rel-17.

RAN4#108bis:

Concerning Pre-MG and NCSG in Rel-18 MUSIM WI RAN4 agreed in the Xiamen meeting that Pre-MG and NCSG are not considered in Rel-18 MUSIM WI.

Additionally, it was agreed that there will be no further requirement and specification work needed related to number of MUSIM gaps the UE can request.

RAN4#108:

No agreements.

RAN4#107:

In Incheon meeting RAN4#107 meeting, RAN4 continued discussing ‘general aspects’ with progress on 2 issues [1]:

- It was agreed not to define any overhead cap for MUSIM gaps (Issue 1-1-2).

- It was agreed not to continue the discussion related to ‘General rule on properties for NW-A and NW-B procedures’ (Issue 1-1-4) under general aspects.

Additionally, in the RAN4#107 meeting, it was when discussing solutions for collision between different MUSIM gaps, RAN4 will define two solutions for collision handling between different MUSIM gaps:

- Priority based solution (i.e., network controls the MUSIM gaps priority).

- “Keep” solution (i.e., keep all collided MUSIM gaps).

It was recommended no longer to discuss any clarifications on the scope under the general aspects. Any such discussion can be under each Issue if/when needed (Issue 1-1-1).

RAN4#106bis:

In RAN4#106bis meeting (e-meeting) some agreements were made related to issue 1-1-3: Total number of gaps when MUSIM gaps are configured. However, one part was left open related to issue 1-1-3 (P4).

RAN4#106:

In the Athens 106 meeting an agreement was reached regarding one-shot RRM mobility procedures where RAN4 agreed that there is no need to consider the collision between SMTC and MUSIM gaps for RRC Re-establishment, RRC Connection Release with Redirection [2].

(Earlier meeting agreements are not listed).

In the last RAN4 meeting in Xiamen (RAN4#108bis), the discussion related to general aspects continued. One aspect is still open:

- Mandatory MUSIM gap patterns.

We give our view on this aspect in this paper.

Discussion

Mandatory MUSIM gap patterns

In RAN4#108bis meeting in Xiamen the discussion related to defining one or more mandatory MUSIM gaps continued. The outcome from the meeting is captured in the WF [1]:

Issue 1-1-1: Mandatory MUSIM gap patterns

- Proposals

  - P1: No need to discuss further whether to introduce mandatory MUSIM gap patterns (Apple oppo Huawei MTK Qualcomm)

  - P2: RAN4 to define the mandatory MUSIM gap patterns (CMCC Ericsson Nokia Charter Communications)

  - P3: No more discussion if there is no consensus (vivo)

No agreement was reached.

Although we support the introduction of at least 1 or 2 mandatory MUSIM gaps, our understanding is that the current RAN4 MUSIM work is related to defining UE requirements for the MUSIM gaps RAN4 defined in Rel-17, which were introduced without defining any associated UE requirements. We do not however see that the Rel-17 decision would hinder RAN4 in introducing one or more mandatory MUSIM gaps in Rel-18.

Rel-17 decision not introducing any mandatory MUSIM gaps does not hider introduction of one or more mandatory MUSIM gaps in Rel-18.

However, as mentioned several times during the discussions, the complexity of supporting MUSIM gaps on the network side will be either be very large, if the network has to support all possible MUSIM gaps. Alternative is that the network down selects to supporting only a few selected MUSIM gap patterns. This way the network complexity can be greatly reduced. However, the drawback of such down selection is of course that the network may then not support the MUSIM gap(s) requested by the UE, and network cannot allocate one or more of requested MUSIM gaps.

Without any mandatory MUSIM gaps, we see following scenarios:

- Network supports all possible MUSIM gaps and hence can handle any MUSIM gap pattern request from any UE in the field.

- Network supports a limited number of MUSIM gaps. MUSIM gaps will only be allocated if the UE requests one or more of the MUSIM gap patterns supported by the network.

- Network does not support MUSIM gaps due to the uncertainty which MUSIM gaps to support.

From a network perspective it does not seem realistic that network will be able to implement and support all the defined MUSIM gaps.

If the requested MUSIM gaps are not supported by the network, this brings up two possible options for the network:

- Network allocates the requested MUSIM which are supported. Other non-supported MUSIM gaps can (of course) not be allocated to the UE. Drawback of the network not being able to allocate all UE requested MUSIM gaps, is that this may lead to that there will be no applicable UE requirements related to the allocated MUSIM gaps. Hence, it is questionable if it makes sense to even allocate any requested MUSIM gaps if network cannot allocate exactly what is requested by the UE.

- Network chose not to allocate any MUSIM gaps to the UE if the UE requests one or more MUSIM gaps which are not supported by the network. This has the drawback that the UE may have difficulties in performing MUSIM operations in network-B without interfering (causing interruptions) to network-A. However, RAN4 has earlier agreed that the UE shall not cause any interruptions in network-A due to MUSIM operations in network-B if UE has requested but is not allocated the requested MUSIM gaps.

Hence, to provide a better overall MUSIM feature we suggest defining 1 or 2 mandatory MUSIM gaps.

Introduce 1 or 2 mandatory MUSIM gaps.

RAN4 can discuss which MUSIM gaps should be mandatory. Such discussion could account any use case and the UE NW-B requirements.

In general, we believe a MUSIM gap with MGRP of 160ms should be reasonable based on our expectation of UE operations in NW-B:

- ‘Intra-frequency’ cell detection and measurements in NW-B

- Paging reception on ‘serving cell’ in NW-B

However, the need for NW-B ‘inter-frequency’ measurements is still under discussion elsewhere.

As minimum the UE shall support MUSIM gap 6ms MGL and 160ms MGRP.

Conclusion

In the paper, we have been discussing the general aspects related to defining UE requirements related to the MUSIM gaps introduced in Rel-17.

Based on the open issues from last meeting and considering the discussion in this paper, we observe and propose:

Mandatory MUSIM gap patterns:

- Rel-17 decision not introducing any mandatory MUSIM gaps does not hider introduction of one or more mandatory MUSIM gaps in Rel-18.

- Introduce 1 or 2 mandatory MUSIM gaps.

- As minimum the UE shall support MUSIM gap 6ms MGL and 160ms MGRP.

- References

- R4-2317425, WF on R18 MUSIM, vivo

- R4-2314364, WF on R18 MUSIM, vivo

- R4-2310165, WF on MUSIM RRM requirements, vivo

- R4-2303248, WF on NR Dual TxRx Multi-SIM, vivo

- R4-2217261, WF on RRM requirements for Rel-17 MUSIM gaps, vivo.

- R4-2220443, WF on NR Dual Tx/Rx Multi-SIM, vivo.

- R4-2306357, WF on NR Dual Tx/Rx Multi-SIM, vivo