3GPP TSG-RAN WG4 Meeting #109	R4-2321563

Chicago, USA, November 13 – November 17, 2023

Title: 	WF on NR_MIMO_evo_DL_UL WI

Agenda Item:	8.29.5

Source: 	Samsung

Document for:	Approval

<Topic 1 Feasibility study of TDCP accuracy >

Issue 1-1-1: Is it feasible to define TDCP accuracy requirement for TDCP?

Agreement:

- No RRM core requirement to be defined, and further discuss the following options:

  - Option 1: Define accuracy and test cases as part of performance requirement

  - Option 2: Do not define accuracy requirement but define test cases for particular configurations as part of performance requirements

  - Option 3: Do not define accuracy requirement and test cases as part of performance requirements

<Topic 2 Timing requirements for UL multi-DCI multi-TRP with two TAs >

Issue 2-1-1: TAG management for multi-TRP with 2 TAs

Agreement:

- When the transmission timing difference between two TAGs for multi-TRP operation exceeds the MTTD value, no additional UE behaviour to be defined in RAN4.

Issue 2-1-2: Number of downlink timing reference

Way forward:

- Option 1:

  - The number of downlink timing references should be equal to the number of TAGs across CCs.

<Topic 3 Unified TCI Framework extended to M-TRP >

Issue 3-1-1: For sDCI mTRP, how to specify DL MAC CE based dual TCI state switch the switching delay requirements for Case 1, if SSB are adjacent in FR2?

Agreement:

The SSB periodicity is the same for serving cell

- THARQ + 3Nslotsubframe,µ + max{TOk1*(Tfirst-SSB1 +AD1*TSSB1+ TSSB-proc), TOk2*(Tfirst-SSB2 +AD2*TSSB2+ TSSB-proc)} / NR slot length

  - AD1 = 1 if SSBs are adjacent in FR2 and TSSB1 =TSSB2 ; 0 otherwise

  - AD2 = 1 if SSBs are adjacent in FR2 and TSSB2 = TSSB1 ; 0 otherwise

Issue 3-1-2: For sDCI mTRP, how to specify DL MAC CE based dual TCI state switch the switching delay requirements for Case 2, if SSB are adjacent in FR2?

Agreement:

- (TSSB1 = TSSB2): THARQ + 3Nslotsubframe,µ + TL1-RSRP1 + max {TOuk1*(Tfirst-SSB1+ AD1*TSSB1 + TSSB-proc), TOk2*(Tfirst-SSB2 + TSSB-proc)} / NR slot length; AD1 = 1 if SSBs are adjacent in FR2; 0 otherwise

Issue 3-1-3: For sDCI mTRP, how to specify DL MAC CE based dual TCI state switch the switching delay requirements for Case 3, if SSB are adjacent in FR2?

Way forward:

- Option 1:

  - Longer delay is expected

- Option 2:

  - Additional SSB can be added in the MAC-CE based TCI state switching

Issue 3-1-4: For sDCI mTRP, end point of dual TCI state switch delay requirements?

Agreement:

- Dual TCI state switch requirements for sDCI are defined for the case when UE is expected to receive PDCCH/PDSCH from both TCI states/ TRPs after the switch.

Issue 3-1-5 For sDCI mTRP, active TCI state list update delay

Agreement:

- For the case of overlapping/adjacent SSB in FR2, additional SSB can be added in active TCI state list update delay

Issue 3-1-7-a For mDCI mTRP, how to specify DL TCI state switching requirements for eUTCI if UE not supporting two TAs?

Agreement:

- [Known]: THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) + OL*TSSB

- [Unknown]: THARQ + 3Nslotsubframe,µ+ TL1-RSRP + TOuk*(Tfirst-SSB + TSSB-proc) + OL*TSSB

- OL=1 if SSB overlaps or adjacent to SSB from other TRP in FR2 and SSB periodicity is less than that of other TRP, 0 otherwise

Issue 3-1-7-b For mDCI mTRP, how to specify UL TCI state switching requirements for eUTCI if UE not supporting two TAs?

Agreement:

- Rel-17 unified TCI state switching requirements are applicable for each TCI state associated with coresetPoolIndex independently

Issue 3-1-8-a: For mDCI mTRP, how to specify DL TCI state switching requirements for eUTCI if UE supporting two TAs and but not supporting RTD>CP?

Agreement:

- [Known]: THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) + OL*TSSB

- [Unknown]: THARQ + 3Nslotsubframe,µ+ TL1-RSRP + TOuk*(Tfirst-SSB + TSSB-proc) + OL*TSSB

- OL=1 if SSB overlaps or adjacent to SSB from other TRP in FR2 and SSB periodicity is less than that of other TRP, 0 otherwise

Issue 3-1-8-b: For mDCI mTRP, how to specify UL TCI state switching requirements for eUTCI if UE supporting two TAs and but not supporting RTD>CP?

Way forward:

- Option 1:

  - [Known case] THARQ + 3Nslotsubframe,µ + TOk-ref (Tfirst-SSB-DLRef + 2ms)+NM*( Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms)

  - [Unknown case]: THARQ + 3Nslotsubframe,µ + TL1-RSRP + TOuk-ref (Tfirst-SSB-DLRef + 2ms)+ Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms

- Option 2:

  - Rel-17 unified TCI state switching requirements are applicable for each TCI state associated with coresetPoolIndex independently

  - [Known case] THARQ + 3Nslotsubframe,µ + NM*( Tfirst_target-PL-RS  + 4*Ttarget_PL-RS + 2ms)

  - [Unknown case]: THARQ + 3Nslotsubframe,µ + TL1-RSRP + Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms

Issue 3-1-9-a: For mDCI mTRP, how to specify DL TCI state switching requirements for eUTCI if UE supporting two TAs and supporting RTD>CP in FR1?

Agreement:

- Rel-17 unified TCI state switching requirements are applicable for each TCI state associated with coresetPoolIndex independently

Issue 3-1-9-b: For mDCI mTRP, how to specify UL TCI state switching requirements for eUTCI if UE supporting two TAs and supporting RTD>CP in FR1?

Way forward:

- Option 1:

  - [Known case] THARQ + 3Nslotsubframe,µ + TOk-ref (Tfirst-SSB-DLRef + 2ms)+NM*( Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms)

  - [Unknown case]: THARQ + 3Nslotsubframe,µ + TL1-RSRP + TOuk-ref (Tfirst-SSB-DLRef + 2ms)+ Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms

- Option 2:

  - Rel-17 unified TCI state switching requirements are applicable for each TCI state associated with coresetPoolIndex independently

  - [Known case] THARQ + 3Nslotsubframe,µ + NM*( Tfirst_target-PL-RS  + 4*Ttarget_PL-RS + 2ms)

  - [Unknown case]: THARQ + 3Nslotsubframe,µ + TL1-RSRP + Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms

Issue 3-1-10 For mDCI mTRP, active TCI state list update delay

Agreement:

- For the case of overlapping/adjacent SSB in FR2, additional SSB can be added in active TCI state list update delay

Issue 3-1-11: L1-RSRP measurement when RTD>CP

Agreement:

- Define requirements for FR1 for intra/inter-cell.

- For FR1, if UE supports RTD>CP, reuse Rel-17 L1-RSRP measurement period in 9.13 for a cell with different PCI from serving cell, remove the RTD is smaller than CP side condition depending on UE capability. Reuse L1-RSRP measurement period in 9.5 for serving cell.

- If UE does not support RTD>CP, for RTD is larger than CP, no requirements.

- FFS on scheduling/measurement restriction for different SCS and TDD.

Issue 3-1-12: L1-RSRP measurement for resource type of mTRP for a cell with different PCI from serving cell

Agreement:

- SSB only, no CSI-RS

<Topic 4 performance part >

Issue 5-1-2: TDCP Measurement Report Mapping

Way forward:

- Option 1:

  - Add the below table for amplitude reporting in 38.133

- Table: Quantization of amplitude value for TDCP reporting

| Value of q | TDCP Range |  |
| --- | --- | --- |
| 0 | 1-2-16-1/2 ~ 1 | 0.9945< TDCP <=1 |
| 1 | 1-2-16-2/2 ~ 1-2-16-1/2 | 0.9922< TDCP <=0.9945 |
| 2 | 1-2-16-3/2 ~ 1-2-16-2/2 | 0.9890< TDCP <=0.9922 |
| 3 | 1-2-16-4/2 ~ 1-2-16-3/2 | 0.9844< TDCP <=0.9890 |
| … | … | … |
| 12 | 1-2-16-13/2 ~ 1-2-16-12/2 | 0.6464< TDCP <=0.75 |
| 13 | 1-2-16-14/2 ~ 1-2-16-13/2 | 0.5< TDCP <=0.6464 |
| 14 | 1-2-16-15/2 ~ 1-2-16-14/2 | 0.2929< TDCP <=0.5 |
| 15 | 1-2-16-16/2 ~ 1-2-16-15/2 | 0< TDCP <=0.2929 |

Issue 5-2-1: Whether to define TCs for two TAs?

Way forward:

- Option 1:

  - Yes

| Samsung | Uplink transmit timing for two cells to support two TA – NR SA in FR1 Uplink transmit timing for two cells to support two TA – NR SA in FR2 |
| --- | --- |
| Huawei | New timing advance adjustment test |
| Nokia | - Test case for the two TA feature including: - A setup with two TRPs, each sending its own TA command; - An adjustable timing delay between the two cells/TRPs to verify MRTD and MTTD requirements. - Test case for the two TA feature including two MRTD configurations, assuming either MRTD > CP or MRTD = CP; a UE will only run one of the two configurations depending on whether it supports MRTD > CP or MRTD = CP. |
| Ericsson | • UE Transmit Timing Test in FR1 • UE Transmit Timing Test in FR2 • Timing advance adjustment accuracy for FR1 • Timing advance adjustment accuracy for FR2 |

Issue 5-2-2: Whether to define TCs for m-DCI mTRP cases?

Way forward:

- Option 1:

  - Yes

| Samsung | m-TRP MAC-CE based TCI state switch delay - TC-x: NR FR2, inter-cell mDCI + DL TCI + both two are known, RTD<CP - TC-x: NR FR2, inter-cell mDCI + UL TCI + both two are known, RTD>CP |
| --- | --- |
| Huawei | - mDCI FR2 MAC-CE based active joint TCI state switch for a known TCI state - mDCI FR2 MAC-CE based active DL TCI state switch for a known TCI state - mDCI FR2 MAC-CE based active UL TCI state switch for a known TCI state - mDCI FR2 MAC-CE based active joint TCI state switch for a known TCI state with RTD larger than CP - mDCI FR2 MAC-CE based active DL TCI state switch for a known TCI state with RTD larger than CP - mDCI FR2 MAC-CE based active UL TCI state switch for a known TCI state with RTD larger than CP |

  - Option 2: (Apple)

    - No

Issue 5-2-2: Whether to define TCs for s-DCI mTRP cases?

Way forward:

- Option 1:

  - Yes

| Apple | - No TC for eUTCI for sDCI mTRP with single TCI state switch - TCs for eUTCI for sDCI mTRP with dual TCI state switch in FR2 - Separate TCI state switch on DL, with sDCI TDM transmission scheme - Separate UL TCI state switch, with PUSCH repetition |
| --- | --- |
| Samsung | m-TRP MAC-CE based TCI state switch delay - TC-x: NR FR2, intra-cell (Serving cell ) sDCI + DL TCI + dual TCI state switching + one is known; one is unknown - TC-x: NR FR2, intra-cell (Serving cell ) sDCI + UL TCI + dual TCI state switching + both two are known - TC-x: NR FR2, intra-cell (Serving cell ) sDCI + DL TCI + dual TCI state switching + two are unknown |
| Huawei | - sDCI FR2 MAC-CE based active joint TCI state switch for a known TCI state - sDCI FR2 MAC-CE based active DL TCI state switch for a known TCI state - sDCI FR2 MAC-CE based active UL TCI state switch for a known TCI state |
| Nokia | Dual TCI state switching: - Case 1: Both target TCIs are known; - Case 2: One of target TCIs is unknown and another is known; - Case 3: Both target TCIs are unknown. |
| Ericsson | MAC CE based unified TCI state switching should be tested for - joint TCI state switching - Separate DL TCI state switch - Separate UL TCI state switch |