3GPP TSG-RAN WG4 Meeting # 110	R4-2402358

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

Title:	TP to TR 36.718-02-01 Addition of CA_7-40

Source:	Samsung, Spark NZ Ltd

Agenda item:	9.1.3.1

Document for:	Approval

# 1	Introduction

This TP is to add CA_7A-40A, CA_7A-40C, CA_7A-40D, and CA_7A-40A-40A with ULCA. CA_7A-40A, CA_7A-40C, CA_7A-40D, and CA_7A-40A-40A with single UL have been specified.

************************************* Start of TP*****************************************

### 5.3.x	CA_7-40, CA_7-40-40

#### 5.3.x.1	Channel bandwidths per operating band for CA

Table 5.3.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 5.3.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_7A-40A | CA_7A-40A | 7 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7A-40C | CA_7A-40A | 7 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_7A-40D | CA_7A-40A | 7 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 40 | See CA_40D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_7A-40A-40A | CA_7A-40A | 7 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |

#### 5.3.x.2	Co-existence studies

Since 2DL / 1UL fallbacks have been specified, there is no need to study harmonic and cross band isolation issues again. For 2DL / 2UL own receiver desensitization study 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.3.x.2-1.

Table 5.3.x.2-1: IMD analysis UL_CA_7A-40A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 2500 | 2570 | 2300 | 2400 |
| 2nd order IMD products | ¦fx_low – fy_high¦ | ¦fx_high – fy_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 100 | 270 | 4800 | 4970 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2600 | 2840 | 2030 | 2300 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7300 | 7540 | 7100 | 7370 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5100 | 5410 | 4330 | 4700 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9800 | 10110 | 9400 | 9770 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 200 | 540 | 9600 | 9940 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6630 | 7100 | 7600 | 7980 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11700 | 12170 | 12300 | 12680 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1760 | 2200 | 2700 | 3110 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11900 | 12340 | 12100 | 12510 |

Based on Table 5.3.x.2-1, IMD3 issue caused by 7+40 falls into band 7 Rx.

Table 5.3.x.2-2: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_7-40 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

#### 5.3.x.3	∆TIB and ∆RIB values

Already included in TS 36.101.

#### 5.3.x.4	REFSENS Requirements

Based on the co-existence studies there is a need to define additional REFSENS requirements. MSD value and test points are taken from NR CA_n7-n40.

Table 5.3.x.4-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_7A-40A CA_7A-40C CA_7A-40D CA_7A-40A-40A | 7 | 2510 | 5 | 25 | 2630 | 23 | FDD | IMD3 |
|  | 40 | 2390 | 5 | 25 | 2390 | N/A | TDD | N/A |

************************************* End of TP*****************************************

# 2	References

[1] RP-233927_Revised WID: Rel-18 LTE-Advanced Carrier Aggregation for x bands (2<=x<= 6) DL with y bands (y=1, 2) UL