3GPP TSG-RAN4 Meeting #110	R4-2400100

Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3960 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_Mob_enh2-Core) CR on intra-f L1 measurements for neighbor cell for LTM requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-01-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Correct the clause number for known cell conditions for LTM. - Remove redundant description of note 2 in Table 9.14.5.1-3 for intra-f L1 measurement period. - Correct note 3 in Table 9.14.5.1-3 and note 2 in Table 9.14.5.1-4 for intra-f L1 measurement period. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise the following parts in TS 38.133 for R18 LTM intra-f L1 measurement requirements: - Revise ‘9.x.2’ to ‘9.14.2’ in clause 9.14.5.1. - Delete ‘no requirements’ in note 2 of Table 9.14.5.1-3 in clause 9.14.5.1. - Add ‘and inter-frequency without gap’ in note 3 of Table 9.14.5.1-3 and note 2 of Table 9.14.5.1-4 in clause 9.14.5.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - ‘9.x.2’ in clause 9.14.5.1 will be unclear. - Note 2 in Table 9.14.5.1-3 in clause 9.14.5.1 will be unclear. - Necessary description in note 3 of Table 9.14.5.1-3 and note 2 of Table 9.14.5.1-4 in clause 9.14.5.1 will be missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.14.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.14.5.1	SSB based L1-RSRP Reporting

The requirements specified in this clause are only applicable when

-	highSpeedMeasFlag-r16 is not configured, and

-	highSpeedMeasFlagFR2-r17 is not configured, and

-	highSpeedMeasCA-Scell-r17 is not configured.

-	The SSB from the neighbor cell completely contained in the active BWP of the UE

If a neighbor cell is known according 9.14.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_intra if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell. Otherwise, UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_intra+TSSB_time_index_intra, where TSSB_time_index_intra is the time period used to acquire the index of the SSB being measured given in Table 9.2.5.1-3.

The value of TL1-RSRP_Measurement_Period_SSB_intra is defined for FR1 in Table 9.14.5.1-1 for UE incapable of [capability of measurement with RTD>CP] and in Table 9.14.5.1-2 for UE capable of [capability of measurement with RTD>CP].  The value of TL1-RSRP_Measurement_Period_SSB_intra is defined for FR2 in Table 9.14.5.1-3 for UE incapable of [capability of measurement with RTD>CP] and in Table 9.14.5.1-4 for UE capable of [capability of measurement with RTD>CP], where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3  otherwise

-	N= 8.

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

For a window W of duration max (TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target SSB.

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = [3], otherwise.

-	PL1_sharing is defined as

-	When number of neighboring cells to be measured is 1

-	PL1_sharing = 2, if any symbol of the SSBs from serving cell and neighbor cell are overlapping or adjacent (in time domain)

-	PL1_sharing = 1, otherwise

-	When number of neighboring cells to be measured is more than 1

-	When TCI state of neighbor cells are not in the active TCI state list

-	PL1_sharing = 3*NNeighbor_Cell, where NNeighbor_Cell is the number of neighbor cells to measure on intra-frequency and inter-frequency without gap

-	Otherwise

-	PL1_sharing = 3*NNeighbor_Cell_in_list, where NNeighbor_Cell_in_list  is the number of neigbour cells (including intra-frequency neighbor cells and inter-frequency without gap neighbor cells) whose TCI state(s) are in the active TCI state list.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Longer measurement period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For either an FR1 or FR2 cell, longer measurement period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.14.5.1-1: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE incapable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. Note 3: No requirements if the actual RTD of serving cell and neighbor cell is larger than CP. |  |

Table 9.14.5.1-2: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE capable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC*NLayer) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC) *NLayer) |
| DRX cycle > 320ms | ceil(M*P)*TDRX*NLayer |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. [NLayer = the number of intra-frequency layers configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18] + the number of inter-frequency layers without measurement gaps which are configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18]]. Note 2: K = 1.5. |  |

Table 9.14.5.1-3: Intra-frequency L1-RSRP measurement period TIntra_ L1-RSRP_Measurement_Period_SSB in FR2 for UE incapable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*PL1_sharing *N)*max(TDRX,TSSB_NBC) ) |
| DRX cycle > 320ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: No requirements, if the actual RTD of serving cell and neighbor cell is larger than CP. Note3: When the number of neighbor cells to be measured on intra-frequency and inter-frequency without gap is more than 1 and TCI state(s) of one of these neighbor cells is in the active TCI state, no requirements for other intra-frequency and inter-frequency without gap neighbor cells except the one whose TCI state(s) is in the active TCI state. |  |

Table 9.14.5.1-4: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR2 for UE capable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*PL1_sharing*N)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: When the number of neighbor cells to be measured on intra-frequency and inter-frequency without gap is more than 1 and TCI state(s) of one of these neighbor cells is in the active TCI state, no requirements for other intra-frequency and inter-frequency without gap neighbor cells except the one whose TCI state(s) is in the active TCI state. |  |

<End of Change 1>