3GPP TSG-RAN WG4 Meeting # 110	           R4-2400102

Athens, GR, 26th Feb – 01st Mar, 2024

Title:	Reply LS on PDCCH order RACH on neighbour cell

Response to:	R1-2304276

Release:	Rel-18

Work Item:	NR_mob_enh2-Core

Source:	RAN WG 4

To:	RAN WG 1

Cc:	RAN WG 2

Contact person:

Name:	Lingyu Gao

E-mail Address:	gaolingyu@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1 Overall description

RAN4 thanks RAN1 for the LS sent in R1-2304276 asking about the time gap between a PDCCH order and the corresponding PRACH transmission and the impact/interruption on UL Tx and/or DL Rx of serving cell due to the PRACH Tx on a candidate cell.

In RAN4 #108 meeting, RAN4 replied to RAN1 in R4-2314454 and RAN4’s feedback is copied below:

For the need for any update is required to ΔBWPSwitching, ΔDelay, - For ΔDelay: RAN4 has agreed to not change the component. - For ∆BWPSwitching: It is not needed. For the need for additional latency, RAN4 agreed to introduce new additional delay components at least for SSB based T/F tracking (TSSB) and RF and/or BB preparation and retuning (∆RF/BB_preparation). The additional delay components introduced are clarified as follows: TSSB: - If TCI state of target cell has been activated before PDCCH ordered RACH, and if SSB index indicated in PDCCH order is in the active TCI state list, and measurement period of L1-RSRP is no longer than 160ms, UE doesn’t need additional time for SSB based T/F tracking to meet UL transmission timing requirements, that is, TSSB = 0. - If SSB index indicated in PDCCH order is not in the active TCI state list that has been activated for the target cell, and when the measurement period of L1-RSRP is no longer than 160ms, additional delay is needed for fine time tracking, TSSB is FFS. - Otherwise, TSSB is needed, and the value is FFS. ∆RF/BB_preparation: - For the case of PRACH bandwidth of neighbor cell is within active UL BWP, ∆RF/BB_preparation is FFS. - For the case of PRACH bandwidth outside active UL BWP but within one of configured UL BWPs of any active serving cell, ∆RF/BB_preparation is DCI based BWP switching delay specified in clause 8.6 of TS 38.133 (in TS 38.133, DCI based BWP switch delay value is dependent on UE capability). - For the case of PRACH bandwidth is not within any of the configured UL BWPs of any active serving cell, ∆RF/BB_preparation is FFS For any impact/interruption on UL Tx and/or DL Rx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH, RAN4 is still discussing.

For FFS, RAN4 has further discussed delay requirements and interruption requirements for PDCCH ordered RACH on neighbour cell in RAN4 #108-bis and #109 meeting. RAN4 sincerely asks RAN1 to consider potential spec update based on RAN4’s following feedback.

RAN4 feedback:

In RAN4 understanding, the interruption requirements of PDCCH ordered RACH on neighbour cell have been captured in RAN4 spec and the delay requirements of PDCCH ordered RACH on neighbour cell will be captured in RAN1 spec as legacy. RAN4 spec will not capture any definition of TSSB and ∆RF/BB_preparation for PDCCH ordered RACH on neighbour cell. RAN4 sincerely asks RAN1 to consider whether and how to capture the definition for TSSB and ∆RF/BB_preparation in RAN1 spec.

For information, the new agreements regarding delay requirements and interruption requirements for PDCCH ordered RACH on neighbour cell in RAN4 #108-bis and #109 meeting are as follows:

For additional latency，

TSSB：

- For FR2, TSSB is always assumed before UE transmit PDCCH-ordered RACH.

- For FR1, when TCI state associated the PDCCH-order RACH has not been activated,

- TSSB is always assumed before UE transmit PDCCH-ordered RACH.

- For FR1, when TCI state associated the PDCCH-order RACH is activated,

- If L1-RSRP measurement delay is less than or equal to 160ms, TSSB is not needed. UE is required to meet the UL Tx timing accuracy requirements.

- If L1-RSRP measurement delay is more than 160ms, UE is allowed to transmit PDCCH-ordered RACH with or without TSSB. No UL Tx timing accuracy requirement for UE will be defined.

Note：The additional time for T/F tracking TSSB is one complete SSB burst.

∆RF/BB_preparation:

- For the case of PRACH bandwidth of neighbor cell is within active UL BWP, ∆RF/BB_preparation = 0.

- For the case of PRACH bandwidth is not within any of the configured UL BWPs of any active serving cell, RAN4 will introduce the UE capability to report ∆RF/BB_preparation.

For any impact/interruption on UL Tx and/or DL Rx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH,

- There will be interruption on UL Tx and DL Rx of all the serving cells during PRACH transmission.

- RAN4 understands that RAN1 is already discussing the interruption on UL during PDCCH order RACH transmission.

- RAN4 will introduce the UE capability to indicate whether there will be interruption on DL during PDCCH order RACH transmission to target cell.

- There will be interruption on UL Tx and DL Rx of all the serving cells due to RF/BB retuning to target cell before RACH transmission or retuning back to serving cell after RACH transmission.

- When PRACH bandwidth is in the UL active BWP, reuse legacy N symbols defined in TS 38.213.

- For the case of PRACH bandwidth outside active UL BWP but within one of configured UL BWPs of any active serving cell, reuse interruption requirements of BWP switching on other serving cells in NR-DC for asynchronous scenarios which are defined in TS 38.133 cl. 8.2.4.2.5.

- For the case of PRACH bandwidth not within any of the configured UL BWPs of any active serving cell, the interruption on both UL and DL is Y, which is up to UE capability. The candidate values for Y are 0.25ms, 0.5ms, 1ms and 2ms.

- The scheduling restriction or interruption due to additional DL synchronization after reception of PDCCH order on neighbour cell if needed can be covered by scheduling restriction due to L1/L3 measurement or MG.

2 Actions

To RAN WG1 group

ACTION: 	RAN4 sincerely asks RAN1 to take into account the above feedback into consideration, and consider to capture the definition for TSSB and ∆RF/BB_preparation in RAN1 spec, and provide feedback if any.

3 Dates of Next TSG RAN WG4 Meetings

TSG-RAN WG4 Meeting #110-bis	15 Apr – 19 Apr 2024	      China (TBC), CN

TSG-RAN WG4 Meeting #111	20 May –24 May 2024	       Japan , JP