3GPP TSG-RAN WG4 Meeting #110	R4-2400107

Athens, GR, 26th Feb – 01st Mar, 2024

Title: 	Discussion on RRM performance requirements for R18 enhanced RedCap

Source: 	CATT

Agenda item:	8.23.3

Document for:	Discussion

- Introduction

In previous meeting, RRM core requirements for R18 enhanced RedCap has been defined. This paper will discuss RRM performance requirement for enhanced RedCap and presents our views and proposals.

- Discussion

Test cases for enhanced RedCap

In Rel-18 enhanced RedCap core requirements, RAN4 has discussed the RRM impact on eDRX cycle > 10.24 sec and baseband reduction, and mainly focused on the requirements for eDRX cycle > 10.24 sec in FR1 and FR2 for RedCap UE [1]. For performance requirements, we provide our views and proposals about the test coverage for eRedCap UE in this contribution.

Firstly, considering the eDRX cycle range in FR1 and FR2 and the baseband reduction for Rel-18 enhanced RedCap UE, at least the new BWP configurations and SSB configurations shall be introduced in clause A.3. For NR configuration in different tests, a new supported configuration in test parameters shall be introduced.

Proposal 1: For test of Rel-18 enhanced RedCap UE, at least the new BWP configurations and SSB configurations shall be introduced in clause A.3.

Proposal 2: For NR configuration in different tests, a new supported configuration in test parameters shall be introduced into the existing testing parameters.

According to the core requirements in Rel-18 enhanced RedCap, the new test cases for RRC_INACTIVE state mobility in SA can be introduced and the test parameters for RRC_IDLE state mobility can be used as a baseline [1]. For the specific test cases, at least the following new test cases for cell re-selection for eRedCap can be specified in RAN4, that is:

Cell re-selection to NR for eRedCap

- Test cases for cell reselection to FR1 intra-frequency NR case

- Test cases for cell reselection to FR2 intra-frequency NR case

- Test cases for cell reselection to FR1 inter-frequency NR case

- Test cases for cell reselection to FR2 inter-frequency NR case

Inter-RAT E-UTRAN cell re-selection for eRedCap

- Test cases for cell reselection to higher priority E-UTRAN

- Test cases for cell reselection to lower priority E-UTRAN

Proposal 3: The new test cases for RRC_INACTIVE state mobility in SA shall be introduced and the test parameters for RRC_IDLE state mobility can be used as a baseline.

Proposal 4: For Rel-18 eRedCap, the following new test cases for cell re-selection for RRC_INACTIVE state in SA should be specified in RAN4:

- Cell re-selection to NR for eRedCap

- Test cases for cell reselection to FR1 intra-frequency NR case

- Test cases for cell reselection to FR2 intra-frequency NR case

- Test cases for cell reselection to FR1 inter-frequency NR case

- Test cases for cell reselection to FR2 inter-frequency NR case

- Inter-RAT E-UTRAN cell re-selection for eRedCap

- Test cases for cell reselection to higher priority E-UTRAN

- Test cases for cell reselection to lower priority E-UTRAN

- Conclusion

In this paper, we provide our views on RRM performance requirement for R18 enhanced RedCap. From this discussion we have derived the following proposals:

Proposal 1: For test of Rel-18 enhanced RedCap UE, at least the new BWP configurations and SSB configurations shall be introduced in clause A.3.

Proposal 2: For NR configuration in different tests, a new supported configuration in test parameters shall be introduced into the existing testing parameters.

Proposal 3: The new test cases for RRC_INACTIVE state mobility in SA shall be introduced and the test parameters for RRC_IDLE state mobility can be used as a baseline.

Proposal 4: For Rel-18 eRedCap, the following new test cases for cell re-selection for RRC_INACTIVE state in SA should be specified in RAN4:

- Cell re-selection to NR for eRedCap

- Test cases for cell reselection to FR1 intra-frequency NR case

- Test cases for cell reselection to FR2 intra-frequency NR case

- Test cases for cell reselection to FR1 inter-frequency NR case

- Test cases for cell reselection to FR2 inter-frequency NR case

- Inter-RAT E-UTRAN cell re-selection for eRedCap

- Test cases for cell reselection to higher priority E-UTRAN

- Test cases for cell reselection to lower priority E-UTRAN

- Reference

[1]    3GPP TS 38.133: “NR; Requirements for support of radio resource management”.