3GPP RAN WG4 Meeting #110		R4-2400193

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	8.12.2

Source:	Apple

Title:	On FR1 MIMO OTA Lab Alignment PADs measurement results

Release:	Rel-18

WI/SI:	NR_MIMO_OTA_enh-Core

Document for:	Information

# 1	Discussion

This contribution provides the measurement data at Apple’s FR1 MIMO OTA lab, in compliance with requirements and timeline defined on [1].

### Sub-topic 1-3 Rel-18 FR1 MIMO OTA lab alignment

Issue 1-3: Preliminary outcome of Rel-18 FR1 MIMO OTA lab alignment

For information: The summary of current lab alignment results is shown in Table 1 and Figs. 1~2.

Table 1.  Summary of measurement results from 5 labs

| Device | Band | TRMS measurement result [dBm/15kHz] |  |  |  |  |  | Average approach | Average value | Max-Min deviation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Lab 1 | Lab 2 | Lab 3 | Lab 4 | Lab 5 | Lab 6 |  |  |  |
| PAD_1 | n28 | -89.34 | -90.58 | -89.95 | -88.26 | -89.22 |  | Linear average | -89.47 | 2.32 |
| PAD_2 | n28 | -86.46 | -88.39 | -88.61 | -85.50 | -85.86 |  |  | -86.96 | 3.12 |
| PAD_3 | n28 | -86.26 | -89.80 | -88.75 | -86.19 | -86.49 |  |  | -87.50 | 3.61 |
| Device | Band | TRMS offset [dBm/15kHz] |  |  |  |  |  | Pass/fail limit |  |  |
|  |  | Lab 1 | Lab 2 | Lab 3 | Lab 4 | Lab 5 | Lab 6 |  |  |  |
| PAD_1 | n28 | 0.13 | -1.11 | -0.48 | 1.21 | 0.25 |  | [+/- 0.75 MU, i.e., +/- 2.25 dB] |  |  |
| PAD_2 | n28 | 0.50 | -1.42 | -1.65 | 1.47 | 1.10 |  |  |  |  |
| PAD_3 | n28 | 1.23 | -2.30 | -1.25 | 1.31 | 1.01 |  |  |  |  |

Fig. 1  Measurement results from 5 labs

Fig. 2  Deviation between each measurement result and the average value

Agreement:

- Reuse the pass/fail limit of Rel-17 FR1 MIMO OTA lab alignment, i.e., +/- 0.75*preliminary MU (+/- 2.25 dB for bands < 3GHz).

- Start the Measurement Campaign after RAN4#109 immediately, based on the preliminary outcome that ≥ 3 labs can be aligned.

- The reference values of lab alignment will be derived by averaging the results from all 6 labs submitted in the 1st round. Then determine which labs are aligned; the potential failed labs can have the chance to retest the PAD(s). The reference values will not be changed, and the aligned labs will not be affected.

The data is in an attached file R4-2400193 3GPP FR1 MIMO OTA Lab Alignment Activity.xlsx

# 2	Reference

R4-2321085, " WF on [109][336] NR_MIMO_OTA_enh ", CAICT, RAN4#109