3GPP TSG-RAN WG4 Meeting #110	R4-2400251

Athens, Greece Feb 26 – Mar 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulations for Coverage Enhancement BS Demodulation

Agenda item:	8.19.2

Document for:	Discussion

Introduction

This document introduces to RAN4 the features associated with coverage enhancement under the Work Item “Further NR coverage enhancements” and the impact of the newly introduced features on Base Station demodulation within RAN4.

Specifically, within this paper we provide simulation results for the PRACH repetition feature introduced in RAN1 and its impact for BS demodulation performance requirements.

Discussion

PRACH Repetitions

TDL-A (99% Missed Detection)

The following section presents the simulation results of Nokia’s simulation campaign on PRACH repetitions and the impact for coverage enhancement in a ‘TDL-A 30-300 Low’ Channel.

Table 1 : 99% Missed Detection for Different PRACH formats with Coverage Enhancements (TDLA 30-300,1T2R)

| BW (MHz) | SCS (kHz) | Repetition Number | Burst Format | Freq Offset (Hz) | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  | 60 | 2 |  |  | -4.8 |
|  |  | 4 | B4 |  | -7.0 |
|  | 120 | 2 |  |  | -14.8 |
|  |  | 4 |  |  | -15.8 |
|  | 60 | 2 |  |  | -11.4 |
| 50 |  | 4 | C2 | 4000 | -13.2 |
|  | 120 | 2 |  |  | -12.3 |
|  |  | 4 |  |  | -12.9 |
|  | 60 | 2 |  |  | -11.1 |
|  |  | 4 | A2 |  | -12.8 |
|  | 120 | 2 |  |  | -12.1 |
|  |  | 4 |  |  | -13.6 |

AWGN (99% Missed Detection)

The following section presents the simulation results of Nokia’s simulation campaign on PRACH repetitions and the impact for coverage enhancement in an AWGN channel.

Table 2 : Table 1 : 99% Missed Detection for Different PRACH formats with Coverage Enhancements (AWGN,1T2R)

| BW (MHz) | SCS (kHz) | Repetition Number | Preamble Format | Freq Offset (Hz) | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  | 60 | 2 |  |  | -23.5 |
|  |  | 4 | B4 |  | -24.5 |
|  | 120 | 2 |  |  | -23.5 |
|  |  | 4 |  |  | -24.3 |
|  | 60 | 2 |  |  | -18.7 |
| 50 |  | 4 | C2 | 0 | -19.6 |
|  | 120 | 2 |  |  | -18.8 |
|  |  | 4 |  |  | -19.8 |
|  | 60 | 2 |  |  | -18.7 |
|  |  | 4 | A2 |  | -19.7 |
|  | 120 | 2 |  |  | -18.8 |
|  |  | 4 |  |  | -19.7 |

Conclusion

This contribution provides initial results on our simulation campaign for PRACH repetitions on Coverage Enhancement and its impact on BS demodulation. No observations were made:

References

- RP-221858 “Further NR coverage enhancements” RAN #96 Budapest, Hungary, June 2022