3GPP TSG-RAN WG4 Meeting #110	R4-2400253

Athens, Greece, Feb 26 – Mar 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on NR NTN SAN Demodulation

Agenda item:	8.18.8.1

Document for:	Discussion

Introduction

The introduction of NR NTN enhancements for Rel-18 has been outlined in the WID [1], with an aim to add functionality to increase features that were introduced in Rel 15, 16 and 17.

RAN4 Demodulation will specify the performance of these new enhancements.

The main objectives from the WID [1] are as follows:

- Specify RRM performance requirements and test cases [RAN4] - Specify UE demodulation and CSI reporting requirements [RAN4] - Specify satellite access node demodulation requirements [RAN4] - Specify satellite access node conformance tests [RAN4]

For the following enhancements

4.1.1 Coverage enhancement 4.1.2 NR-NTN deployment in above 10 GHz bands 4.1.3 Network verified UE location 4.1.4 NTN-TN and NTN-NTN mobility and service continuity enhancements

In this paper, we present Nokia’s view on the impact to SAN demodulation requirements.

Within RAN4#109 several open issues were presented in the way forward [3] which will be discussed in this contribution.

Issue 3-2-4: MCS - Proposals - Option 1: 2/16/20 - Option 2: MCS4 (QPSK, 308/1024) and MCS5 (QPSK, 99/1024, Table 3, for PUSCH repetition Type A case only) Issue 3-3-1: Test setup for SAN PUSCH demodulation requirements with CP-OFDM - Proposals - Option 1: Parameter; Value. Transform precoding; Disabled. HARQ; Maximum number of HARQ transmissions; 4. RV sequence; 0, 2, 3, 1. DM-RS; DM-RS configuration type; 1. DM-RS duration; single-symbol DM-RS. Additional DM-RS symbols; Option 1a (Ericsson, Huawei): Pos1 Option 1b (Samsung):Pos2. Number of DM-RS CDM group(s) without data; 2. Ratio of PUSCH EPRE to DM-RS EPRE; -3 dB. DM-RS port(s); {0} {0, 1} (if 2Tx is agreed). DM-RS sequence generation; NID0=0, nSCID =0. Time domain; PUSCH mapping type; B. resource; Start symbol index; 0. Allocation length; 10. Frequency domain; RB assignment; Full applicable test bandwidth. resource; Frequency hopping; Disabled. TPMI index for 2Tx two-layer spatial multiplexing transmission (If 2Tx is agreed); 0. Code block group based PUSCH transmission; Disabled. PT-RS; Frequency density (KPT-RS); Option 1a (Ericsson): 2, Disabled Option 1b (Samsung): 2 Option 1c (Huawei): Disabled. configuration; Time density (LPT-RS); Option 1a (Ericsson): 1, Disabled Option 1b (Samsung): 1 Option 1c (Huawei): Disabled. Issue 3-3-2: Test setup for SAN PUSCH demodulation requirements with DFT-s-OFDM - Proposals - Option 1: Parameter; Value. Transform precoding; Enabled. HARQ; Maximum number of HARQ transmissions; 4. RV sequence; 0, 2, 3, 1. DM-RS; DM-RS configuration type; 1. DM-RS duration; single-symbol DM-RS. Additional DM-RS symbols; Option 1a (Ericsson, Huawei): Pos1 Option 1b (Samsung):Pos2. Number of DM-RS CDM group(s) without data; 2. Ratio of PUSCH EPRE to DM-RS EPRE; -3 dB. DM-RS port(s); {0} {0, 1} (if 2Tx is agreed). DM-RS sequence generation; NID0=0, group hopping and sequence hopping are disabled. Time domain; PUSCH mapping type; B. resource; Start symbol index; 0. Allocation length; 10. Frequency domain; RB assignment; Option 1a (Ericsson, Huawei): Full applicable test bandwidth Option 1b (Samsung): 30 PRBs in the middle of the test bandwidth. resource; Frequency hopping; Disabled. TPMI index for 2Tx two-layer spatial multiplexing transmission (If 2Tx is agreed); 0. Code block group based PUSCH transmission; Disabled. PT-RS; Frequency density (KPT-RS); Option 1a (Ericsson): 2, Disabled Option 1b (Samsung): Not configured Option 1c (Huawei): Disabled. configuration; Time density (LPT-RS); Option 1a (Ericsson): 1, Disabled Option 1b (Samsung): Not configured Option 1c (Huawei): Disabled. Issue 3-3-3: Test setup for SAN PUSCH demodulation requirements with repetition Type A (If agreed to be introduced) - Proposals - Option 1: Parameter; Value. Transform precoding; Disabled. HARQ; Maximum number of HARQ transmissions; 4. RV sequence; 0, 3, 0, 3 [Note 1]. DM-RS; DM-RS configuration type; 1. DM-RS duration; single-symbol DM-RS. Additional DM-RS symbols; Pos1. Number of DM-RS CDM group(s) without data; 2. Ratio of PUSCH EPRE to DM-RS EPRE; -3 dB. DM-RS port(s); 0. DM-RS sequence generation; NID=0, nSCID =0. Time domain resource; PUSCH mapping type; B. Start symbol index; 0. Allocation length; 10. PUSCH aggregation factor; n2. Frequency domain resource; RB assignment; Full applicable test bandwidth. Frequency hopping; Disabled. Code block group based PUSCH transmission; Disabled. PT-RS configuration; Frequency density (KPT-RS); Disabled. Time density (LPT-RS); Disabled. NOTE 1: The effective RV sequence is {0,2,3,1} with slot aggregation. Issue 3-3-4: Test setup for SAN PUCCH demodulation performance requirements - Proposals - Option 1: Table 3: Test parameters of PUCCH format 0 Parameter; Test. Number of UCI information bits; 1. Number of PRBs; 1. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; N/A for 1 symbol Enabled for 2 symbols. First PRB after frequency hopping; The largest PRB index – (Number of PRBs - 1). Group and sequence hopping; neither. Hopping ID; 0. Initial cyclic shift; 0. First symbol; 13 for 1 symbol 12 for 2 symbols. Test metric; DTX to ACK probability ACK missed detection probability. Table 4: Test parameters of PUCCH format 1 Parameter; Test. Number of information bits; 2. Number of PRBs; 1. Number of symbols; 14. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; enabled. First PRB after frequency hopping; The largest PRB index – (nrofPRBs – 1). Group and sequence hopping; neither. Hopping ID; 0. Initial cyclic shift; 0. First symbol; 0. Index of orthogonal cover code (timeDomainOCC); 0. Test metric; DTX to ACK probability NACK to ACK probability ACK missed detection probability. Table 5 Test parameters of PUCCH format 2 Parameter; Value. Modulation order; QSPK. Starting RB location; 0. Intra-slot frequency hopping; N/A. Number of PRBs; 4. Number of symbols; 1. The number of UCI information bits; 4. First symbol; 13. DM-RS sequence generation; NID0=0. Test metric; DTX to ACK probability ACK missed detection probability. Table 6: Test parameters of PUCCH format 2 Parameter; Value. Modulation order; QSPK. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; enabled. Frist PRB after frequency hopping; The largest PRB index – (Number of PRBs – 1). Number of PRBs; 9. Number of symbols; 2. The number of UCI information bits; 22. First symbol; 12. DM-RS sequence generation; NID0=0. Test metric; BLER. Table 7: Test parameters of PUCCH format 3 Parameter; Test 1; Test 2. Modulation order; QPSK. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; enabled. First PRB after frequency hopping; The largest PRB index – (Number of PRBs – 1). Group and sequence hopping; neither. Hopping ID; 0. Number of PRBs; 1; 3. Number of symbols; 14; 4. The number of UCI information bits; 16; 16. First symbol; 0; 0. Test metric; BLER. Table 8: Test parameters of PUCCH format 4 Parameter; Value. Modulation order; QPSK. First PRB prior to frequency hopping; 0. Number of PRBs; 1. Intra-slot frequency hopping; enabled. First PRB after frequency hopping; The largest PRB index – (Number of PRBs – 1). Group and sequence hopping; neither. Hopping ID; 0. Number of symbols; 14. The number of UCI information bits; 22. First symbol; 0. Length of the orthogonal cover code; n2. Index of the orthogonal cover code; n0. Test metric; BLER. Issue 3-4-1: Antenna configuration - Proposals - Option 1: 1Tx1Rx and 1Tx2Rx - Option 1a: Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test Issue 3-4-2: Channel model - Proposals - Option 1: NTN-TDLC5-200 for DMRS bundling requirement - Option 2: NTN-TDLC5-200 for multi-slot PUCCH requirement Issue 3-4-3: Test setup for SAN PUSCH demodulation performance requirements for DMRS bundling (If agreed to be introduced) - Proposals - Option 1: Parameter; Value. Transform precoding; Disabled. HARQ; Maximum number of HARQ transmissions; 4. RV sequence [Note 1]; Option 1a (Ericsson, Huawei) 0, 0, 0, 0 for FDD Option 1b (Samsung) : 0, 3, 0, 3 for FDD. DM-RS; DM-RS configuration type; 1. DM-RS duration; single-symbol DM-RS. Additional DM-RS position; Option 1a (Ericsson): pos0, pos1 Option 1b (Samsung, Huawei): pos1. Number of DM-RS CDM group(s) without data; 2. Ratio of PUSCH EPRE to DM-RS EPRE; -3 dB. DM-RS port; {0}. DM-RS sequence generation; NID0=0, nSCID =0. Time domain resource assignment; PUSCH mapping type; Option 1a (Ericsson, Samsung): A Option 1b (Huawei): A, B. Start symbol; 0. Allocation length; 14. PUSCH aggregation factor; Option 1a (Ericsson): n4 for FDD Option 1b (Samsung): [n2] or [n8] for FDD Option 1c (Huawei): n8 for FDD. pusch-TimeDomainWindowLength; Option 1a (Ericsson): 4 slots for FDD Option 1b (Samsung): [2] or [4] slots for FDD Option 1c (Huawei): 8 slots for FDD. Frequency domain resource assignment; RB assignment; Option 1 (Ericsson, Samsung, Huawei):Full applicable test bandwidth Option 2 (Samsung): 6RBs for both 15KHz and 30KHz. Frequency hopping; Disabled. Code block group based PUSCH transmission; Disabled. Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation.. Issue 3-4-4: Test setup for SAN multi-slot PUCCH format 1 demodulation performance requirements under LOS channel (If agreed to be introduced) - Proposals - Option 1: Parameter; Test. Number of information bits; 2. Number of PRBs; 1. Number of symbols; 14. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; disabled. Inter-slot frequency hopping; enabled. First PRB after frequency hopping; The largest PRB index – (nrofPRBs – 1). Group and sequence hopping; neither. Hopping ID; 0. Initial cyclic shift; 0. First symbol; 0. Index of orthogonal cover code (timeDomainOCC); 0. Number of slots for PUCCH repetition; 2.

Discussion

Issue 3-2-4 MCS

Both of the proposals provided during RAN4#109 are feasible, however option 1 provides a wider range of operating SNR points. Depending upon the SNR operating point, discussion can be had at RAN4#110 about which is taken forward for performance requirements.

MCS 2/16/20 to be taken forward for performance requirements, with downselection based on SNR operating point.

Issue 3-3-1 Test setup for SAN PUSCH demodulation requirements with CP-OFDM

Additional DM-RS Symbol

Due to fact that only 200 Hz of doppler post-compensation, an additional DMRS in Pos2 would be redundant; as such Nokia believes that only 1 additional DMRS position is required.

RAN4 shall use a single additional DMRS in pos1 for NTN SAN demodulation performance requirements.

DM-RS Port

As 2 Tx has not been agreed within previous meetings on this work item, Nokia believes that only DMRS port 0 requires to be configured.

RAN4 shall use DMRS port 0 for NTN SAN demodulation performance requirements.

RB Assignment

Nokia believes that the full RB assignment should be utilised for requirements derivation.

RAN4 shall use full RB assignment for NTN SAN demodulation performance requirements.

PT-RS

Due to no company wishing to define requirements with PT-RS and the effect of pre-compensation negating the need, Nokia feels no strong need to define requirements with PT-RS.

RAN4 shall not use PT-RS for NTN SAN demodulation performance requirements.

Issue 3-3-2 Test setup for SAN PUSCH demodulation requirements with DFT-s-OFDM

Additional DM-RS

Due to fact that only 200 Hz of doppler post-compensation, an additional DMRS in Pos2 would be redundant; as such Nokia believes that only 1 additional DMRS position is required.

RAN4 shall use a single additional DMRS in pos1 for NTN SAN DFT-s-OFDM demodulation performance requirements.

DM-RS Ports

As 2 Tx has not been agreed within previous meetings on this work item, Nokia believes that only DMRS port 0 requires to be configured.

RAN4 shall use DMRS port 0 for NTN SAN demodulation performance requirements for DFT-s-OFDM.

RB Assignment

Nokia believes that the full RB assignment should be utilised for requirements derivation for DFT-s-OFDM

RAN4 shall use full RB assignment for NTN SAN DFT-s-OFDM demodulation performance requirements.

PT-RS

Due to no company wishing to define requirements with PT-RS and the effect of pre-compensation negating the need, Nokia feels no strong need to define requirements with PT-RS.

RAN4 shall not use PT-RS for NTN SAN DFT-s-OFDM demodulation performance with requirements.

Issue 3-3-3: Test setup for SAN PUSCH demodulation requirements with repetition Type A

For this test setup, Nokia does not see any concerns with the Option proposed as part of the Way Forward from RAN4#109.

RAN4 shall use the test setup identified as Option 1 in the way forward from RAN4#109 for PUSCH demodulation (repetition type A)

Issue 3-3-4: Test setup for SAN PUCCH demodulation performance requirements

For this test setup, Nokia does not see any concerns with the Option proposed as part of the Way Forward from RAN4#109.

RAN4 shall use the test setup identified as Option 1 in the way forward from RAN4#109 for PUCCH demodulation.

Issue 3-4-1 Antenna Configuration

Nokia believes that due to time of Rel-18, RAN4 should define requirements with a focus on 1Tx1Rx with future study on 1Tx2Rx. Further to this, we believe that for the minimum deployment case, only 1 element will be available for transmit and receive.

RAN4 shall use 1Tx1Rx for Antenna Configuration for SAN demodulation.

Issue 3-4-2 Channel Model

Following RAN4#109 NTN-TDLC5-200 is proposed for both DMRS bundling and multi-slot PUCCH, thus Nokia believes that NTN-TDLC5-200 should be used for all SAN demodulation cases. Whereby 200 Hz represents 0.01 ppm of  a 20 GHz carrier frequency

200Hz doppler is 0.01 ppm of 20GHz, thus representing the residual doppler after pre-compensation.

NTN-TDLC5-200 shall be used for all SAN demodulation performance requirements.

Issue 3-4-3: Test setup for SAN PUSCH demodulation performance requirements for DMRS bundling (If agreed to be introduced)

Additional DM-RS

Due to fact that only 200 Hz of doppler post-compensation, an additional DMRS in Pos2 would be redundant; as such Nokia believes that only 1 additional DMRS position is required.

RAN4 shall use a single additional DMRS in pos1 for NTN SAN DMRS bundling demodulation performance requirements.

Issue 3-4-4: Test setup for SAN multi-slot PUCCH format 1 demodulation performance requirements under LOS channel (If agreed to be introduced)

For this test setup, Nokia does not see any concerns with the Option proposed as part of the Way Forward from RAN4#109.

RAN4 shall use the test setup identified as Option 1 in the way forward from RAN4#109 for PUCCH format 1 demodulation

Conclusion

This contribution provides our views on SAN demodulation requirements for Rel-18 NR NTN. Specifically, a series of observation and proposals are made as follows:

Proposal 1: MCS 2/16/20 to be taken forward for performance requirements, with downselection based on SNR operating point.

Proposal 2: RAN4 shall use a single additional DMRS in pos1 for NTN SAN demodulation performance requirements.

Proposal 3: RAN4 shall use DMRS port 0 for NTN SAN demodulation performance requirements.

Proposal 4: RAN4 shall use full RB assignment for NTN SAN demodulation performance requirements.

Proposal 5: RAN4 shall not use PT-RS for NTN SAN demodulation performance requirements.

Proposal 6: RAN4 shall use a single additional DMRS in pos1 for NTN SAN DFT-s-OFDM demodulation performance requirements.

Proposal 7: RAN4 shall use DMRS port 0 for NTN SAN demodulation performance requirements for DFT-s-OFDM.

Proposal 8: RAN4 shall use full RB assignment for NTN SAN DFT-s-OFDM demodulation performance requirements.

Proposal 9: RAN4 shall not use PT-RS for NTN SAN DFT-s-OFDM demodulation performance with requirements.

Proposal 10: RAN4 shall use the test setup identified as Option 1 in the way forward from RAN4#109 for PUSCH demodulation (repetition type A)

Proposal 11: RAN4 shall use the test setup identified as Option 1 in the way forward from RAN4#109 for PUCCH demodulation.

Proposal 12: RAN4 shall use 1Tx1Rx for Antenna Configuration for SAN demodulation.

Observation 1: 200Hz doppler is 0.01 ppm of 20GHz, thus representing the residual doppler after pre-compensation.

Proposal 13: NTN-TDLC5-200 shall be used for all SAN demodulation performance requirements.

Proposal 14: RAN4 shall use a single additional DMRS in pos1 for NTN SAN DMRS bundling demodulation performance requirements.

Proposal 15: RAN4 shall use the test setup identified as Option 1 in the way forward from RAN4#109 for PUCCH format 1 demodulation

References

- RP-230809 “Revised WID on NR NTN (Non-Terrestrial Networks) enhancements”

- TS 38.108 v17.4.0. Technical Specification Group Radio Access Network; NR; Satellite Access Node radio transmission and reception. June 2023

- R4-2321187 “WF on NR_NTN_enh_SAN_UE_demod”, Moderator (QualcommHuawei, HiSilicon), RAN4#109, Chicago, November, 2023