3GPP TSG-RAN4 #110	R4-2400307

Athens, Greece, 26th Feb – 1st Mar 2024

Title:	[NR_FR2_multiRX_DL-Perf] Tx EVM assumption for PMI reporting

Source:			Anritsu Corporation

Agenda Item:			8.3.4.3

Document for:			Approval

# 1.	Introduction

At the RAN4 #109 meeting in November, there was a proposal to reduce an assumption of Tx EVM (DL signal quality from a TE) from 6% to 3% for FR2 PMI reporting under the agenda of FR2 multi-Rx chain DL reception [1]. And the outcome was captured as FFS in the session report. Online minutes are excerpted below for reference.

TxEVM Qualcomm: We had previous agreement that if we as common assumption consider EVM, we will assume 6% for 64QAM. Nokia: If a company wants to consider TxEVM in their impairment results, they are free to consider 6% Huawei: We should consider TxEVM, otherwise, the requirement will be tightened. This is how other requirements are defined. Nokia: Our compromise proposal is that companies can decide on their own. We believe the correct number is 3%. R&S: This was discussed for non-colocated for FR1. This is FR2. A lower TxEVM will result in a lower testable SNR in the TE. Nokia: RAN4 should set the requirement, and RAN5 should set the uncertainty. R4-2321140 WF on [109][318] NR_FR2_multiRX_DL_Demod Type: other For: Approval Source: Qualcomm Decision: Approved. Chair: Suggest to indicate TxEVM as FFS

In this contribution we share our views based on a study of a level diagram when the PMI reporting test is conducted. In short, there is a concern that the 3% Tx EVM assumption may cause a testability issue depending on a frequency to carry out the tests due to a low Noc level definition.

# 2.	Discussion

  - Consideration on an influence of Tx EVM reduction with test equipment

Figure 2.1-1 is shown to depict an image of the level diagram for the FR2 PMI reporting test, representing the case of band n260 (37 to 40 GHz) as an example. When conducting the PMI reporting test, we first try to find an SNR level where a 90% throughput can be obtained with a random PMI configuration, for example starting from a +15 dB SNR compared to Noc + Noc offset level, which is defined in TS 38.521-4 cl. 4.5.3 [2].

Now, from the test equipment viewpoint, decreasing the Tx EVM from 6% to 3% means that we need to increase a back-off from a 1 dB compression point (P1dB) of the mmWave converter for the DL signal transmission. The increase of the back-off depends on each test system, but it can be expected approximately 5 to 15 dB also depending on a frequency.

Observation 1:  From the test equipment viewpoint, decreasing the Tx EVM from 6% to 3% means that we need to increase a back-off from a 1 dB compression point (P1dB) of the mmWave converter, ranging approximately from 5 to 15 dB depending on the frequency around FR2.

Figure 2.1-1: Image of a level diagram for the FR2 PMI reporting test with band n260.

2.2 Anticipated impact to the performance specifications

Next, we consider an anticipated impact to the performance specifications when applying the 3% Tx EVM. As mentioned above, to achieve the 3% Tx EVM with the test equipment, we need to lower the output power from the test equipment. When we consider the test procedure of the PMI reporting, it implies that we need to lower the PDSCH level from the legacy requirement as well. Now as the PDSCH level is decided based on the Noc level at the bottom in the figure 2.1-1 and SNR of PDSCH, Noc level will also be lowered to keep the same SNR range for the test. However, since the current test equipment for the UE (signalling tester) is designed based on the assumption of the 6% Tx EVM, and because the FR2 OTA test includes a huge OTA path loss, a noise floor (NF) level of the test equipment is very close to the legacy Noc level. Thus, there is a concern especially at higher frequency bands that the Noc level may fall below the NF level of the test equipment.

Observation 2: To achieve the 3% Tx EVM, we need to lower the PDSCH level from the legacy requirement, which also implies that Noc level will also be lowered.

Observation 3: There is a concern that Noc level may fall below the NF level of the test equipment especially at higher frequency bands.

Based on a study of our test equipment, we identified that even band n260 may have the testability issue even without taking into account of a measurement uncertainty. Therefore, we propose to keep the legacy 6% Tx EVM assumption (DL signal quality from TE) also for FR2.

Proposal 1: Keep the legacy 6% Tx EVM assumption (DL signal quality from TE) also for FR2.

- Conclusion

In this contribution we shared our views against the proposal of 3% Tx EVM assumption based on the study of a level diagram when the PMI reporting test is conducted.

Observation 1:  From the test equipment viewpoint, decreasing the Tx EVM from 6% to 3% means that we need to increase a back-off from a 1 dB compression point (P1dB) of the mmWave converter, ranging approximately from 5 to 15 dB depending on the frequency around FR2.

Observation 2: To achieve the 3% Tx EVM, we need to lower the PDSCH level from the legacy requirement, which also implies that Noc level will also be lowered.

Observation 3: There is a concern that Noc level may fall below the NF level of the test equipment especially at higher frequency bands.

Proposal 1: Keep the legacy 6% Tx EVM assumption (DL signal quality from TE) also for FR2.

# 4.	References

[1] R4-2400001, “RAN4#109 Meeting Report”, ETSI MCC, RAN4 #110, Athens, Greece

[2] TS 38.521-4, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance requirements”, v18.1.0, 2023-12