# 3GPP TSG-RAN WG4 Meeting #110                                                             R4-2400326

# Athens, GR, 26th February – 1st March 2024

# Title:	TP for TR38.898 HP-ENDC 8-42_n77

# Source:	Softbank

# Agenda Item:	7.16.2

# Document for:	Approval

- Introduction

This contribution is a text proposal for TR38.898[5] to include the following HP-ENDC combinations as requested in RAN4#108.

- DC_8A-42A_n77A

- Reference

[3] 3GPP TS 38.101-3 "User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (Release 18)", V18.4.0.

[5] 3GPP TR38.898 “High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL) (Release 18)”, V0.7.0

[8] 3GPP TR37.716-21-11 “Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) (Release 16)”, V16.1.0

- Text Proposal

- -- Start of TP –

-- Unaffected parts omitted –

### 5.x	DC_8-42_n77

#### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A-42A_n77A14,15,16 DC_8A-42C_n77A15,16 | DC_8A_n77A14 |
| NOTE1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |

#### 5.x.2	Maximum output power for DC

The maximum output power requirement for PC2 UL DC_8_n77 is defined as Table 6.2B.1.3-1 in TS38.101-3[3]. So, this section can be omitted.

#### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of PC3 DC_8-42_n77 defined to 5.1.60.3 in TR37.716-21-11[8], there is no MSD issue for this configuration based on the following conditions.

-	 LTE Band 42 and NR Band n77 are synchronized.

-	 Simultaneous transmission of LTE Band 42 and NR Band n77 are not supported.

#### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

-- Unaffected parts omitted –

-- End of TP –