# 3GPP TSG-RAN WG4 Meeting #110                                                             R4-2400331

# Athens, GR, 26th February - 1st March 2024

# Title:	TP for TR38.899 to include new HP-NRCA combinations for FR1

# Source:	Softbank Corp.

# Agenda Item:	7.18.2

# Document for:	Approval

- Introduction

This contribution is a text proposal for TR 38.899[1] to include the following NRCA combinations as requested in RAN4#108.

- CA_n28-n41 (PC1.5 2DL and single UL)

- Reference

[1] TR38.899 “High power UE for FR1 NR inter-band CA/DC or NR SUL band combination with y (1<y<=6) bands DL and x (x=1, 2) bands UL and power class m (m<3) and high power on TDD band(s) (Release 18)” V0.7.0

- Text Proposal

- -- Start of TP –

-- Unaffected parts omitted –

## 5.x	CA_n28-n41

### 5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n41A | n418,9 CA_n28A-n41A8, 13 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | 5, 10, 15, 20, 30 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |

### 5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28-n41 | CA power class | Carrier n28 power class | Carrier n41 power class |
| --- | --- | --- | --- | --- |
| CA_n28-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.x.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.x.3.1	Power class 2 case a, b

Based on calculation of previous release, no IMD of dual uplink falls into n28 DL, the MSD exception is not needed.

#### 5.x.3.3	Power class 2 for single uplink n41

Based on calculation of previous release,

- No harmonic mixing of n41 fall into Rx frequencies of n28.

- No cross band isolation issue.

Therefore, no MSD exception needs to be defined.

#### 5.x.3.4	Power class 1.5 for single uplink n41

Based on above,

- No harmonic mixing of n41 fall into Rx frequencies of n28.

- No cross band isolation issue.

Therefore, no MSD exception needs to be defined.

### 5.x.4	∆TIB and ∆RIB values

There is no change by compairing to the values for PC3 CA.

-- Unaffected parts omitted --

-- End of TP --