3GPP TSG-RAN WG4 Meeting # 110		R4-2400345

Athens, Greece, 26th Feb 2024 - 1st Mar 2024

Agenda Item:	12.1.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	On R2-2211023 on ue-PowerClassPerBandPerBC-r17

Document for:	Approval

# Introduction

This paper addresses following two questions included in RAN2 LS of [1].

- Whether R4 16-8 is applicable to only inter-band CA?

- What is the interaction between R4 16-8 and the existing power class capabilities (i.e. ue-PowerClass/ue-PowerClass-v1610/ue-PowerClass-1700, powerClassNRPart-r16 (if R4 16-8 is also applicable to the cases other than inter-band CA) and powerClass/powerClass-v1610) ?

# Discussion

## Answer for the question 1

In short, our view is very similar to draft LSs in [2, 3]. Since a similar issue for MR-DC is covered by powerClassNRPart-r16, ue-PowerClassPerBandPerBC-r17 doesn’t need to cover MR-DC assuming that there will be no MR-DC with three bands UL (one LTE band and two different NR bands).

Proposal 1: Possible answer for the 1st question

- ue-PowerClassPerBandPerBC-r17 is applicable to only NR inter-band UL CA with two different operating bands where each uplink band contains single CC or multiple CCs (intra-band contiguous UL CA). ue-PowerClassPerBandPerBC-r17 is not applicable to MR-DC.

## Answer for the question 2

### Inter band UL CA without intra band contiguous UL CA part

If a UE cannot attain the power indicated by ue-PowerClass for a band(s) in NR inter band UL CA, the UE has an option to indicate ue-PowerClassPerBandPerBC-r17 in the corresponding band(s) in the NR UL inter band CA. Process to determine the highest attainable power for a band in NR inter-band UL CA is shown in Figure 1.

Observation 1: Process to determine the highest attainable power for a band in NR inter-band UL CA is as follows.

Figure 1: Process to determine the highest attainable power for a band in NR inter-band UL CA without intra band contiguous UL CA part

### Inter band UL CA with intra band contiguous UL CA part

We elaborated the reason why ue-PowerClass should be referred to as the highest attainable power for UL intra band contiguous CA part within UL inter band CA, e.g., n41C part in UL CA_n1A-n41C in [4, 5]. As a result, RAN4#109 agreed the CRs of [6, 7].

Observation 2: RAN4#109 agreed to use ue-PowerClass to refer to the highest attainable power for UL intra band contiguous CA part within UL inter band CA in CRs of [R4-2318030, R4-2318031].

Observation 3: Observation 2 means that even if a UE supporting e.g., UL CA_n1A-n41C, indicates powerClass for intra band contiguous UL CA, e.g., CA_n41C, the highest attainable power for n41C part in CA_n1A-n41C is not determined by powerClass for CA_n41C, but rather ue-PowerClass for n41, if ue-PowerClassPerBandPerBC-r17 in n41 is not indicated.

Observation 4: Observation 3 means that powerClass for intra band contiguous UL CA like CA_n41C is irrelevant to the determination of the power for intra band contiguous UL CA part in NR inter-band UL CA.

Observation 5: From Observation 2 - 4, the same process in Figure 1 applies to inter-band UL CA with intra band contiguous UL CA.

With the consideration of the observations 1-5, we propose the content of the LS as follows.

Proposal 2: Possible answer for the 2nd question

- If ue-PowerClassPerBandPerBC-r17 is indicated for a band(s) in an NR inter-band UL CA and

  - if the power class in ue-PowerClassPerBandPerBC-r17 is less than or equal to powerClass for the NR inter-band UL CA, the power class in ue-PowerClassPerBandPerBC-r17 determines the highest power over the carrier(s) of the band(s) in the NR inter-band UL CA,

  - otherwise, the powerClass determins the highest power in the band(s)

- In other words, the above corresponds to taking min {power class in ue-PowerClassPerBandPerBC-r17, powerClass for the NR inter-band UL CA} for a band(s), in case ue-PowerClassPerBandPerBC-r17 for the band(s) is indicated.

- It is noted that, in a), the term carrier”s” corresponds to the case where the NR inter-band UL CA includes intra-band contiguous UL CA in it.

# Conclusion

With the above, we obtained following observations and proposal.

Observation 1: Process to determine the highest attainable power for a band in NR inter-band UL CA is as follows.

Figure 1: Process to determine the highest attainable power for a band in NR inter-band UL CA without intra band contiguous UL CA part

Observation 2: RAN4#109 agreed to use ue-PowerClass to refer to the highest attainable power for UL intra band contiguous CA part within UL inter band CA in CRs of [R4-2318030, R4-2318031].

Observation 3: Observation 2 means that even if a UE supporting e.g., UL CA_n1A-n41C, indicates powerClass for intra band contiguous UL CA, e.g., CA_n41C, the highest attainable power for n41C part in CA_n1A-n41C is not determined by powerClass for CA_n41C, but rather ue-PowerClass for n41, if ue-PowerClassPerBandPerBC-r17 in n41 is not indicated.

Observation 4: Observation 3 means that powerClass for intra band contiguous UL CA like CA_n41C is irrelevant to the determination of the power for intra band contiguous UL CA part in NR inter-band UL CA.

Observation 5: From Observation 2 - 4, the same process in Figure 1 applies to inter-band UL CA with intra band contiguous UL CA.

With the consideration of the observations 1-5, we propose the content of the LS as follows.

Proposal 2: Possible answer for the 2nd question

- If ue-PowerClassPerBandPerBC-r17 is indicated for a band(s) in an NR inter-band UL CA and

  - if the power class in ue-PowerClassPerBandPerBC-r17 is less than or equal to powerClass for the NR inter-band UL CA, the power class in ue-PowerClassPerBandPerBC-r17 determines the highest power over the carrier(s) of the band(s) in the NR inter-band UL CA,

  - otherwise, the powerClass determins the highest power in the band(s)

- In other words, the above corresponds to taking min {power class in ue-PowerClassPerBandPerBC-r17, powerClass for the NR inter-band UL CA} for a band(s), in case ue-PowerClassPerBandPerBC-r17 for the band(s) is indicated.

- It is noted that, in a), the term carrier”s” corresponds to the case where the NR inter-band UL CA includes intra-band contiguous UL CA in it.

# Reference

- R2-2211023, LS on the ue-PowerClassPerBandPerBC-r17(R4 16-8), RAN2, TSG-RAN WG2 #119bis

- R4-2311917, [DRAFT] Reply LS on the ue-PowerClassPerBandPerBC-r17 (R4 16-8), Samsung, TSG-RAN WG4 #108

- R4-2312161, [DRAFT] Reply LS on the ue-PowerClassPerBandPerBC-r17 (R4 16-8), Ericsson, TSG-RAN WG4 #108

- R4-2318038, On R2-2211023 on ue-PowerClassPerBandPerBC-r17, Nokia, Nokia Shanghai Bell, TSG-RAN WG4 #109

- R4-2318038, Power Class in UL intra band CA within UL inter band CA, Nokia, Nokia Shanghai Bell, TSG-RAN WG4 #109

- R4-2318030, Introduction of higherPowerLimit-r17 into NR CA of PC3+PC5 including UL Intra band CA, Nokia, Nokia Shanghai Bell, TSG-RAN WG4 #109

- R4-2318031, Introduction of higherPowerLimit-r17 into NR CA of PC3+PC5 including UL Intra band CA, Nokia, Nokia Shanghai Bell, TSG-RAN WG4 #109