3GPP TSG-RAN WG4 Meeting #110                                          R4-2400346

Athens, GR, 26 Feb – 01 Mar, 2024

Title:	Draft LS reply to R2-2211023 on ue-PowerClassPerBandPerBC-r17

Response to:	R2-2211023

Release:	Rel-17

Work Item:	NR_RF_FR1_enh

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact person:

#### Name:	Hiromasa Umeda

#### E-mail Address:	hiromasa.umeda@nokia.com

Attachments: None

# 1		Overall description

RAN4 further discussed two questions in the LS of R2-2211023 and would like to share following answers for the respective questions.

Q1. Whether R4 16-8 is applicable to only inter-band CA?

A1. ue-PowerClassPerBandPerBC-r17 is applicable to only NR inter-band UL CA with two different        operating bands where each uplink band contains single CC or multiple CCs (intra-band contiguous UL CA). It’s also noted that ue-PowerClassPerBandPerBC-r17 is not applicable to MR-DC.

Q2. What is the interaction between R4 16-8 and the existing power class capabilities (i.e. ue-PowerClass/ue-PowerClass-v1610/ue-PowerClass-1700, powerClassNRPart-r16 (if R4 16-8 is also applicable to the cases other than inter-band CA) and powerClass/powerClass-v1610) ?

A2: If ue-PowerClassPerBandPerBC-r17 is indicated for a band(s) in an NR inter-band UL CA and

- if the power class in ue-PowerClassPerBandPerBC-r17 is less than or equal to powerClass for the NR inter-band UL CA, the power class in ue-PowerClassPerBandPerBC-r17 determines the highest power over the carrier(s) of the band(s) in the NR inter-band UL CA,

- otherwise, the powerClass determines the highest power in the band(s)

In other words, the above corresponds to taking min {power class in ue-PowerClassPerBandPerBC-r17, powerClass for the NR inter-band UL CA} for a band(s), in case ue-PowerClassPerBandPerBC-r17 for the band(s) is indicated. It is noted that, in a), the term carrier”s” corresponds to the case where the NR inter-band UL CA includes intra-band contiguous UL CA in it..

# 2		Actions

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to take into account the two answers for the future work.

# 3Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #110bis	     April 15 –19, 2024	         Changsha, China

TSG RAN WG4 Meeting #111	   	  May 20 – 24, 2024	         Fukuoka, Japan