3GPP TSG-RAN WG4 Meeting #110 						                     R4-2400385

Athens, Greece, February 26 – March 1, 2024

Agenda item:	8.6.1

Source:	Apple

Title:	Discussion on RAN2 LS on BWP operation without restriction

Document for:	Discussion

# Introduction

In RAN2#114, an LS was approved to RAN4 [1]. RAN4 is asked to provide feedback on RAN2’s question regarding option C for PSCell. In this contribution, we will provide our view on the question and some draft proposal to reply to RAN2.

# Discussion

Question from RAN2 is duplicated here for information:

Besides, regarding RAN4 agreement below: - The following scenario is supported from RAN4 requirement perspective: - For UE supporting option C and configured with CA, NCD-SSB based L1 and L3 intra-frequency measurement requirements are only applicable for the PCell.. RAN2 would like to check whether it is also applicable for the PSCell.

RAN4 related agreements were made in RAN4#108bis [3]:

#### Issue 4-6: CA requirements for UE supporting option C

- The following scenario is supported from RAN4 requirement perspective:

  - NCD-SSB + CA, NCD-SSB only in PCell

- Inform RAN1/2 the above agreement

It can be observed that RAN4 only discussed CA during that meeting. DC was not discussed. However, we believe the logic is still the same. It is not preferred to further extend the scope, especially when core part has been concluded. Technical aspect, PSCell is similar with SCell. PSCell can be added and released. UE would search for CD-SSB for PSCell addition. Note that PSCell activation/deactivation is also supported now. RRM measurement on deactivated PSCell is still based on CD-SSB in current design. Therefore, we propose to only keep PCell in the scope and not to have further extension.

Proposal 1: reply RAN2 that in R18 RAN4 requirements apply only when NCD-SSB is configured only on in PCell.

Proposal 2: clarify in RAN4 spec that option C related requirements apply when NCD-SSB is configured only on in PCell.

# Conclusion

In this contribution, we provide discussion on RAN2 LS on BWP operation without restriction. After discussion, the following conclusions are provided:

Proposal 1: reply RAN2 that in R18 RAN4 requirements apply only when NCD-SSB is configured only on in PCell.

Proposal 2: clarify in RAN4 spec that option C related requirements apply when NCD-SSB is configured only on in PCell.

# References

- R2-2313951, Reply LS to RAN4 on BWP operation without restriction, vivo

- R4-2321607, WF on BWP without restriction, vivo, Vodafone

- R4-2317337, WF on support for bandwidth part operation without restriction, vivo

- R4-2314339, WF on support for bandwidth part operation without restriction, vivo

- RP-232677, Further revised WID: Complete the specification support for BandWidth Part operation without restriction in NR, Vodafone, vivo