3GPP TSG-RAN WG4 Meeting #110						                 					     R4-2400388

Athens, Greece, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for PDCCH ordered based RACH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-01-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN1 PDCCH ordered based RACH delay requirements, TSSB, TBWPswitchDelay and ∆RF/BB preparation are referred to TS38.133. If a random access procedure is initiated by a PDCCH order, the UE, if requested by higher layers, transmits a PRACH in the selected PRACH occasion, as described in [11, TS 38.321], for which a time between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission is larger than or equal to NT,2+TBWPswitchDelay+∆Delay+Tswitch+TSSB+∆RF/BB preparation msec, where - NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission - TBWPswitchDelay=0 if the active UL BWP does not change, or if a cell indicator field in the PDCCH order indicates a non-serving cell [5, TS 38.212], and TBWPswitchDelay is defined in [10, TS 38.133] otherwise - ∆Delay=0.5 msec for FR1 and ∆Delay=0.25 msec for FR2 - Tswitch is a switching gap duration as defined in [6, TS 38.214] - TSSB=0 if a cell indicator field in the PDCCH order indicates a serving cell or if cell indicator field is not present, and TSSB is defined in [10, TS 38.133] otherwise - ∆RF/BB preparation=0 if a cell indicator field in the PDCCH order indicates a serving cell or if cell indicator field is not present, and ∆RF/BB preparation is defined in [10, TS 38.133] otherwise. However, they haven’t been specified in TS38.133 yet. RAN4 has already agreed the value of Tssb in RAN4#109: Issue 1-2-1-2: The value of additional time for DL synchronization when needed in the delay requirements for PDCCH ordered RACH before cell switch command <Agreement>: - For FR2, one Tssb delay is always assumed before UE transmit PDCCH-ordered RACH. - For FR1, when TCI state associated the PDCCH-order RACH has not been activated, - one Tssb delay is always assumed before UE transmit PDCCH-ordered RACH. - For FR1, when TCI state associated the PDCCH-order RACH is activated, - If L1-RSRP measurement delay is less than or equal to 160ms, Tssb is not needed. UE is required to meet the UL Tx timing accuracy requirements - If L1-RSRP measurement delay is more than 160ms, UE is allowed to transmit PDCCH-ordered RACH with or without one Tssb delay. No UE UL Tx timing accuracy requirement will be defined.. Even though RAN4 agreed not to use the term TBWPswitchDelay, instead it is replaced with ∆RF/BB preparation, TBWPswitchDelay is still used in RAN1 spec. To avoid further revision of RAN1 spec, we suggest clarify TBWPswitchDelay in RAN4 spec to cover the case wherein RACH BW is outside active BWP but within other configured UL BWP. ∆RF/BB preparation can be used to only cover the case wherein RACH BW is out of all the configured BWP. |
| Summary of change: |  | - Introudce TSSB and ∆RF/BB preparation in TS38.133 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDCCH ordered RACH delay requirements would still be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 8.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

## 8.X	PDCCH ordered based RACH delay for LTM candidate cell

### 8.X.1	Introduction

The requirements in this clause apply for a UE capable of [FG 45-5: RACH-based early TA acquisition] configured to trigger RACH procedure toward target LTM candidate cell before receiving cell switch command MAC-CE.

### 8.x.2	PDCCH ordered based RACH delay

UE shall complete the RACH transmission within the delay defined in TS38.213 [3] clause 8.1, in which TSSB, TBWPswitchDelay and ∆RF/BB preparation  are defined as the following:

-	TSSB:

-	for target LTM candidate cell is in FR1, TSSB = 0 if the following conditions are met. Otherwise, TSSB equals to one SSB periodicity in the LTM candicate cell configuration.

-	UE has received MAC-CE to activate TCI of target candicate cell and the time between receiving the MAC-CE activating the target TCI state and the PDCCH order for RACH is at least [THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length, where THARQ, TOk, Tfirst-SSB and TSSB-proc are as stated in section 8.15.3], and

-	L1-RSRP measurement period as defined in clause 9.14 and 9.15 respectivefully for intra-frequency and inter-frequency is less than or equal to 160ms.

-	for target LTM candidate cell is in FR2, TSSB equals to one SSB periodicity in the LTM candicate cell configuration.

-	TBWPswitchDelay: if PRACH bandwidth is outside active UL BWP but within one of configured UL BWPs of any active serving cell, TBWPswitchDelay is the DCI based BWP switching delay specified in clause 8.6. Otherwise, TBWPswitchDelay = 0.

-	∆RF/BB preparation: if PRACH bandwidth is within one of configured UL BWPs of any active serving cell, ∆RF/BB preparation = 0. Otherwise, ∆RF/BB preparation equals to [UE capability of [1ms, 3ms, 5ms, 10ms]].

End of Change