3GPP TSG-RAN4 Meeting  #110	R4-2400427

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0691 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on FR2 multi-RX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-Core |  |  |  |  |  | Date: |  |  | 2024-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To further clarify the recently introduced spherical coverage requirement for simultaneous reception in multiple directions to avoid misinterpretation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Main changes: - Clause 5.5K is voided as there is no applicable configuration in 5.5 - In Clause 7.3K.3, refine the wording that “The UE is only required to fulfill the requirement at any one of AoA separations declared from table 7.3K.3-1.” - In Clause L.2, add the clarifcation “In the test, n is set to 2.” per previous RAN4 agreement. There are other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is confusion in interpreting the requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3, 5.5K, 7.3K, Annex L |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change #1>

## 4.3	Specification suffix information

Unless stated otherwise the following suffixes are used for indicating at 2nd level clause, shown in Table 4.3-1.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplement Uplink (SUL) |
| D | UL MIMO |
| K | Simultaneous reception or transmission in multiple directions |
| NOTE 1: Suffix D in this specification represents either polarized UL MIMO or spatial UL MIMO. RF requirements are same. If UE supports both kinds of UL MIMO, then RF requirements only need to be verified under either polarized or spatial UL MIMO. NOTE 2: Suffix K applies to simultaneous reception or transmission with different TCI states and different QCL-TypeD reference signals across the TCI states |  |

# <End of Change #1>

# <Start of Change #2>

## 5.5K	Void

# <End of Change #2>

# <Start of Change #3>

## 7.3K	Spherical coverage requirement for simultaneous reception in multiple directions

### 7.3K.0	General

For this release, the requirement applies only to FR2-1 UEs that support the following set of capabilities:

1.	simultaneousReceptionDiffTypeD-r16

2.	At least one of:

a.	singleDCI-SDM-scheme-r16 or

b.	multiDCI-MultiTRP-r16 and either of:

i.	overlapPDSCHsFullyFreqTime-r16.

ii.	overlapPDSCHsInTimePartiallyFreq-r16

The requirement applies for simultaneous reception of rank 2 PDSCH, where each layer uses overlapping RBs in both time and frequency, and is associated with a unique TCI state and AoA. The scheduled TCI states for the rank 2 PDSCH shall be configured with different QCL typeD reference signals respectively. The DL power at the center of quiet zone from each AoA equals the EIS spherical coverage requirement from sub-clause 7.3.4.

For UEs supporting singleDCI-SDM-scheme-r16, the cumulative throughput in the DL associated with both TCI states shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified A.3.3.2-5 and A.3.3.2-6 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1).

For UEs supporting multiDCI-MultiTRP-r16, the throughput in the DL associated with each TCI state shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified A.3.3.2-1 and A.3.3.2-2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1).

The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to the downlink transmission bandwidth. The UL is assigned to any one of the two TCI states scheduled for simultaneous DL reception, with reference measurement channel as specified in Annex A.2.3.2. The transmitter shall be set to PUMAX as defined in clause 6.2.4.

Unless otherwise specified, the minimum requirements shall be verified with the network signalling value NS_200 (Table 6.2.3.1-1) configured.

### 7.3K.1	(Reserved)

### 7.3K.2	(Reserved)

### 7.3K.3	2AoA spherical coverage of power class 3

The requirement applies to the UE when tested in a test system as described in Annex L. The requirement is verified with the test metric of throughput (Link= 2AoA spherical coverage grid, Meas=Link Angle).

The spherical coverage requirement for simultaneous reception from multiple directions is defined in terms of the probability to support simultaneous reception of rank 2 PDSCH defined in sub-clause 7.3K.0. The probability (see Annex L) is defined as the spatial average over the full sphere around the UE of the probability of any one direction to support 2 AoA reception. In the applicable test system (see Annex L), the probability of any one direction of the UE to support 2 AoA reception for any specific AoA separation is the ratio of the number of unique AoA pairs that include that direction and can support 2 AoA reception to the total number of verified unique AoA pairs that include that direction.

The requirement applies only for the UE’s declared orientation in the positioner of the test system. The requirement for each AoA separation condition applies only for the UE’s declared orientation in the positioner of the test system for that AoA separation. The minimum required overall probability to support 2 AoA reception for power class 3 UEs for any channel bandwidth is specified by AoA separation in table 7.3K.3-1. The UE is only required to fulfill the requirement at any one of AoA separations declared from table 7.3K.3-1.

Table 7.3K.3-1: Requirement for power class 3

| AoA separation (degrees) | Probability (%) |
| --- | --- |
| 30 | 18.5 |
| 60 | 13.5 |
| 90 | 12.5 |
| 120 | 20.5 |
| 150 | 28.5 |

# <End of Change #3>

# <Start of Change #4>

Annex L (normative): Requirement metric for simultaneous reception or transmission in multiple directions

# L.1	Spherical coverage grid for simultaneous reception or transmission in multiple directions

The reference coordinate system grid is defined as follows: The grid is the set of locations described by a source/probe from the UE’s perspective when the TE completes a 3D spherical scan with a constant step size for each rotational axis of its positioner. The grid comprises intersection points of circles of fixed polar angle (geographic analog: ‘latitude’) and meridians of fixed azimuthal angle (geographic analog: ‘longitude’) and completely covers the sphere around the UE. A valid set of locations for the source/probes are those that describes the same constant step size grid from the UE’s perspective over a complete spherical scan. This condition is satisfied when the active probes/sources are always contained in a plane of constant azimuthal angle ‘’ in this reference system. The reference coordinate system with example UE orientations is shown in figures in tables J.2-x.

# L.2	Probability to support 2AoA reception

For UEs required to fulfil a requirement on the probability for 2AoA reception, the metric for a given AoA separation is the spatial average:

Pspatial_av= θ1φ1Pdirectional  (θ1,φ1).AW(θ1)

Where:

(1,1) are coordinates of any grid point in the UE-centered reference system described above.

AW(1) is the area weight associated with each grid point with polar angle ‘1’, and:

θ1φ1AW(θ1)=1

Pdirectional(1,1) is the probability that the direction given by coordinates (1,1) can support 2 AoA reception under the agreed UERF test conditions. Pdirectional(1,1)  is given by:

Pdirectional(θ1,φ1)= {PFAoApair#1+PFAoApair#2+…PFAoApair#nn}

Where:

AoApair#x represents an AoApair with the chosen AoA separation of test that has one source at coordinates (1,1).

‘n’ is the maximum number of unique AoApairs that the TE can generate with one source at coordinates (1,1) . In the test, n is set to 2.

PF(AoApair#x) is the pass/fail outcome (1/0 respectively) of the throughput verification test.

# <End of Change #4>