3GPP TSG-RAN4 Meeting  #110	R4-2400442

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4012 | rev | - | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_newRAT-Core] CR on active TCI state list update delay - R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2024-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is ambiguity/inaccuracy in the current wording in active TCI state list update delay requirement. • The delay is from the time UE receives PDSCH carrying MAC-CE active TCI state list update to the time UE receives PDCCH scheduling PDSCH with the new target TCI state, not the PDCCH with the new target TCI state. The current wording “with the new target TCI state” could be misinterpreted as PDCCH with the new target TCI state. • The delay is the time needed for UE to decode the MAC-CE and perform T/F synchronization for the known TCI state(s) to be added to the active TCI state list. Using the time instant to receive PDCCH as the endpoint of the active TCI state list update is not precise, because PDCCH is up to network scheduling and the network could transmit PDCCH before the UE completes active TCI state list update. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the wording to “If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the active TCI state list update by the end of slot n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length and be able to receive scheduled PDSCH with the new target TCI state at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length + ceiling(timeDurationForQCL/14). Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3, ceiling(x) is the smallest integer such that ceiling(x) x, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], and the value of timeDurationForQCL is defined in TS 38.331 [2].” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is confusion in interpreting the requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10.6, 8.10A.6, 8.10B.6, 8.10C.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change #1>

### 8.10.6	Active TCI state list update delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the active TCI state list update by the end of slot n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length and be able to receive scheduled PDSCH with the new target TCI state at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length + ceiling(timeDurationForQCL/14). Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3, ceiling(x) is the smallest integer such that ceiling(x)  x, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], and the value of timeDurationForQCL is defined in TS 38.331 [2].

# <End of Change #1>

# <Start of Change #2>

### 8.10A.6	Active TCI state list update delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the active TCI state list update by the end of slot n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length and be able to receive scheduled PDSCH with the new target TCI state at the first slot that is after n + 3Nslotsubframe,µ + (THARQ +TOk*(Tfirst-SSB + TSSB-proc+TSSB*LMAC,known)) / NR slot length + ceiling(timeDurationForQCL/14). Where THARQ, Tfirst-SSB, TSSB-proc  , TSSB, LMAC,known and TOk are as defined in clause 8.10A.3, ceiling(x) is the smallest integer such that ceiling(x)  x, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], and the value of timeDurationForQCL is defined in TS 38.331 [2].

# <End of Change #2>

# <Start of Change #3>

### 8.10B.6	Active TCI state list update delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the active TCI state list update by the end of slot n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length and be able to receive scheduled PDSCH with the new target TCI state at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length + ceiling(timeDurationForQCL/14). Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10B.3, , ceiling(x) is the smallest integer such that ceiling(x)  x, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], and the value of timeDurationForQCL is defined in TS 38.331 [2].

# <End of Change #3>

# <Start of Change #4>

### 8.10C.6	Active TCI state list update delay

Upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the active TCI state list update by the end of slot n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length and be able to receive scheduled PDSCH with the new target TCI state at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10C.2, ceiling(x) is the smallest integer such that ceiling(x)  x, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], and the value of timeDurationForQCL is defined in TS 38.331 [2].

# <End of Change #4>