# 3GPP TSG-RAN WG4 Meeting #110	R4-2400486

# Athens, GR, 26 Feb – 01 Mar, 2024

Title:	Reply LS on RAN2 agreements for satellite switch with resync

Response to:	R2-2314016

Release:	Rel-18

Work Item:	NR_NTN_enh-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact Person:

Name:	Jie Cui

E-mail Address:	Jie_cui@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1. Overall Description:

RAN4 would like to thank RAN2 for the LS on RAN2 agreements for satellite switch with resync. RAN4 #110 had discussion on the feasibility of UE to perform the downlink synchronization with the target satellite and keep the communication with the source satellite of the same serving cell simultaneously in soft satellite switch. Following conclusion have been achieved:

During soft satellite switching without PCI change,

- if SSB burst from source satellite and target satellite are not colliding on time domain at UE Rx side, it is feasible that a UE supporting soft satellite switch without PCI change can start synchronizing to the target satellite while still being connected to the source satellite.

- Otherwise, it is infeasible.

Two SSB bursts are considered colliding if at least one of the following conditions is met:

- the two bursts are fully or partially overlapping in time domain, or

- the distance between the two bursts is equal to or smaller than 1 ODFM symbol.

# 2. Actions:

To RAN WG2 group.

ACTION: RAN4 kindly requests RAN2 to take the above agreement into consideration for future work.

# 3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #110bis			15 – 19 April 2024						Changsha, China

TSG RAN WG4 Meeting #111				20 – 24 May 2024						Fukuoka, Japan