3GPP TSG-RAN WG4 Meeting # 110	R4-2400527

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.1.3.1.2

Source:	MediaTek Inc.

Title:	Simulation results on PDSCH requirements for 8Rx UE

Document for:	Information

- Introduction

In RAN4#109 meeting, a WF related to 8Rx UE requirements was agreed [1]. In this contribution, we provide simulation results based on the WF.

- Simulation Results

8Rx single carrier requirements

In Table 1, we provide our simulation results for rank 2, 4 and 8 for Rx UE receiver.

Table 1. Simulation results for 8Rx single carrier

| Rank | MCS | Propagation Condition | Channel Correlation | Alignment results TDD SNR (dB) | Impairment results TDD SNR (dB) | Alignment results FDD SNR (dB) | Impairment results FDD SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 19 (64 QAM, MCS Table 1) | TDLC 300-100 | ULA Medium B | 12.2 | 14.7 | 12.0 | 14.5 |
| 4 | 17 (64 QAM, MCS Table 1) | ULA Low | TDLA 30-10 | 12.5 | 15.0 | 12.7 | 15.2 |
| 8 | 17 (64 QAM, MCS Table 1) | ULA Low | TDLA 30-10 | 20.8 | 23.3 | 20.4 | 22.9 |

8Rx CA requirements

We provide our simulation results for 8Rx CA requirements in the following tables.

Table 2. Simulation results for 8Rx CA (TDD, Rank2, TDLC300-100, 2x8 ULA Medium B)

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDD | 30 | 5 | 2 | 19 (Table 1) | TDLC300-100 | 2x8 ULA Medium B | 12.2 | 14.7 |
|  |  | 10 |  |  |  |  | 12.3 | 14.8 |
|  |  | 15 |  |  |  |  | 12.2 | 14.7 |
|  |  | 20 |  |  |  |  | 12.3 | 14.8 |
|  |  | 25 |  |  |  |  | 12.2 | 14.7 |
|  |  | 30 |  |  |  |  | 12.3 | 14.8 |
|  |  | 40 |  |  |  |  | 12.2 | 14.7 |
|  |  | 50 |  |  |  |  | 12.4 | 14.9 |
|  |  | 60 |  |  |  |  | 12.3 | 14.8 |
|  |  | 80 |  |  |  |  | 12.4 | 14.9 |
|  |  | 90 |  |  |  |  | 12.4 | 14.9 |
|  |  | 100 |  |  |  |  | 12.5 | 15.0 |

Table 3. Simulation results for 8Rx CA (FDD, Rank2, TDLC300-100, 2x8 ULA Medium B)

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 15 | 5 | 2 | 19 (Table 1) | TDLC300-100 | 2x8 ULA Medium B | 12.2 | 14.7 |
|  |  | 10 |  |  |  |  | 12.5 | 15.0 |
|  |  | 15 |  |  |  |  | 12.4 | 14.9 |
|  |  | 20 |  |  |  |  | 12.3 | 14.8 |
|  |  | 25 |  |  |  |  | 12.3 | 14.8 |
|  |  | 30 |  |  |  |  | 12.3 | 14.8 |
|  |  | 35 |  |  |  |  | 12.4 | 14.9 |
|  |  | 40 |  |  |  |  | 12.3 | 14.8 |
|  |  | 45 |  |  |  |  | 12.3 | 14.8 |
|  |  | 50 |  |  |  |  | 12.4 | 14.9 |

Table 4. Simulation results for 8Rx CA (TDD, Rank2, TDLA30-10, 2x8 ULA Low)

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDD | 30 | 5 | 2 | 19 (Table 1) | TDLA30-10 | 2x8 ULA Low | 7.2 | 9.7 |
|  |  | 10 |  |  |  |  | 7.3 | 9.8 |
|  |  | 15 |  |  |  |  | 7.2 | 9.7 |
|  |  | 20 |  |  |  |  | 7.2 | 9.7 |
|  |  | 25 |  |  |  |  | 7.3 | 9.8 |
|  |  | 30 |  |  |  |  | 7.3 | 9.8 |
|  |  | 40 |  |  |  |  | 7.4 | 9.9 |
|  |  | 50 |  |  |  |  | 7.4 | 9.9 |
|  |  | 60 |  |  |  |  | 7.3 | 9.8 |
|  |  | 80 |  |  |  |  | 7.4 | 9.8 |
|  |  | 90 |  |  |  |  | 7.5 | 10.0 |
|  |  | 100 |  |  |  |  | 7.4 | 9.9 |

Table 5. Simulation results for 8Rx CA (FDD, Rank2, TDLA30-10, 2x8 ULA Low)

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 15 | 5 | 2 | 19 (Table 1) | TDLA30-10 | 2x8 ULA Low | 7.2 | 9.7 |
|  |  | 10 |  |  |  |  | 7.3 | 9.8 |
|  |  | 15 |  |  |  |  | 7.2 | 9.7 |
|  |  | 20 |  |  |  |  | 7.2 | 9.7 |
|  |  | 25 |  |  |  |  | 7.2 | 9.7 |
|  |  | 30 |  |  |  |  | 7.3 | 9.8 |
|  |  | 35 |  |  |  |  | 7.4 | 9.9 |
|  |  | 40 |  |  |  |  | 7.3 | 9.8 |
|  |  | 45 |  |  |  |  | 7.4 | 9.9 |
|  |  | 50 |  |  |  |  | 7.4 | 9.9 |

Table 6. Simulation results for 8Rx CA (TDD, Rank8)

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDD | 30 | 5 | 8 | 17 (Table 1) | TDLA30-10 | 8x8 low | 20.8 | 23.3 |
|  |  | 10 |  |  |  |  | 20.6 | 23.1 |
|  |  | 15 |  |  |  |  | 20.7 | 23.2 |
|  |  | 20 |  |  |  |  | 20.6 | 23.1 |
|  |  | 25 |  |  |  |  | 20.7 | 23.2 |
|  |  | 30 |  |  |  |  | 20.9 | 23.4 |
|  |  | 40 |  |  |  |  | 20.8 | 23.3 |
|  |  | 50 |  |  |  |  | 20.7 | 23.2 |
|  |  | 60 |  |  |  |  | 20.8 | 23.3 |
|  |  | 80 |  |  |  |  | 20.7 | 23.2 |
|  |  | 90 |  |  |  |  | 20.7 | 23.2 |
|  |  | 100 |  |  |  |  | 20.8 | 23.3 |

Table 7. Simulation results for 8Rx CA (FDD, Rank8)

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 15 | 5 | 8 | 17 (Table 1) | TDLA30-10 | 8x8 low | 20.5 | 23.0 |
|  |  | 10 |  |  |  |  | 20.4 | 22.9 |
|  |  | 15 |  |  |  |  | 20.4 | 22.9 |
|  |  | 20 |  |  |  |  | 20.3 | 22.8 |
|  |  | 25 |  |  |  |  | 20.5 | 23.0 |
|  |  | 30 |  |  |  |  | 20.4 | 22.9 |
|  |  | 35 |  |  |  |  | 20.6 | 23.1 |
|  |  | 40 |  |  |  |  | 20.7 | 23.2 |
|  |  | 45 |  |  |  |  | 20.7 | 23.2 |
|  |  | 50 |  |  |  |  | 20.8 | 23.3 |

- Conclusion

In this contribution, we provide simulation results for 8Rx PDSCH single carrier and CA requirements.

- Reference

- R4-2321199, “WF for UE 8Rx performance requirements”, Huawei, HiSilicon