3GPP TSG-RAN WG4 Meeting #110	R4-2400650

# Athens, Greece, 26 Feb – 01 Mar, 2024

Agenda item:	8.25.3

Source: 	Qualcomm Incorporated

Title: 	On mobile-IAB RF conformance requirements

Document for:	Approval

# Introduction

In RAN#94e, the work item on mIAB (Integrated Access and Backhaul) for NR was approved in RP-222671 and revised in RP-232643. The objective of performance part of the WI are as follows:

- Specify RF conformance requirements for the mobile IAB-node, if needed.

In RAN4#109, initial discussions on mIAB conformance aspects started. In this paper we provide our proposals on conformance requirements for release 18 mIAB.

# Discussion

In RAN4#109, BigCR [2] for mIAB RF core requirements has been agreed where the RF requirements for mIAB-MT were added in TS 38.174. It was agreed that the requirements for mIAB-MT, unless otherwise stated, follows Local area IAB-MT RF requirements. For other requirements where above statement does not apply (i.e., mIAB-MT dynamic range), an additional suffix (i.e., Suffix B) is added to capture the new mIAB-MT requirements.

Observation 1: RF and RRM discussions agreed to include additional requirements for mIAB-MT in TS 38.174 via an additional suffix (i.e., Suffix B).

Following the same rationale, it is preferred to harmonize the conformance requirements for mIAB-MT in TS 38.176-1 and 38.176-2 following the same methodology.

Proposal 1: Add a new suffix (i.e., Suffix B) in TS 38.176-1 and 38.176-2 to capture additional requirements for mIAB.

Considering the operating bands and channel bandwidths/SCS supported by mIAB-MT are the same as Local area IAB-MT, the current measurement uncertainty and test tolerance for IAB-MT for the IAB-MT output power and output power dynamics for type 1-H, type 1-O and type 2-O mIAB-MT nodes.

Proposal 2: For mIAB-MT type 1-H, type 1-O and type 2-O, proposal to adopt the same test tolerance for Local area IAB-MT given in TS 38.176-1 and TS 38.176-2.

Based on [1], a new requirement for the mIAB-MT total power dynamic range has been agreed, following a UE-type dynamic range mechanism. For the Total power dynamic range, we propose to reuse the test method, highlighted in Section 6.3.2.1.4 and Section 6.4.2.1.4 in TS 38.176-1 for conducted and radiated requirements, respectively.

For the test requirement since the mIAB-MT Tx dynamic range has been changed, we propose to set ΔP between the power measurements as (similar to [3]):

Table 1: mIAB-MT output power dynamics test requirements.

| mIAB-MT type | mIAB-MT channel bandwidth | Requirement |
| --- | --- | --- |
| mIAB-MT 1-H | ≤ 40MHz | 10 log(Maximum RB) + (73 -10log10 (BWChannel /20)) – ([6.5] + 0.7) < ΔP ≤ 10 log(Maximum RB) + (73 -10log10 (BWChannel /20)) + ([6.5] + 0.7) |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + (73 -10log10 (BWChannel /20)) – ([6.5] + 1.0) < ΔP ≤ 10 log(Maximum RB) + (73 -10log10 (BWChannel /20)) + ([6.5] + 1.0) |
| mIAB-MT 1-O | ≤ 40MHz | 10 log(Maximum RB) + 45 – ([6.5] + 0.7) < ΔP ≤ 10 log(Maximum RB) + 45 + ([6.5] + 0.7) |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 45 – ([6.5] + 1.0) < ΔP ≤ 10 log(Maximum RB) + 45 + ([6.5] + 1.0) |
| mIAB-MT 2-O | ≤ 40MHz | 10 log(Maximum RB) + 30 – ([6.5] + 0.7) < ΔP ≤ 10 log(Maximum RB) + 30 + ([6.5] + 0.7) |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 30 – ([6.5] + 1.0) < ΔP ≤ 10 log(Maximum RB) + 30 + ([6.5] + 1.0) |

Proposal 3: RAN4 to adopt the following mIAB-MT output power dynamics test requirements.

| mIAB-MT type | mIAB-MT channel bandwidth | Requirement |
| --- | --- | --- |
| mIAB-MT 1-H | ≤ 40MHz | 10 log(Maximum RB) + 65.8 -10log10 (BWChannel /20) < ΔP ≤ 10 log(Maximum RB) + 80.2 -10log10 (BWChannel /20) |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 65.5 -10log10 (BWChannel /20) < ΔP ≤ 10 log(Maximum RB) + 80.5 -10log10 (BWChannel /20) |
| mIAB-MT 1-O | ≤ 40MHz | 10 log(Maximum RB) + 37.8 < ΔP ≤ 10 log(Maximum RB) + 52.2 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 37.5 < ΔP ≤ 10 log(Maximum RB) + 52.5 |
| mIAB-MT 2-O | ≤ 40MHz | 10 log(Maximum RB) + 22.8 < ΔP ≤ 10 log(Maximum RB) + 37.2 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 22.5 < ΔP ≤ 10 log(Maximum RB) + 37.5 |

Additionally, no specific test or test requirements are defined for Relative power tolerance for legacy mIAB-MT in 38.176-1 and 38.176-2. The Total power dynamic range, as described in Section 6.3.2.1 and Section 6.4.2.1 for conducted and radiated tests provides sufficient test coverage for this requirement. Thus, we do not see a need to propose relative power tolerance conformance requirements for mIAB-MT.

Proposal 4: RAN4 to not specify test requirements for Relative power tolerance for mIAB-MT, following legacy IAB-MT in TS 38.176-1 and TS 38.176-2.

To reflect the above proposals and changes in 38.176-1 and 38.176-2, we have submitted two draftCRs to RAN4#110 to introduce the RF conformance requirements for mIAB.

# Conclusion

In this paper, we have shared our initial views on mIAB RF conformance aspects. Our observations and proposals can be summarized as follows:

Observation 1: RF and RRM discussions agreed to include additional requirements for mIAB-MT in TS 38.174 via an additional suffix (i.e., Suffix B).

Proposal 1: Add a new suffix (i.e., Suffix B) in TS 38.176-1 and 38.176-2 to capture additional requirements for mIAB.

Proposal 2: For mIAB-MT type 1-H, type 1-O and type 2-O, proposal to adopt the same test tolerance for Local area IAB-MT given in TS 38.176-1 and TS 38.176-2.

Proposal 3: RAN4 to adopt the following mIAB-MT output power dynamics test requirements.

| mIAB-MT type | mIAB-MT channel bandwidth | Requirement |
| --- | --- | --- |
| mIAB-MT 1-H | ≤ 40MHz | 10 log(Maximum RB) + 65.8 -10log10 (BWChannel /20) < ΔP ≤ 10 log(Maximum RB) + 80.2 -10log10 (BWChannel /20) |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 65.5 -10log10 (BWChannel /20) < ΔP ≤ 10 log(Maximum RB) + 80.5 -10log10 (BWChannel /20) |
| mIAB-MT 1-O | ≤ 40MHz | 10 log(Maximum RB) + 37.8 < ΔP ≤ 10 log(Maximum RB) + 52.2 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 37.5 < ΔP ≤ 10 log(Maximum RB) + 52.5 |
| mIAB-MT 2-O | ≤ 40MHz | 10 log(Maximum RB) + 22.8 < ΔP ≤ 10 log(Maximum RB) + 37.2 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 22.5 < ΔP ≤ 10 log(Maximum RB) + 37.5 |

Proposal 4: RAN4 to not specify test requirements for Relative power tolerance for mIAB-MT, following legacy IAB-MT in TS 38.176-1 and TS 38.176-2.

# References

- [1] 		R4-2313901, “WF for mobile IAB RF requirements and conformance”, Qualcomm, RAN4#109 .

- [2]		R4-2319785, “Big CR to TS 38.174 on RF core requirements for NR Mobile IAB”, Qualcomm, Ericsson, Nokia, NEC, RAN4#109.

- [3]	R4-2320530, “On mIAB RF Conformance test”, Ericsson, RAN4#109.