3GPP TSG-RAN WG4 Meeting # 110		R4-2400850

Athens, Greece, Feb 26 - Mar 1, 2024

Agenda Item:	9.4.4

Source: 	CMCC

Title: 			 (IoT_NTN_enh-Perf) Discussion on RRM test cases for IOT NTN enhancement

Document for:	Discussion

- Introduction

In RAN4#109 meeting, RAN4 initiated the discussion of the RRM perf for IOT NTN enhancement, the WF has been approved in [1]. All the issues are kept as open issues.

In this contribution, we provide our views on RRM test cases for IOT NTN enhancement based on the core requirements.

- Discussion

Issue 2-2-1: For NB/eMTC, test cases suspended due to lack of neighbour cell assistant information

FFS

- Define test cases which are suspended due to lack of neighbour cell assistant information.

  - Introduce inter-frequency test cases

  - Introduce NGSO configuration for the existing intra-frequency test cases

In R18 IOT-NTN enh WI, since network is able to provide the neighbour cell assistant information to UE, the inter-frequency test cases which were suspended in LTE_NBIOT_eMTC_NTN_req WI can be supported. Furthermore, the general parameters for SIB33 setup for neighbour satellite assistance information should be introduced.

Proposal 1: Introduce inter-frequency test cases as follows:

For NB1 UE

| Cell re-selection | - HD–FDD Inter frequency case for UE Category NB1 |
| --- | --- |
| RRC re-establishment | - HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 |

For Cat-M1 UE

| Cell re-selection | - FDD–FDD Inter frequency case for Cat-M1 UE in normal coverage |
| --- | --- |
|  | - HD–FDD Inter frequency case for Cat-M1 UE in normal coverage |
| Handover | - FDD-FDD Inter frequency handover for Cat-M1 UEs |
|  | - HD-FDD Inter frequency handover for Cat-M1 UEs |
|  | - FDD-FDD Inter frequency conditional handover for Cat-M1 UEs |
|  | - HD-FDD Inter frequency conditional handover for Cat-M1 UEs |
| RRC re-establishment | - FDD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE |
|  | - HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE |
| UE measurement procedure in RRC-CONNECTED | - FDD-FDD inter-frequency event triggered reporting for Cat-M1 UE in CE-ModeA |
|  | - HD-FDD inter-frequency event triggered reporting for Cat-M1 UE in CE-ModeA |
|  | - FDD-FDD inter-frequency event triggered reporting for Cat-M1 UE in CE-ModeA when DRX is used |
|  | - HD-FDD inter-frequency event triggered reporting for Cat-M1 UE in CE-ModeA when DRX is used |

Proposal 2: Introduce the general parameters for SIB33 setup for neighbour satellite assistance information  as follows:

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference configuration for serving satellite |  | NSC.1 | NSC.2 |
| Scenario |  | GSO | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 |
| neighValidityDuration-r18 | s | 900 | 5 |
| k-Mac-r18 | slot | Not configured | Not configured |
| nta-Common-r18 |  | 0 | 0 |
| nta-CommonDrift-r18 |  | 0 | 0 |
| nta-CommonDriftVariation-r18 |  | 0 | 0 |
| ephemerisInfo |  | According to Annex B.8 |  |

Besides, since network could provide the neighbour cell assistance information to UE, both GSO test configuration and NGSO test configuration should be supported for all the test cases.

Proposal 3: Both GSO test configuration and NGSO test configuration should be supported for all the test cases.

Issue 2-2-2: For NB-IoT, neighbour cell measurement in CONNNECTED mode

FFS

- For NB-IoT, introduce test cases for neighbour cell measurement in CONNNECTED mode.

Issue 2-2-3: For NB/eMTC, test cases for time/location based triggering of cell reselection in IDLE mode

FFS

- RAN4 to define test cases for time/location based triggering of cell reselection in IDLE mode based on the following table:

| Requirements | NB/eMTC |
| --- | --- |
| IDLE: Time-based measurement triggering | NB/eMTC |
| IDLE: Location-based measurement triggering | NB/eMTC |

- RAN4 to further discuss whether and how to reduce the number of test cases

Issue 2-2-4: For NB/eMTC, test cases for time/location based triggering of neighbour cell measurements in CONNECTED mode

FFS

- RAN4 to discuss feasibility of defining tests for time- and location based triggering of neighbour cell measurements.

  - Option 1: not to define the corresponding test cases.

  - Option 2: RAN4 to define test cases for time- and location based triggering of neighbour cell measurements, based on the following table:

| Requirements | NB/eMTC |
| --- | --- |
| CONN: Location-based measurement triggering [NB/eMTC] | NB/eMTC |
| CONN: Time-based measurement triggering | eMTC |

Issue 2-2-5: For eMTC, test cases for CHO

FFS

- RAN4 to define test cases for eMTC CHO based on the following table:

| Requirements | NB/eMTC |
| --- | --- |
| CHO configured with condEventD1 | eMTC |
| CHO configured with condEventT1 | eMTC |

- RAN4 to further discuss whether and how to reduce the number of test cases

In R18 IOT-NTN enh WI, some enhanced features had been introduced, which including:

- Time/Location based triggering of cell re-selection for both NB and eMTC over NTN

- Time/Location-based CHO for eMTC over NTN

- Time/Location only-based CHO for eMTC over NTN

- Intra/Inter-frequency measurement of neighbour cell in RRC-CONNECTED for NB over NTN

- Time/Location based triggering of intra/inter frequency measurements in RRC-CONNECTED for both NB and eMTC over NTN.

Accordingly, the test cases are needed to be introduced for all above new features. In order to limit the test number, we propose to only cover unknown case for Time/Location only-based CHO for eMTC over NTN.

Proposal 4: Introduce following test cases to verify the new features, only unknown case for time/location only-based CHO for eMTC over NTN is covered in order to limit the test number.

For NB1 UE

| Cell re-selection | Time-based measurement initiation to HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage |
| --- | --- |
|  | Location-based measurement initiation to HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage |
|  | Time-based measurement initiation to HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage |
|  | Location-based measurement initiation to HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage |
| UE measurement procedure in RRC-Connected | HD-FDD intra-frequency event triggered reporting for UE Category NB1 in normal coverage |
|  | Time-based measurement initiation to HD – FDD Intra frequency case for UE Category NB1 in normal coverage |
|  | location-based measurement initiation to HD – FDD Intra frequency case for UE Category NB1 in normal coverage |
|  | HD-FDD inter-frequency event triggered reporting for UE Category NB1 in normal coverage |
|  | Time-based measurement initiation to HD – FDD Inter frequency case for UE Category NB1 in normal coverage |
|  | location-based measurement initiation to HD – FDD Inter frequency case for UE Category NB1 in normal coverage |

For Cat-M1 UE

| Cell re-selection | Time-based measurement initiation to E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
| --- | --- |
|  | Location-based measurement initiation to E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
|  | Time-based measurement initiation to E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
|  | Location-based measurement initiation to E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
|  | Time-based measurement initiation to E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
|  | Location-based measurement initiation to E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
|  | Time-based measurement initiation to E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
|  | Location-based measurement initiation to E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
| CHO | E-UTRAN FDD-FDD Intra frequency Time-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN FDD-FDD Intra frequency Location-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN FDD-FDD Intra frequency Time only-based conditional handover for Cat-M1 UEs in CEModeA (for unknown target cell) |
|  | E-UTRAN FDD-FDD Intra frequency Location only-based conditional handover for Cat-M1 UEs in CEModeA (for unknown target cell) |
|  | E-UTRAN HD-FDD Intra frequency Time-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN HD-FDD Intra frequency Location-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN HD-FDD Intra frequency Time only-based conditional handover for Cat-M1 UEs in CEModeA(for unknown target cell) |
|  | E-UTRAN HD-FDD Intra frequency Location only-based conditional handover for Cat-M1 UEs in CEModeA(for unknown target cell) |
|  | E-UTRAN FDD-FDD Inter frequency Time-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN FDD-FDD Inter frequency Location-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN FDD-FDD Inter frequency Time only-based conditional handover for Cat-M1 UEs in CEModeA (for unknown target cell) |
|  | E-UTRAN FDD-FDD Inter frequency Location only-based conditional handover for Cat-M1 UEs in CEModeA (for unknown target cell) |
|  | E-UTRAN HD-FDD Inter frequency Time-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN HD-FDD Inter frequency Location-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN HD-FDD Inter frequency Time only-based conditional handover for Cat-M1 UEs in CEModeA (for unknown target cell) |
|  | E-UTRAN HD-FDD Inter frequency Location only-based conditional handover for Cat-M1 UEs in CEModeA (for unknown target cell) |
| UE measurement procedure in RRC-Connected | Time-based measurement initiation to FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
|  | location-based measurement initiation to FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
|  | Time-based measurement initiation to HD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
|  | location-based measurement initiation to HD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
|  | Time-based measurement initiation to FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
|  | location-based measurement initiation to FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
|  | Time-based measurement initiation to HD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
|  | location-based measurement initiation to HD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
| Measurement performance requirements | FDD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA |
|  | HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA |

Propagation condition

AWGN channel with doppler and timing shifts setting defined in Annex B.8 is widely used in RRM test cases for NTN and IOT-NTN, except for the existing tests of intra-frequency event triggered reporting for Cat-M1 UE (A.14.5 in TS36.133), which use the fading propagation condition ETU30 instead.

Based on our understanding, compared to ETU30, AWGN is more in line with the actual NTN propagation condition, it can also work together with the doppler and timing shifts design in Annex B.8. Therefore, we prefer to first revise the propagation condition to AWGN for the existing tests of intra-frequency event triggered reporting for Cat-M1 UE. Secondly, for the new test cases which to be introduced, we propose to set AWGN as propagation condition.

Proposal 5: Revise the propagation condition to AWGN for the existing tests of intra-frequency event triggered reporting for Cat-M1 UE (A.14.5 in TS36.133).

Proposal 6: For the new test cases which to be introduced, set AWGN as propagation condition.

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: Introduce inter-frequency test cases as follows:

For NB1 UE

| Cell re-selection | - HD–FDD Inter frequency case for UE Category NB1 |
| --- | --- |
| RRC re-establishment | - HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 |

For Cat-M1 UE

| Cell re-selection | - FDD–FDD Inter frequency case for Cat-M1 UE in normal coverage |
| --- | --- |
|  | - HD–FDD Inter frequency case for Cat-M1 UE in normal coverage |
| Handover | - FDD-FDD Inter frequency handover for Cat-M1 UEs |
|  | - HD-FDD Inter frequency handover for Cat-M1 UEs |
|  | - FDD-FDD Inter frequency conditional handover for Cat-M1 UEs |
|  | - HD-FDD Inter frequency conditional handover for Cat-M1 UEs |
| RRC re-establishment | - FDD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE |
|  | - HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE |
| UE measurement procedure in RRC-CONNECTED | - FDD-FDD inter-frequency event triggered reporting for Cat-M1 UE in CE-ModeA |
|  | - HD-FDD inter-frequency event triggered reporting for Cat-M1 UE in CE-ModeA |
|  | - FDD-FDD inter-frequency event triggered reporting for Cat-M1 UE in CE-ModeA when DRX is used |
|  | - HD-FDD inter-frequency event triggered reporting for Cat-M1 UE in CE-ModeA when DRX is used |

Proposal 2: Introduce the general parameters for SIB33 setup for neighbour satellite assistance information as follows:

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference configuration for serving satellite |  | NSC.1 | NSC.2 |
| Scenario |  | GSO | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 |
| neighValidityDuration-r18 | s | 900 | 5 |
| k-Mac-r18 | slot | Not configured | Not configured |
| nta-Common-r18 |  | 0 | 0 |
| nta-CommonDrift-r18 |  | 0 | 0 |
| nta-CommonDriftVariation-r18 |  | 0 | 0 |
| ephemerisInfo |  | According to Annex B.8 |  |

Proposal 3: Both GSO test configuration and NGSO test configuration should be supported for all the test cases.

Proposal 4: Introduce following test cases to verify the new features, only unknown case for time/location only-based CHO for eMTC over NTN is covered in order to limit the test number.

For NB1 UE

| Cell re-selection | Time-based measurement initiation to HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage |
| --- | --- |
|  | Location-based measurement initiation to HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage |
|  | Time-based measurement initiation to HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage |
|  | Location-based measurement initiation to HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage |
| UE measurement procedure in RRC-Connected | HD-FDD intra-frequency event triggered reporting for UE Category NB1 in normal coverage |
|  | Time-based measurement initiation to HD – FDD Intra frequency case for UE Category NB1 in normal coverage |
|  | location-based measurement initiation to HD – FDD Intra frequency case for UE Category NB1 in normal coverage |
|  | HD-FDD inter-frequency event triggered reporting for UE Category NB1 in normal coverage |
|  | Time-based measurement initiation to HD – FDD Inter frequency case for UE Category NB1 in normal coverage |
|  | location-based measurement initiation to HD – FDD Inter frequency case for UE Category NB1 in normal coverage |

For Cat-M1 UE

| Cell re-selection | Time-based measurement initiation to E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
| --- | --- |
|  | Location-based measurement initiation to E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
|  | Time-based measurement initiation to E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
|  | Location-based measurement initiation to E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
|  | Time-based measurement initiation to E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
|  | Location-based measurement initiation to E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
|  | Time-based measurement initiation to E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
|  | Location-based measurement initiation to E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
| CHO | E-UTRAN FDD-FDD Intra frequency Time-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN FDD-FDD Intra frequency Location-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN FDD-FDD Intra frequency Time only-based conditional handover for Cat-M1 UEs in CEModeA (for unknown target cell) |
|  | E-UTRAN FDD-FDD Intra frequency Location only-based conditional handover for Cat-M1 UEs in CEModeA (for unknown target cell) |
|  | E-UTRAN HD-FDD Intra frequency Time-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN HD-FDD Intra frequency Location-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN HD-FDD Intra frequency Time only-based conditional handover for Cat-M1 UEs in CEModeA (for unknown target cell) |
|  | E-UTRAN HD-FDD Intra frequency Location only-based conditional handover for Cat-M1 UEs in CEModeA (for unknown target cell) |
|  | E-UTRAN FDD-FDD Inter frequency Time-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN FDD-FDD Inter frequency Location-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN FDD-FDD Inter frequency Time only-based conditional handover for Cat-M1 UEs in CEModeA (for unknown target cell) |
|  | E-UTRAN FDD-FDD Inter frequency Location only-based conditional handover for Cat-M1 UEs in CEModeA (for unknown target cell) |
|  | E-UTRAN HD-FDD Inter frequency Time-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN HD-FDD Inter frequency Location-based conditional handover for Cat-M1 UEs in CEModeA |
|  | E-UTRAN HD-FDD Inter frequency Time only-based conditional handover for Cat-M1 UEs in CEModeA (for unknown target cell) |
|  | E-UTRAN HD-FDD Inter frequency Location only-based conditional handover for Cat-M1 UEs in CEModeA (for unknown target cell) |
| UE measurement procedure in RRC-Connected | Time-based measurement initiation to FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
|  | location-based measurement initiation to FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
|  | Time-based measurement initiation to HD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
|  | location-based measurement initiation to HD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
|  | Time-based measurement initiation to FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
|  | location-based measurement initiation to FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
|  | Time-based measurement initiation to HD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
|  | location-based measurement initiation to HD – FDD Inter frequency case for Cat-M1 UE in normal coverage |
| Measurement performance requirements | FDD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA |
|  | HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA |

Proposal 5: Revise the propagation condition to AWGN for the existing tests of intra-frequency event triggered reporting for Cat-M1 UE (A.14.5 in TS36.133).

Proposal 6: For the new test cases which to be introduced, set AWGN as propagation condition.

- Reference

- R4-2321577, WF on R18 IoT NTN RRM requirements, MediaTek.