3GPP TSG-RAN WG4 Meeting #110	R4-2400892

Athens, Greece, Feb 26 – Mar 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	(NR_newRAT-Perf) 38.101-4 wrong values in RMC

Agenda item:	4.5

Document for:	Information

Introduction

The contribution will provide information on why we see the current values used in 38.101-4 “Table A.3.2.2.5-2: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 (16QAM)” R.PDSCH.5-2.1 TDD for “Binary Channel Bits Per Slot” specifically for slots I=82 and I=83 as being incorrect.

Cutout from 38.101-4 “Table A.3.2.2.5-2: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 (16QAM)” R.PDSCH.5-2.1-TDD

Binary Channel Bits Per Slot. For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159}; Bits; N/A. For Slots i = 80, 81; Bits; 36564. For Slots i = 82; Bits; 35028. For Slots i = 83; Bits; 22884. For Slot i, if mod(i, 5) = 3 for i from {0,…, 159}; Bits; 24420. For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79,84,…,159}; Bits; 36564.

Note the difference between i=82 and i=83 is due to i=83 is a special slot.

Background

Specifically for the slots i=82 and i=83 is included CSI-RS TRS. The specification for CSI-RS TRS is as follows as defined in 38.101-4 “Table 7.2-1: Common Test Parameters”:

CSI-RS for tracking; First subcarrier index in the PRB used for CSI-RS (k0); 0 for CSI-RS resource 1,2,3,4. First OFDM symbol in the PRB used for CSI-RS (l0); 6 for CSI-RS resource 1 and 3 10 for CSI-RS resource 2 and 4. Number of CSI-RS ports (X); 1 for CSI-RS resource 1,2,3,4. CDM Type; 'No CDM' for CSI-RS resource 1,2,3,4. Density (ρ); 3 for CSI-RS resource 1,2,3,4. CSI-RS periodicity; Slots; 60 kHz SCS: 80 for CSI-RS resource 1,2,3,4 120 kHz SCS: 160 for CSI-RS resource 1,2,3,4. CSI-RS offset; Slots; 60 kHz SCS: 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 120 kHz SCS: 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4. Frequency Occupation; Start PRB 0 Number of PRB = ceil(BWP size/4)*4. QCL info; TCI state #0.

According to 38.101-4 “R.PDSCH.5-1.1 TDD”, the BWP size is set to 66 PRB. This gives the following frequency occupation:

Frequency Occupation= ceilBWPsize4*4=ceil664*4= 68

According to TS 38.331 the following applies:

#### CSI-FrequencyOccupation The IE CSI-FrequencyOccupation is used to configure the frequency domain occupation of a channel state information measurement resource (e.g. NZP-CSI-RS-Resource, CSI-IM-Resource). CSI-FrequencyOccupation information element -- ASN1START -- TAG-CSI-FREQUENCYOCCUPATION-START CSI-FrequencyOccupation ::= SEQUENCE { startingRB INTEGER (0..maxNrofPhysicalResourceBlocks-1), nrofRBs INTEGER (24..maxNrofPhysicalResourceBlocksPlus1), ... } -- TAG-CSI-FREQUENCYOCCUPATION-STOP -- ASN1STOP CSI-FrequencyOccupation field descriptions. nrofRBs Number of PRBs across which this CSI resource spans. Only multiples of 4 are allowed. The smallest configurable number is the minimum of 24 and the width of the associated BWP. If the configured value is larger than the width of the corresponding BWP, the UE shall assume that the actual CSI-RS bandwidth is equal to the width of the BWP.. startingRB PRB where this CSI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid. Only multiples of 4 are allowed (0, 4, ...).

As the calculated frequency occupation is higher than the BWPsize (68 vs 66) the resulting frequency occupation must be 66.

The following are the configuration values taken from 38.101-4 where R.PDSCH.5-2.1-TDD is used:

| Parameter | Normal slot | Special slot |
| --- | --- | --- |
| MO (# bits/RE) | 4 | 4 |
| #PDSCH_RE | 13 | 9 |
| #PRB | 66 | 66 |
| #Layers | 1 | 1 |
| #DMRS | 2 | 2 |
| - #DMRS_REs (per PRB) | #DMRS*6=12 | #DMRS*6=12 |
| #PTRS_REs | 13-2=11 every 2nd PRB | 9-2=7 every 2nd PRB |
| #CSI-RS_REs (TRS) per PRB | 6 | 6 |
| CSI-FrequencyOccupation (#PRBTRS) | 66 | 66 |

We use the following formular for calculating Binary Channel Bits Per Slot:

BCBPS=MO*#Layers*(12*#PDSCHRE-#DMRSRE*#PRB-#PTRSRE*#PRBwith-PTRS-#CSI_RSRE*#PRBTRS)

We can now calculate the following:

BCBPSi81=4*1*(12*13-12*66-11*33-6*66)=34980

BCBPSi82=4*1*12*9-12*66-7*33-6*66=22836

For reference, we have found that using CSI Frequency Occupation of 64 PRBs will using above formular give the currently used values in 38.101-4:

BCBPSi81=4*1*(12*13-12*66-11*33-6*64)=35028

BCBPSi82=4*1*12*9-12*66-7*33-6*64=22884

Additionally checking the Binary Channel Bits Per Slot for the slots without CSI-RS TRC we can confirm the formular provides the same results as is in the specification:

BCBPSNormal_slot=4*1*(12*13-12*66-11*33-0*66)=36546

BCBPSSpecial_slot=4*1*12*9-12*66-7*33-0*66=24420

The existing values for Binary Channel Bits Per Slot for R.PDSCH.5-2.1-TDD i=82 and i=83 are incorrect as the wrong value (64) has been used for CSI Frequency Occupation, which should have been 66.

Based on above finding we have contributed with a CR to correct the issue (see [2]). In addition, we will check if the same issue can be found in other RMCs where the CBW is not a multiple of 4, hence we will fully check the specification for next meeting and if needed contribute a CR with any remaining changes if found.

Our CR (R4-2400893) contributed in RAN4#110 aims a correcting the issue described here. In addition, we will check the other RMCs in 38.101-4 and contribute a CR in the next meeting if we find other RMCs with similar issue.

Conclusion

In this contribution we have provide our reasoning for why the two of the existing values in RMC R.PDSCH.5-2.1 TDD are incorrect.

We have the following observations:

- The existing values for Binary Channel Bits Per Slot for R.PDSCH.5-2.1-TDD i=82 and i=83 are incorrect as the wrong value (64) has been used for CSI Frequency Occupation which should have been 66.

- Our CR contributed in RAN4#110 aims a correcting the issue described here. In addition, we will check the other RMCs in 38.101-4 and contribute a CR in the next meeting if we find other RMCs with similar issue.

- References

- TS38.101-4 v18.2.0 User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)

- R4-2400893 - (NR_newRAT-Perf) CR for 38.101-4 on correction of wrong values in RMC (Rel-15, Cat F)