3GPP TSG-RAN WG4 Meeting # 110 	R4-2400898

# Athens, Greece, Febuary 26 – March 01, 2024

Source:		Verizon, AT&T, Samsung, Ericsson

Title:			TP for TR 38.880: PC2 3Tx inter-band CA_n5-n77

Agenda item:		7.25.2

Document for:		Discussion/Approval

# Introduction

The requrement of PC1.5 inter-band UL CA_n5-n77 with 3Tx was introduced in R4-2315969 and added in the specification [2]. Still the PC2 requirement is missing.

This is a TP to TR 38.880 to add PC2 CA_n5_n77 requirement.

# Reference

[1] RP-232941, WID revision for Rel-18 basket WI 3Tx NR inter-band UL Carrier Aggregation (CA) and EN-DC

[2] TS 38.101-1 V18.4.0

# Text proposal

====================== <Start of Text Proposal> ======================

### 5.x	CA_n5A-n77A

### 5.x.1	Operating bands for CA

Table 5.x.1-1:  CA band combination of band n5+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n5-n77 | n5, n77 |

### 5.x.2	Maximum output power for inter-band CA

Table 5.x.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n77A |  |  | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. NOTE 4: The UE supports PC2 with UL MIMO in either one of the TDD bands and PC2 in the other TDD band. |  |  |  |  |  |  |

Table 5.x.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n77A |  |  | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.x.3	MSD scenario studies

Table 5.x.3-1 and 5.x.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_ n5-n77.

Table 5.x.3-1: MSD scenarios for CA_n5-n77 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | 1st order of n77 UL | 4nd order of n5 DL | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD | IMD4 (1*f_n77-3*f_n5) | n5 DL | Yes |
|  | IMD5 (1*f_n77-4*f_n5) | n5 DL | Yes |

### 5.x.4	REFSENS requirements

#### 5.x.4.1	REFSENS requirements for total power class 2

In previous meeting, RAN4 agreed that the following rules will be as the baseline to define MSD requirement for PC2 inter-band CA or EN-DC:

- Same MSD requirements can be applied for the band combination with either 1Tx or 2Tx in TDD aggressor band UL with same power class for harmonic mixing and cross band isolation, and

- the requirements specified based on 1Tx-1Tx UL configuration are applicable for 1Tx-2Tx UL configuration

Based on the agreement, the analysis MSD results are summarized in both Table 5.x.4.1-1. Actually, the same MSD PC2 2Tx configuration from the specification [2] is applied for this 3Tx requirement based on the rule above.

According to the RAN4 specification [2], both PC2 IMD4 and IMD5 are the subjects to DL n5 in CA_n5-n77, and IMD values are derived in Table 5.x.4.1-1 based on linear extrapolation and referred the MSD for PC3 configuration.

Table 5.x.4.1-1: Derived MSD values for PC2 UL configurations

| CA Combination | Power Class | NR Band | PUL (dBm) | IMD Source | IMD Order | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n5-n77 | 3 | n5 | 20 | IMD4 | 3 | 8.3 |
|  |  | n77 | 20 |  | 1 |  |
|  | 2 | n5 | 23 |  | 3 | 18.6 |
|  |  | n77 | 23 |  | 1 |  |
|  | 3 | n5 | 20 | IMD5 | 4 | 5.5 |
|  |  | n77 | 20 |  | 1 |  |
|  | 2 | n5 | 23 |  | 4 | 20.0 |
|  |  | n77 | 23 |  | 1 |  |

For the MSD requirements, the required values are summarized in the Table 5.x.4.1-2 based on discussion above.

Table 5.x.4.1-2: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n77 | 5 |  | 844 | 5 | 25 | 889 | 18.6 | FDD | IMD4 |
|  | n77 |  | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
|  | n5 | 829 |  | 5 | 25 | 874 | 20.0 | FDD | IMD5 |
|  | n77 | 4190 |  | 10 | 50 | 4190 | N/A | TDD | N/A |

For the harmonic mixing requirement to this PC2 UL CA_n5-n77 combo, the requirements are reused from [2] into Table 5.x.4.1-2.

Table 5.x.4.1-2 (same as Table 7.3A.4-4a of 38.101-1): Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n5 | 10 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n5 | 20 | 15 | 20 (RBstart=0) | 20 | 4.3 | NOTE 5 | UL1/DL4 |

RAN4 has concluded that there is no cross-band isolation issue to this combo. Thus, the same conclusion is held in this proposal.

====================== <End of Text Proposal> ======================