# 3GPP TSG-RAN WG4 Meeting # 110	R4-2400902

Athens, Greece, February 26-March 01, 2024

Source:	Verizon, Samsung, Ericsson

Title:	MSD analysis for DL band combinations with ULCA_n77C configuration

Agenda item:	7.1.1.1

Document for:	Approval

# Introduction

The configurations of DL CA_n2(2A)-n77C, CA_n13A-n77C, CA_n66(3A)-n77C, CA_n48A-n77C, CA_n48(A-B)-n77C with UL CA_n77C are in WID RP-232920. The related MSD requirements for the band combination will be analysed in this contribution.

# Considerations

2.1	Analysis of IMD order for downlink FDD and TDD CA configuration

The following configurations with intra-band UL CA configuration are required. Some IMD order are calculated and listed in table below.

| NR CA band combination | UL CA configuration | Wgap | Min (Max class, BW) | IMD order |
| --- | --- | --- | --- | --- |
| CA_n2(2A)-n77C | CA_n77C | 1460 | 200 | 17 |
| CA_n13A-n77C | CA_n77C | 2694 | 200 | 29 |
| CA_n66(3A)-n77C | CA_n77C | 1250 | 200 | 15 |

In analysis, we account for the US spectrum of band n77 which is 3450 – 3980MHz. Based on RAN4 agreed rule, these IMD order are large enough, and the impacts to the specification from these MSD could be ignored.

2.2	Analysis of IMD order for downlink band n48 and n77 CA configuration

Both CA_n48A-n77C and CA_n48(A-B)-n77C configurations are required in Rel-18. In practical, first these two TDD CA will be in fully synched in operation. Following the existing condition in 38,101-1, also the UL CA configuration will be only when there is non-simultaneous Tx/Rx operation. To ensure this, the uplink band n48 will be not configured with the uplink CA_n77C configured in the operation.

Therefore, there is no con-existence issue identified for UL CA_n77C configuration to the downlink band combination CA_n48-n77.

For the purpose of fall-back combo from other higher order combinations, the uplink CA_77C is needed in the downlink CA_n48-n77 configuration. Thus, it is proposed in this analysis for the configuration.

# Conclusion

There is no IMD issue identified from the intra-band UL CA_n77C configuration. One of draftCR will be used to introduce the UL CA_n77C configuration accordingly to the spec.